SA WG2 Meeting #136 	S2-1911477

18 - 22 Nov 2019, Reno, USA	 (revision of S2-190xxxx)

Source:	vivo

Title:	Solution to support UE-to-UE Relay connection establishment

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: this paper proposes to add a solution to support the establishment of UE-to UE relay connection.

Discussion

UE-to-UE Relay should be supported as specified in KI#4.

This P-CR proposes to reuse the V2X unicast commutation mechanisms defined in TS 23.287 [5] clause 6.2 to realize the support of UE-to-UE Relay connection establishment.

Proposal

It is proposed to add a new solution for UE-to-UE Relay.

First change

6.X	Solution to support UE-to-UE Relay connection establishment

6.X.1	Description

This solution is addressed to KI#4 "Support of UE-to-UE Relay ".

This solution is based on V2X direct communication establishment mechanism defined in TS 23.287 [5] clause 6.2 and does not need to have a discovery procedure previously, which means the Direct communication request massage can also be used to discover the UE-to-UE Relay.

The main addition of the solution is to add a relay information in the PC5 Direct Communication Request.

6.X.2	Procedures

6.X.2.1	Direct Communication via UE-to-UE Relay

Figure 6.X.2.1-1: Direct Communication via UE-to-UE Relay establishment procedure

- UE-1 decides to initiate the UE-to-UE relay communication with UE-2.

- UE-1 broadcasts a Direct Communication Request message with the relay info.

2a.	If there already exists a PC5 connection between the UE-to-UE Relay-1 and the UE-2, the authorised UE-to-UE Relay-1 sends a Direct communication request message by unicast with relay info to UE-2.

Otherwise, the authorised UE-to-UE Relay-1 broadcasts a Direct communication request message with the relay info.

2b.	The authorised UE-to-UE Relay-2 broadcast a Direct communication request with the relay info.

Editor's Note: The details of relay authorization are FFS.

- If UE-2 accepts the UE-to-UE relay communication with UE-1 via the UE-to-UE Relay-1, UE-2 sends the Direct communication accept message to UE-to-UE Relay-1.

4.	After receiving the Direct communication accept message from UE-2, UE-to-UE Relay-1 sends the Direct communication accept message to UE-1.

6.X.3	Impacts on Existing Nodes and Functionality

Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

End of change