SA WG2 Meeting #S2-138E	S2-2003121

20 - 24 April, 2020, Electronic meeting	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2349 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of NOTE on Hold and Forward buffering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 5.27.5 contains the following NOTE: NOTE 2: With supporting the hold and forward buffering mechanism in UE-DS-TT and NW-TT, the values of independentDelayMax and independentDelayMin for 5GS Bridge can be the same. While not explicitly stated, the text in the NOTE refers to per-stream H&F buffering. During the SA2 conference call held on 08 Apr 2020 the following working assumption was endorsed: - Support for PSFP based Hold and Forward Buffering rule - Given no clear majority this functionality will not be supported in Rel-16. If PFSP (i.e. per-stream) Hold and Forward buffering is not supported, then the configuration where independentDelayMax and independentDelayMin can be the same does not make sense. Reducing the jitter to zero (as implied by NOTE 2) can only be achieved by using per-stream H&F buffering at the egress ports. It is proposed to remove NOTE 2. Note that the removal of NOTE 2 does not preclude the use of per-class (802.1Qbv) Hold and Forward buffering, which is a different thing. Clause 5.28.3.1: NOTE 10 in Table 5.28.3.1-1 refers to the optional use of PFSP at the DS-TT and the NW-TT. Given the working agreement that PFSP H&F buffering is not supported in Rel-16, the PFSP information is not available at the DS-TT and NW-TT. Therefore NOTE 10 needs to be changed accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.27.5: Removed NOTE 2. Clause 5.28.3.1: Changed NOTE 10 in Table 5.28.3.1-1 to indicate that the PFSP information in this release is only available at the AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Erroneous specifiction text implying that per-stream H&F buffering can be used at the egress ports in Rel-16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.5; 5.28.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***BEGIN CHANGES***

### 5.27.5	5G System Bridge delay

In order for the 5G System to participate as a TSN bridge according to gate schedules specified, the 5GS Bridge is required to provide Bridge Delays as defined in IEEE 802.1Qcc [95] for each port pair and traffic class of the 5GS bridge to an IEEE TSN system. In order to determine 5GS Bridge Delays, the following components are needed:

1.	UE-DS-TT Residence Time: the time taken within the UE and DS-TT to forward a packet between the UE and DS-TT port. UE-DS-TT Residence Time is provided at the time of PDU Session Establishment by the UE to the network.

NOTE 1:	UE-DS-TT Residence Time is the same for uplink and downlink traffic and applies to all traffic classes.

2.	Per traffic class delay between the UE and the UPF/NW-TT that terminates the N6 interface (including UPF and NW-TT residence times), independent of frame length that a given 5GS deployment supports. The per-traffic class delay between the UE and the UPF/NW-TT is pre-configured in the TSN AF (see clause 5.28.4).

The TSN AF calculates the 5GS independentDelayMin and independentDelayMax values for each port pair and for each traffic class using the above components.

The dependentDelayMin and dependentDelayMax for 5GS Bridge specify the time range for a single octet of an Ethernet frame to transfer from ingress to egress and include the time to receive and store each octet of the frame, which depends on the link speed of the ingress Port as per IEEE 802.1Qcc [95].

NOTE 2:	Further details how TSN AF determines dependentDelayMin and dependentDelayMax and is up to implementation.

Since Residence times may vary among UEs and among UPFs, the 5GS Bridge Delay is determined after the PDU Session Establishment for the corresponding UPF and the UE by the TSN AF. The TSN AF deduces the related port pair(s) from the port number of the DS-TT Ethernet port and port number of the serving NW-TT Ethernet port(s) when the TSN AF receives the 5GS Bridge information for a newly established PDU Session and caculates the bridge delays per port pair.

***NEXT CHANGE***

### 5.28.3	Port management information exchange in 5GS

#### 5.28.3.1	General

Bridge management information is exchanged between CNC and TSN AF. A subset of bridge management information, referred to as port management information, is related to Ethernet ports located in DS-TT or NW-TT.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF and DS-TT or NW-TT, respectively inside a Port Management Information Container. Table 5.28.3.1-1 lists standardized port management information.

Table 5.28.3.1-1: Standardized port management information

| Port management information | Applicability (see Note 6) |  | Supported operations by TSN AF (see Note 1) | Reference |
| --- | --- | --- | --- | --- |
|  | DS-TT | NW-TT |  |  |
| General |  |  |  |  |
| Port management capabilities (see Note 2) | X | X | R |  |
| Bridge delay related information |  |  |  |  |
| txPropagationDelay | X | X | R | IEEE 802.1Qcc [95] clause 12.32.2.1 |
| Traffic class related information |  |  |  |  |
| Traffic class table | X | X | RW | IEEE 802.1Q [98] clause 12.6.3 and clause 8.6.6. |
| Gate control information |  |  |  |  |
| GateEnabled | X | X | RW | IEEE 802.1Q [98] Table 12-29 |
| AdminBaseTime | X | X | RW | IEEE 802.1Q [98] Table 12-29 |
| AdminControlList | X | X | RW | IEEE 802.1Q [98] Table 12-29 |
| AdminCycleTime (see Note 3) | X | X | RW | IEEE 802.1Q [98] Table 12-29 |
| AdminControlListLength (see Note 3) | X | X | RW | IEEE 802.1Q [98] Table 12-28 |
| Tick granularity | X | X | R | IEEE 802.1Q [98] Table 12-29 |
| Traffic forwarding information |  |  |  |  |
| Static Filtering Entry (NOTE 7) |  | X | RW | IEEE 802.1Q [98] clause 8.8.1 |
| General Neighbor discovery configuration (NOTE 4) |  |  |  |  |
| adminStatus | D | X | RW | IEEE 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | D | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | D | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | D | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | D | X | RW | IEEE 802.1AB [97] Table 11-2 |
| NW-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype |  | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocPortId |  | X | RW | IEEE 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype | D | N | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocPortId | D | N | RW | IEEE 802.1AB [97] Table 11-2 |
| Neighbor discovery information for each discovered neighbor of NW-TT |  |  |  |  |
| lldpV2RemChassisIdSubtype |  | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId |  | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype |  | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortId |  | X | R | IEEE 802.1AB [97] Table 11-2 |
| TTL |  | X | R | IEEE 802.1AB [97] clause 8.5.4 |
| Neighbor discovery information for each discovered neighbor of DS-TT (NOTE 5) |  |  |  |  |
| lldpV2RemChassisIdSubtype | D | N | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | D | N | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | D | N | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | D | N | R | IEEE 802.1AB [97] Table 11-2 |
| TTL | D | N | R | IEEE 802.1AB [97] clause 8.5.4.1 |
| Per-Stream Filtering and Policing information (NOTE 10) |  |  |  |  |
| Stream Filter Instance Table (NOTE 8) |  |  |  | IEEE 802.1Q [98] Table 12-32 |
| StreamHandleSpec | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| PrioritySpec | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| StreamGateInstanceID | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| Stream Gate Instance Table (NOTE 9) |  |  |  | IEEE 802.1Q [98] Table 12-33 |
| StreamGateInstance | X | X | R | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminBaseTime | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminControlList | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminCycleTime | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPTickGranularity | X | X | R | IEEE 802.1Q [98] Table 12-33 |
| NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: Indicates which standardized and deployment-specific port management information is supported by DS-TT or NW-TT. NOTE 3: AdminCycleTime and AdminControlListLength are optional for gate control information. NOTE 4: If DS-TT supports neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to DS-TT. If DS-TT does not support neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to NW-TT and NW-TT performs neighbor discovery on behalf on DS-TT. NOTE 5: If DS-TT supports neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from DS-TT. If DS-TT does not support neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from NW-TT, which performs neighbor discovery on behalf on DS-TT. NOTE 6: X = applicable; D = applicable when validation and generation of LLDP frames is processed at the DS-TT; N = applicable when validation and generation of LLDP frames is processed centrally at NW-TT. NOTE 7: NW-TT uses Static Filtering Entry information to determine the NW-TT egress port for forwarding UL TSC traffic. NOTE 8: There is a Stream Filter Instance Table per Stream. NOTE 9: There is a Stream Gate Instance Table per Gate. NOTE 10: In this release the PSFP information is used only at the TSN AF. TSN AF uses the PSFP information at TSN bridge configuration time to identify the DS-TT MAC address of the PDU Session as described in clause 5.28.2 and for determination of the TSCAI information as described in Annex I. |  |  |  |  |

Exchange of port management information between TSN AF and NW-TT or DS-TT allows TSN AF to:

1)	retrieve port management information for a DS-TT or NW-TT Ethernet port;

2)	send port management information for a DS-TT or NW-TT Ethernet port;

3)	subscribe to and receive notifications if specific port management information for a DS-TT or NW-TT Ethernet port changes.

Exchange of port management information between TSN AF and NW-TT or DS-TT is initiated by DS-TT or NW-TT to:

-	notify TSN AF if port management information has changed that TSN AF has subscribed for.

Exchange of port management information is initiated by DS-TT to:

-	provide port management capabilities, i.e. provide information indicating which standardized and deployment-specific port management information is supported by DS-TT.

TSN AF indicates inside the Port Management Information Container whether it wants to retrieve or send port management information or intends to (un-)subscribe for notifications.

***END OF CHANGES***