3GPP TSG SA WG2 Meeting #139E	S2-2003689

# Elbonia, June 1 – 12, 2020	(Revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.737 | CR | 0006 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sol#1: Updates to Solution#1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2020-05-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | Solution#1 describes a basic solution for mobility managemet based on earth-fixed Tracking Areas. The solution describes a possibility that a cell in a satellite RAN is smaller than the beam, and that RAN needs to position the UE in order to determine which cell the UE is slocated in. However, based on the reply LS from RAN2 in R2-2004266, a call cannot be smaller than the satellite beam (“Furthermore, it is not possible to define cells smaller than a satellite beam (Section 7.3 TR38.821).”) There should thus be no need for positioning to determine the cell, as regular cell/beam selection should be sufficient. Also a few other minor updates are needed, e.g. to clarify that RA is not selected by the UE but by AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove option of a cell being smaller than a beam. Miscellaneous corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with RAN assumptions and incorrect description of RA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.1.1, 6.1.3, 6.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** First change ***************

## 6.1	Solution #1: Position-based and fixed TA Satellite Access

### 6.1.1	Description

This is a candidate solution for Key Issue 1, 2 and 6.

As illustrated in figure 6.1.1-1, it is assumed that the satellite(s) whether Non-Geostationary (NGSO) or geostationary (GEO) are connected to a gNB (located on ground on the figure). The gNB is connected to a CN. Aspects relating to motion of the satellite(s), beam steering, etc. are addressed in the RAN.

For non-geostationary-satellites, with moving platforms which satellite orbits are accurately determined and anticipated, satellites beams generate a fixed pattern on ground thanks to a tracking scheme so as to generate a fixed radio coverage towards the UE. This can be implemented with combined electronic and/or mechanical steering of the antennas, as this is already in operation for some satellites.

The size of the beam increases with the altitude of the satellite as the antenna diameter of the satellites is limited by technology. The beam radius can range for instance from tens of kilometres for LEO satellite, to hundreds of kilometres for MEO or GEO satellites.

NOTE:	As this solution relates to the coverage of the satellite, it does not make any assumption on whether the gNB is located on-board the satellite or on-ground. Interconnection of the satellites with ISLs is also possible

Figure 6.1.1-1: Satellite integration in the RAN

A - Principle of the fixed Tracking Areas (TAs) - As for terrestrial networks, Tracking Areas are defined as a set of cells. TAs as well as Cells are fixed with respect the surface of Earth. For NGSO satellites, as time goes, the mapping between gNB and satellite beams will change. For geostationary-satellites (GEO), satellite beams are fixed and generate a fixed radio coverage pattern on Earth.

In both the GEO & NGSO satellite case, there is thus no need to update the CN (AMF) with new mappings between cells/TAs and gNBs.

It is assumed that the cell cannot be smaller than the beam size and that a cell either consists of a single beam or multiple beams, as described in TR 38.821 [7] and RAN2 LS R2-2004266.

NOTE: 	In case a beam/cell covers a larger area than can be tolerated due to e.g. regulatory requirements for beam/cell coverage across country borders, a complementary solution is needed. Such complementary solutions are addressed under Key Issue#10.

This principle is illustrated in Figure 6.1.1-2. In this particular example, TA #1 and TA #2 are defined with respect to the case of beam spots of a fixed (GEO) satellite system: for each of these TA's, a number of fixed cells and satellite beams are associated with a TA. On the same figure are shown (smaller) moving beam spots from a non-geostationary moving satellite. In this case, it does not impact the definition of TAs, as TAs are independent from moving beams.

This definition shall take into account the accuracy of the position estimate system used by the satellite access to locate the UEs.

Figure 6.1.1-2: Satellite beams for Geostationary and Non-Geostationary satellites, cells and tracking areas

Figure 6.1.1-3 illustrates also the handover mechanism from one satellite to another one to guarantee the continuity of service for moving gNBs when embarked on non-geostationary satellites. As the ephemerides of the satellites are mastered by the satellite operator, it is possible to guarantee, thanks to an optimised design of the satellite constellation of the payload, that cells and TAs are always served by at least one active beam. This implies that during handover period, a cell is visible by at least two satellites at the same time. This also implies that whilst for a geostationary-satellite a TA will be served by the same satellite, a TA may be served by different NGSO satellites.

The operating mode is such that when a satellite is not in the position to serve a cell through its steerable beams, there shall be as a minimum another incoming satellite to replace this outgoing satellite, serving the corresponding cell.

Figure 6.1.1-3: Non-Geostationary Satellite Beam steering and Handover

B - Principle for the mobility registration update

Mobility

-	The UE makes cell selection based on similar procedures as for terrestrial NG-RAN. Based on the selected cell, the UE determines the Tracking Area it is located in. The AMF determines Registration Area as per existing specification.

NOTE:	Cell selection and beam selection aspects should be specified by TSG RAN.

C - Principle for CN Paging - The 5GC still pages UE in CM-IDLE within the allocated Registration Area, i.e. a list of TAs.

### 6.1.2	Procedures

No changes are identified to the procedures.

### 6.1.3	Impacts on existing nodes and functionality

UE and 5GC: No impacts to mobility management are identified.

NOTE:	If needed, the specific UE position determination mechanism should be specified during the normative phase by TSG RAN.

### 6.1.4	Solution evaluation

This solution is applicable for Key Issue 1, 2 and 6.

In this solution, the NG-RAN is assumed to handle all aspects with respect to the motion of the satellite (including beam steering with the motion of non-geostationary satellites).

This solution proposes an approach which does not modify the definition of TAs and RAs. No impacts are identified on the CNs procedures or functional elements.

*************** End of changes ***************