3GPP TSG SA WG2 Meeting #139-e		S2-2003757

Electronic, 01 – 12 June 2020

Title:	LS on clarification on TSN for Vertical_LAN

Response to:	C3-202515 / S2-2003546

Release:	Rel-16

Work Item:	Vertical_LAN

Source:	SA2

To:	CT3

Cc:	-

Contact person:	Sašo Stojanovski

saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2003244

# 1	Overall description

SA2 thanks CT3 for their LS in C3-202515/ S2-2003546.

SA2 would like to provide the following answers to the questions in the LS.

1) What’s the criteria in the PCF to select the TSN AF that receives the first Npcf_PolicyAuthorization_Notify service operation? Is it bound to the 5GS Bridge Identity handling the PDU session? Is there only one TSN AF shall be selected for all the PDU sessions corresponding to a 5GS bridge?

A1: When no AF session exists for a PDU session yet, the PCF uses a pre-configured AF session with a pre-configured AF to deliver the Npcf_PolicyAuthorization_Notify message, as described in TS 23.502 clause 4.16.5.1. Please note that this clause has been updated by the attached CR (S2-2003244).

It is assumed that all PDU sessions which connect to the same TSN network via a specific UPF (i.e. via the same 5GS bridge) are handled by the same TSN AF (refer to the NOTE in TS 23.501 clause 5.28.1).

2) After receiving the first Npcf_PolicyAuthorization_Notify service operation with new 5GS Bridge information, the TSN AF triggers an Npcf_PolicyAuthorization_Create service operation. How does the TSN AF find the PCF? Is the PCF Discovery and Selection function modified?

A2: As explained in the answer to the previous question, it is assumed that all PDU sessions which connect to the same TSN network via the same 5GS bridge are handled by the same TSN AF.

# 2	Actions

To CT3

ACTION: 	SA2 kindly asks CT3 to take into consideration the feedback provided above.

# 3	Dates of next TSG SA WG 2 meetings

SA2#140E		19 August - 2 September			Electronic meeting