SA WG2 Meeting #139e	S2-2003705

# June 1th-12th, 2020 ; Elbonia        		                						 	(revision of S2-2003705)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2371 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | URLLC – TSN interworking with ETSUN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN, VERTICAL_LAN, 5G_URLLC |  |  |  |  |  | Date: |  |  | 2020-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CT4 LS on which features cannot interwork with ETSUN C4-202536/ S2-2003549 CT4 kindly asks SA2 group to clarify which features cannot interwork with ETSUN and add the clarification in relevant specification if needed, and to confirm in particular whether URLLC applies to PDU session with I-SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In this release of the specification, support for Time Sensitive Communications (as defined in clause 5.27 or 5.28) operates without impact from ETSUN and Ultra Reliable Low Latency Communications (as defined in clause 5.33.2.2) assumes SMF Service Areas covering the whole PLMN or that a PDU Session supporting Ultra Reliable Low Latency Communicationsin 5.33.2.2 is released by the SMF when the UE moves out of the SMF Service Area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unspecified system behavior when a PDU Session supporting Time Sensitive Communications or Ultra Reliable Low Latency Communications involvres an I-SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R01 |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.34.1	General

When the UE is outside of the SMF Service Area, an I-SMF is inserted between the SMF and the AMF. The I-SMF has a N11 interface with the AMF and a N16a interface with the SMF and is responsible of controlling the UPF(s) that the SMF cannot directly control. The exchange of the SM context and forwarding of tunnel information if needed are done between two SMFs directly without involvement of AMF.

Depending on scenario, a PDU Session in non-roaming case or local breakout is either served by a single SMF or served by an SMF and an I-SMF. When a PDU Session is served by both an SMF and an I-SMF, the SMF is the NF instance that has the interfaces towards the PCF and CHF.

The SMF shall release or reject the PDU Session if the DNN of the PDU Session corresponds to a LADN and the I-SMF is inserted to the PDU Session.

NOTE 1:	This implies that operators need to plan the LADN deployment in such a way that the LADN Service area needs to be within the SMF Service Area, but not across SMFs' Service Areas.

NOTE 2:	This is to cover the case where the UE is not in or moves out of SMF Service Area and an I-SMF is inserted to the PDU Session e.g. during PDU Session Establishment, Service Request. If the PDU Session is maintained with I-SMF, the SMF is not be able to enforce the LADN Service control, e.g. SMF is not notified in the case of Service Request.

Independent of whether an I-SMF is present or not, the SMF may trigger the PDU Session re-establishment to the same DN, if the PDU Session is associated with the SSC mode 2 or SSC mode 3.

NOTE 3:	SSC mode 2 or SSC mode 3 can be used to optimize SMF location for a PDU Session and/or, depending on deployment, ensure that the UE is always within the service area of the SMF controlling the PDU Session. In this case (when PDU Session continuity over the PLMN is not required) procedures described in this clause are not needed.

TSC (as defined in clauses 5.27 and 5.28) is not expected to use PDU sessions requiring the insertion of an I-SMF, hence no dedicated functionality is specified for I-SMF and N16a in order to support TSC.

Redundant User Plane Paths as defined in clause 5.33.2. is not supported for PDU Sessions involving an I-SMF.

In this release, QoS monitoring (as defined in clause 5.33.3) is supported as long as SMF and not I-SMF initiates the QoS monitoring function.

Dynamic CN PDB provisioning (as defined in clause 5.7.3.4) is supported for PDU Sessions involving an I-SMF.

In this release, no dedicated functionality is specified for I-SMF and N16a in order to support NPN.

NEXT CHANGE (2)

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES