3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005415r34

Elbonia, August 19 – September 2, 2020	(revision of S2-200xxxx)

Title:	[DRAFT] LS on RAN impact of FS_5MBS Study

Release:	Release 17

Work Item:	FS_5MBS, NR_MBS-Core

Source:	[Huawei to be] SA2

To:	RAN2, RAN3

Cc:

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TR 23.757 v0.4.0 (version before SA2#140e)

Note: TR 23.757 v0.5.0 containing the results of SA2#140e will be available shortly after SA2#140E

1. Overall Description:

SA2 has been discussing on Rel-17 FS_5MBS solutions since SA2 #135 meeting in TR 23.757, and captured several proposed solutions in clause 6, architecture options in Annex A, and some (partial) conclusions and interim conclusions in clause 8.

SA2 would like to kindly inform RAN2 and RAN3 the following interim agreements in SA2:

-	SA2 will provide means to provide QoS requirements for an MBS Session to RAN nodes.

-	SA2 agrees that for each MBS session an own shared N3 tunnel will be used for DL MBS data delivery from MB-UPF to NG-RAN nodes. This tunnel can use either IP multicast transport (NG-RAN sends IGMP/MLD Join/Leave to a multicast router) or point-to-point unidirectional shared N3 tunnels from MB-UPF to NG-RAN nodes.

-	SA2 agreed that the UE shall be able to receive on-going data of a multicast MBS session while in CM-CONNECTED state..

-	Based on SA plenary decisions, Key Issue #5 ("Support of Broadcast TV Video and Radio communication services"), is out of scope of Rel-17.

SA2 would like to inform that no solution or combination of solutions has been adopted yet, and would like to receive feedback from RAN2 and RAN3 on the RAN impacts produced by some of the company proposed solutions, and for some of the solutions documented in current TR, SA2 would like to kindly ask RAN2 and RAN3 the following questions:

- There are different proposals how to handle the CM-IDLE/CM-CONNECTED state transitions:

  - UE within an multicast session shall stay in CM-CONNECTED state

  - UE can receive data of a multicast session also while in CM-IDLE state

  - UEs can transition into CM-IDLE while no multicast data are transmitted.

  - Some solutions propose that 5G CN may trigger notification (e.g. “paging”) to UEs for establishing transmission resources for a MBS session when data of an MBS session are ready to be delivered.

  - Some solutions propose that the MBS session can be deactivated by network while no multicast data are transmitted to save power.

  - Some solutions propose that the network can activate the MBS session and trigger notification to UEs when multicast data are transmitted again

SA2 would appreciate RAN2 and RAN3 feedback on the above and comments on the input required from 5GC if needed.

- Some Xn/N2 handover solutions in the SA2 study documented in the TR.

  - Some solutions consider to have temporary MBS data forwarding from S-RAN to the T-RAN, to address potential data loss or duplication in case of a UE moving to a T-RAN supporting 5MBS.

  - Some solutions consider MBS Session may have service area restriction, this may impact the Xn/N2 handover procedure.

SA2 would appreciate RAN2 and RAN3 feedback on these solutions.

- SA2 is debating whether broadcast (for other use cases than Broadcast TV) is in scope of Rel-17. SA2 would like to ask RAN2 and RAN3 for feedback on RAN aspects of broadcast support in Rel-17.

SA2 would like to reiterate that none of the solutions with RAN impacts as described above have been concluded as way forward, and intends to use RAN WGs feedback on the RAN considerations, together with SA2’s own system level and core network considerations to reach final conclusions.

2. Actions:

To RAN2 and RAN3 group.

ACTION:

SA2 respectfully requests RAN2 and RAN3 to take the above information into account and to answer the questions raised by SA2 and to provide any other feedback to SA2.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #141E		October 12 – 23, 2020			Elbonia

TSG-SA2 Meeting #142E		November 16 – 20, 2020			Elbonia