3GPP TSG SA WG2 Meeting #140-e		S2-2005689r`12

Electronic, 19 Aug – 02 Sep, 2020

Title:	LS on System support for Multi-USIM devices

Response to:

Release:	Rel-17

Work Item:	FS_MUSIM

Source:	SA2

To:	RAN2, RAN3

Cc:	-

Contact person:	Sašo Stojanovski

saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 would like to inform RAN2 and RAN3 that they have progressed the study on FS_MUSIM (TR 23.761) and are discussing  interim conclusions on all three key issues.

SA2 point out that these interim conclusions are subject to further revision taking into account feedback received from other WGs.

The table below lists the principles that are considered for further study and have RAN impact or require RAN feedback. The table also contains the applicability to key issues in TR 23.761, as well as questions to RAN WGs.

As a reminder, these are the KIs in SA2 study:

- 	Key Issue #1: definition of mechanism for handling of different MT services differently

-	Key Issue #2: resolution of paging conflicts

-	Key Issue #3: How to gracefully signal that a USIM is not reachable temporarily or until further notice on a PLMN

| Solution principle for further study in SA2 | Applicability to Key Issues | Question to RAN WGs |
| --- | --- | --- |
| Paging Cause (as in Solution #1) | SA2’s KI#1. | Q1: Please confirm feasibility of sending a Paging Cause in [Uu] Paging message for EPS and for 5GS. [RAN2, RAN3] Q2 please advise if it would be possible to send a plurality of page causes in a single paging message and if so wha tis the limit of the number of such paging causes Q3: please advise whether it can be assumed all the RAN nodes in a PLMN support the Paging cause, or whether this is supported only by a smaller set that has been upgraded (maybe per Tracking area? Or can this be advertised in SIB?) Qx: Please can RAN advise on the number of paging causes that can be supported in the paging message for all RATs. |
| Busy indication (as in Solution #3) | SA2’s KI#1. | Q4: Please indicate whether it is desirable that the UE always responds to paging to stop paging and avoid paging escalation or needless repetition. [RAN2, RAN3] Q5: Solution 3 proposes to send the Busy Indication as a NAS message which requires an RRC connection. Please provide feedback if it is feasible (and secure) if the Busy Indication is sent at RRC instead i.e. without requiring an RRC connection [RAN2, RAN3] Q6: what is the time expected to be needed for establishing a RRC connection, sending a NAS busy indication, with an assistance info to immediately release the RRC connection and assume the UE is leaving the system |
| RRC-based leaving and returning with the following assumptions: A) - Leaving is always triggered by the UE with an RRC request to the network. - When leaving the UE enters RRC Inactive. The UE may return while still in RRC Inactive using RRC signalling. However if the UE does not return for an extended time period, it enters RRC Idle after which it may return using NAS signaling. NOTE 1: the duration of the extended time period is subject to RAN decision B) - In addition to the above, gaps while RRC Connected are considered, to allow the UE to leave temporarily without interrupting the RRC Connection. NOTE 2: This is considered an Access-Stratum only solution, subject to RAN decision only. Whether the gaps are negotiated or not and their duration is FFS. - The UE uses the above to perform a MO procedure (e.g. periodic mobility registration, keep-alive message, sending busy indoication, etc.) or a MT procedure (e.g. pick-up an SMS, inspect a MT service invite, respond to a network-initiated C-plane procedure, etc.) in the other network. - | SA2’s KI#1 and KI#3. | Q7: Please indicate whether it is feasible and desirable to define an RRC-based leaving and returning procedure in 5GS as per A). [RAN2, RAN3] Q8: Please indicate whether it is feasible and desirable to define gaps in RRC Connected in 5GS as per B). we should obtain from rAN WGs indication of a maximum achievable gap . Q9: Please let us know whether changes to 5GS/E-UTRA (Option 5) to support A) and/or B) may be considered in view of the current RAN Work Item. [RAN2, RAN3] Qx: Can the UE autonomously leave by a given time if no acknowledgement is received from the network without causing undue RRC state mismatch? Qx: Can the UE be configured to enter RRC Inactive autonomously, considering the case that no acknowledgement is received from the network? Qx: Is there any restrictions on the duration for the extended time period? Qx: Please can RAN provide feedback on whether gaps can be negotiated and whether their duration can be negotiated? |

SA2 would also like to point out that TR 23.761 also contains several solutions to KI#2 requiring  RAN’s feedback. The solution principles in these solutions can be categorized as follows:

-	UE -requested 5G-GUTI reassignment using the Mobility Registration Update procedure (as in Sol#14 and Sol#20). The 5G-GUTI is systematically reassignment by the network during the Mobility Registration Update procedure requires.

-    Changes related to the UE_ID (UE Identity Index) that is used for calculation of PF/PO. In EPS the UE Identity Index is provided by the MME to the eNB as part of the paging message. In 5GS the UE Identity Index value is provided by the AMF in the RRC-Inactive Assistance information but not in the paging message:

-    Calculation of PF/PO by using an Alternative UE_ID which is not derived from either the UE’s GUTI or IMSI. The UE ID sent in the paging message is not impacted by this Alternative ID that is only used for PO/PF calculations (Sol#15). Proposed for both EPS and 5GS.

-    Calculation of PF/PO by using an UE_ID which is derived from IMSI+offset value. The offset value negotiated between UE and MME (Sol#16). Proposed for EPS only.

-    Calculation of PF/PO based on MUSIM Assistance Information (Sol#17).

-     Paging on consecutive POs as in Sol#18. For this solution SA2 was not clear whether the network requires an explicit MUSIM capability so that a different paging strategy can be applied to MUSIM- and non-MUSIM capable devices.

-	Implementation-based solution to address overlapping POs (Sol#19).

-	Access Stratum-based solution with scheduling gap (Sol#21).

Q10: SA2 would like to ask RAN2 whether Solutions #14 to #20 are feasible for effective avoidance of paging collision for the RAT combination scenarios to which each of them applies.

NOTE:	While the 5G-GUTI reassignment principle in Sol#14 and Sol#20 has no RAN impact, SA2 welcomes any RAN2 feedback on solution 14 which use no assistance information from the UE in order to avoid PO collisions.

Qx: Solution 18 increases the use of paging resources especially for UE with idle mode mobility. SA2 would like to ask RAN2 to provide feedback on the increased use of paging resources in the system for UEs which require it.

Qxx: SA2 would like to ask RAN2 and RAN3 to take these solutions into consideration and provide any other feedback or recommendation on the applicability of these solutions, or any other Access Stratum-based solutions for KI#2.

# 2	Actions

To RAN2, RAN3

ACTION: 	SA2 kindly asks RAN2 and RAN3 to take into consideration the information above and provide answers to questions as indicated in the table and to the questions on paging collision avoidance.

# 3	Dates of next TSG SA WG 2 meetings

SA2#141E		12-23 October				Electronic meeting

SA2#142E		16-20 November			Electronic meeting