3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-200xxxx

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	23.700-07 KI#2 QoS considerations and conclusion proposal

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN /Rel-17

Abstract: This document presents considerations on QoS support for KI#2 scenario and removal of EN in conclusion proposing that R16 specification is with minor improvements.

# 1. Discussion

## 1.1 QoS aspects

In last meeting we discussed lengthy the support of QoS in KI#2 and we agreed the following EN

Editor's note:	With Rel-16 N3IWF architecture, whether it can ensure that the VIAPA applications obtains QoS in the underlay network, and if not, how to enhance the Rel-16 N3IWF architecture, is FFS.

The QoS support in IKEv2 for connection to N3IWF is based on mapping of 5G QoS to Child SA granularity for avoiding reorder issue and enabling external tunnel marking. This is performed per R15 and R16 specification

- Based on its N3IWF policies and configuration, and based on the QoS profiles received, the N3IWF shall determine the number of IPsec Child SAs to establish and the QoS profiles associated with each IPsec Child SA ,(e.g. one IPsec Child SA and associate all QoS profiles).

- Child SA request includes a 3GPP-specific Notify payload which contains (a) the QFI(s) associated with the Child SA, (b) the identity of the PDU Session associated with this Child SA, (c) optionally, a DSCP value associated with the Child SA, (d) optionally a Default Child SA indication, and (e) optionally, the Additional QoS Information

    - a non-GBR flow: QoS Characteristics defined in TS 23.501 [2], clause 5.7.3,

    - For GBR flow: The GBR QoS Flow Information (defined in TS 38.413 [10]) is part of the QoS Profile received via the N2 interface and contains: MFBR, GFBR, optionally a Notification Control, and optionally Maximum Packet Loss Rate. The Notification Control is not used by the UE

- the UE shall use the Additional QoS Information contained in this message to determine what QoS resources to reserve over the non-3GPP access, including e.g. guaranteed bit rates and delay bounds for UL/DL communication. How the UE determines what QoS resources to reserve over the non-3GPP access and how these QoS resources are reserved, is outside the scope of 3GPP specifications.

In R16 the specification for the scenario related to KI#2 defines that (text extract from the spec)

  - QoS differentiation in the SNPN can be provided on per-IPsec Child Security Association basis by using the UE or network requested PDU Session Modification procedure described in TS 23.502 [3] clause 4.3.3.2. In the PLMN, N3IWF determines the IPsec child SAs as defined in TS 23.502 [3] clause 4.12. The N3IWF is preconfigured by PLMN to allocate different IPsec child SAs for QoS Flows with different QoS profiles.

  - To support QoS differentiation in the SNPN with network-initiated QoS, the mapping rules between the SNPN and the PLMN are assumed to be governed by an SLA including: 1) mapping between the DSCP markings for the IPsec child SAs on NWu and the corresponding QoS, which is the QoS requirement of the PLMN and is expected to be provided by the SNPN, and 2) N3IWF IP address(es) in the PLMN. The non-alteration of the DSCP field on NWu is also assumed to be governed by an SLA and by transport-level arrangements that are outside of 3GPP scope. The packet detection filters in the SNPN can be based on the N3IWF IP address and the DSCP markings on NWu.

  - To support QoS differentiation in the SNPN with UE-requested QoS, the UE can request for an IPsec SA the same 5QI from the SNPN as the 5QI provided by the PLMN. It is assumed that UE-requested QoS is used only when the 5QIs used by the PLMN are from the range of standardized 5QIs. The packet filters in the requested QoS rule can be based on the N3IWF IP address and the SPI associated with the IPsec SA.

It should be further considered that so far it is assumed per SA1 requirement that SNPN is stand alone and that the device accessing both PLMN and SPNP uses different credential. The R16 assumption has been not modified for R17, hence R16 allows the UE to request to the appropriate network the QoS modification using the PDU session modification procedure. The network receiving the request based on user subscription, configuration, etc can accept or reject the QoS modification, so from this point of view this allows to provide the suitable QoS if negotiated among the parties, i.e. in presence of a SLA, without any need of interface exchanging in real time information, as it can done for roaming.

For the 4G in order to help roaming, GSMA has suggested some mapping between 3GPP QCI and the DSCP marking, for 5G it may be suggested in informative annex a possible mapping, however it should be noted that the upper PDU session (in figure 1 for PLMN) can be Ethernet PDU session carried over IKEv2 IP where the mapping is done by N3IWF (in figure 1 of PLMN) according to configuration of this home operator. Then this traffic is mapped considering the IP DSCP marking and the relationship with Ethernet is assumed to be implicitly done properly by home network.

Proposal #1: The R16 QoS management is considered sufficient for addressing VIAPA QoS coordination via UE taking care of it, when it is needed, and whether SLA exists between SNPN and PLMN the suitable QOS profile can be configured in UE user subscription and/or in PCF and/or in N3IWF/SMF/UPF, so no further standardization is required for this release.

Figure 1:

## 1.2 Evaluation clean up

The evaluation session has several reference which has bene not editorially renumbered to the solution number used in the latest version of the TR. In addition some references were for documents not approved.

## 1.3 Session continuity

Regarding session mobility It is proposed a revision of text for clearly states that analyse and conclusions are done as part of KI#1 in order to avoid confusion.

# 2. Text Proposal

It is proposed to capture the changes reported in the following

.

# * * * * First change * * * *

## 7.2	Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)

The evaluation is based on major principles from the KI#2 solutions. The evaluation separates to different areas for each principle as below table:

Table 7.2-1: Evaluation of KI#2 related principles

| Principle | What it solves | Benefit, drawback | System impact | Open issues | Reference |
| --- | --- | --- | --- | --- | --- |
| N3IWF architecture | Service continuity aspect. Simultaneous data service aspect. | Benefit: Drawback: | Based on the existing release 16 architecture, No new architecture impact |  | Solution #13, #14, #15, #17, #18, #48, #50. |
| N3IWF architecture with dual radio capability (2RX/2TX) UE enhancement. | Same as N3IWF architecture. | Benefit: Drawback: |  |  | Solution #13, #15, #50 |
| N3IWF architecture with application layer service continuity support | Same as N3IWF architecture | Benefit: Drawback: |  |  | Solution #48, #49 |
| N3IWF architecture with enhancement to keep UE in CM-Connected state in overlay network. | Paging aspect. | Benefit: Drawback: | UE logic to keep one PDU session in underlay network. Existing liveness check tool to be used over NWu interface to keep the NWu connection open. Apply to single radio UE |  | Solution #13, #14, #50 |
| N3IWF architecture with enhancement to keep UE in CM-Connected state in both overlay network and underlay network. | Paging aspect. | Benefit: Drawback: | RAN and CN impact to keep UE always in RRC-Inactive in the underlay network. PDU session in underlay network remains active. Support single radio UE | Whether it is necessary to keep UE in CM-Connected in underlay network is FFS. | Solution #14 |
| N3IWF architecture with enhancement to introduce "inactive" mode on NWu interface. | Paging aspect. | Benefit: Drawback: |  | IKEv2 impact is FFS. How does the DL IPSec data reach UE when NAT timeout is FFS. | Solution #17 |
| N3IWF architecture with enhancement to split DL traffic to Uu interface and UL traffic to NWu interface for 2RX/1TX UE. | Simultaneous data service aspect. | Benefit: Drawback: | RAN impact for uplink control communication with single Tx UE/AMF impact to support split indication. UE/SMF/UPF impact to support user data plane traffic path steering. |  | Solution #18 |
| N3IWF architecture with enhancement to assist UE to choose either SNPN or PLMN to use Uu interface for better QoS. | Simultaneous data service aspect. | Benefit: Drawback: |  |  | Solution #46, #47, #50 |
| Inter-PLMN like architecture between SNPN and PLMN. | Service continuity aspect. | Benefit: Drawback: | To be concluded from KI#1 evaluation. |  | Solution #1, #2, #50. |
| N3IWF architecture with MUSIM like implementation | Concurrent paging and data service for single radio UE. | Benefit: Drawback: |  | The principle is not clearly stated in the solution. | Solution #15, #50, #52. |
| Inter-PLMN like architecture between SNPN and PLMN with enhancements to enable SNPN operator to control access to PLMN services | Simultaneous data service aspect |  | Network impacts for control the access to PLMN services using a HR PDU session or a secondary registration at PLMN |  | Solution #50 |

The R16 QoS management is considered sufficient for addressing VIAPA QoS coordination via UE taking care of performing QoS request Via PDU session modification to the appropriate PDU session, when it is needed. Whether a SLA exists between SNPN and PLMN the relevant QoS profile is configured in UE user subscriptions in each network  and/or in PCF and/or in N3IWF/SMF/UPF of SNPN and PLMN, so no further standardization is required for this release.

# * * * * Second change * * * *

## 8.2	Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)

Editor's note:	These are INTERIM conclusions for Key issue #2.

When UE only has single subscription, the data service from both V-SNPN and Home SP (PLMN or SNPN), as well as service continuity is to be evaluated and concluded by KI#1.

It is concluded that the existing Rel-16 N3IWF-architecture is used as the basis to address data service from both networks and service continuity between the two networks.

Rel-16 N3IWF architecture can ensure that the VIAPA applications obtains QoS in the underlay network via UE requesting using existing PDU session modification the requested QoS which will be provided if allowed by subscription local configuration and network load condition per current R16 specification.

# * * * * Third change * * * *

# * * * * End of changes * * * *