SA WG2 Meeting #141E (e-meeting)	S2-2006835r04

Oct 12 – Oct 23, 2020, Elbonia	(merger of S2-2006835, S2-2006895, S2-2007465, and S2-2006973)

Source:	CATT, vivo?, Huawei, HiSilicon?, OPPO?

Title:	KI #7 evaluation

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes the evaluation for Key Issue #7.

- Discussion

For Key Issue #7 "Reliable delivery method switching between unicast and multicast", there are the following solutions in the TR:

-	Solution #11, #12, #18, #22, #23, #24, #25, #26, #27,#28, #29, #30,#31, #39, and #40.

These solutions can be categorized as follows:

- Switching between 5GC Shared and Individual MBS traffic delivery methods: Solution #23, #24, #25, #27, #28, #31, and #40 (single RAN case).

- Switching between PTP and PTM delivery methods: Solution #18, #22, #24, #25, and #30.

- Inter-RAN handover / UE mobility related delivery method switching: Solution #11, #12, #26, #27, #29, and #40 (dual RANs case).

- Switching between unicast and multicast: Solution #24, #28, #31, #39, and #40.

Evaluations are made on the solutions for each category.

# 2. Proposal

It is proposed to include the following changes in TR 23.757.

***** Start of 1st Change (all new text) *****

7.X	Key Issue #7: Reliable delivery method switching between unicast and multicast

There are 15 candidate solutions proposed to address key issue#7, i.e. solution#11/#12/#18/#22/#23/#24/#25/#26/#27/ #28/#29/#30/#31/#39/#40, these solutions can be categorized as follows:

- Switching between unicast and multicast: Solution #24, #28, #31, #39,  and #40 (single RAN).

- Non inter-RAN handover / UE mobility related delivery method switching  :

  - Switching between 5GC Shared and Individual MBS traffic delivery methods: Solution #23, #24, #25, #27, #28, #31, and #40 (single RAN case).

  - Switching between PTP and PTM delivery methods: Solution #18, #22 ,#24, #25, and #30.

- Inter-RAN handover / UE mobility related delivery method switching: Solution #11, #12, #26, #27, #29, and #40 (dual RANs case).

Following are the evaluations on the solutions for each category:

1) Switching between unicast and multicast

Solution# (28, 31, 39, and 40) propose the UE can trigger the delivery method switch.

From UE view it can only differentiate the multicast/unicast switch (or vice versa). All the proposal is that UE receive the information from application layer or RAN information, the delivery mode is triggered to be switched. This is related to application logic.

For the AF involved method switch, it is the multicast /unicast switch. It need involve UE, e.g. establish the related MBS session context at the UE side. So it is same as the UE trigger mode switch, i.e. per application logic.

As all this is related to service logic, it is not need to be specified in normative work.

2) Non inter-RAN handover / UE mobility related delivery method switching

A) Switching between 5GC Shared and Individual MBS traffic delivery methods

A comparison between solutions for switching between 5GC Shared and Individual MBS traffic delivery methods is shown in Table 7.X-1.

Table 7.x-1: Comparison of solutions for switching between 5GC Shared and Individual MBS traffic delivery methods

|  | Sol#23 | Sol#24 | Sol#25 | Sol#27 | Sol#28 | Sol#31 | Sol#40 (single RAN case) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Delivery method switching triggers/ criteria | - Application level trigger. - QoS requirements. - UE MBS subscription changes. - Handover or reselection to target RAN supporting or not supporting MBS. - Predefined threshold (i.e. UE number) | - Content/ Application Server decisions. - Handover. - UE population. | - Number of the UEs receiving the multicast service in a certain multicast group | -After Inter-RAN mobility. When UE move from one gNB not supporting MBS to a gNB supporting MBS, the SMF trigger the mode switch. | - Number of devices receiving the specific content. - Number of devices in a specific location area related to the specific content. -Measurement of multicast signalling strength | - MBS service related network policy, (e.g. predefined traffic threshold, network status /performance changes); - UE subscription for MBS service changes. - UE moving in or out of MBS Service Area; - Change of user preferences | - UE MBS subscription changes. - Number of UEs requesting the same content in a specific area |
| NFs triggering delivery method switching | UE, SMF, MSF | RAN, MBSF, Content/Application Server | (MB-)SMF | SMF | UE, (MB-)SMF | UE, RAN, AMF, SMF, UDM, PCF, AF | UE, SMF |
| Procedures for delivery method switching | UE or network initiated PDU Session Establishment or PDU Session Modification procedure | MBSF initiated session update procedure | SMF/UPF initiated new procedure | SMF triggered MBS session establishment procedure. | UE initiated User Multicast Session Join / User Multicast Context Update procedure | PDU Session Modification procedure | - UE triggered PDU Session Modification procedure. - SMF triggered new procedure |
| Baseline architecture used | 1 | 2 | 2 | 1 | 1 | 1 | 1 |

- Triggered by the UE

Solution #23, #28, #31 and #40 propose that the UE can trigger the switching between 5GC Shared and Individual MBS traffic delivery methods, due to trigger conditions including changes of user preferences or UE MBS subscription, moving into/out of the multicast area, 5MBS capability of target gNB during handover, radio signalling condition, etc.

Solution #24, #28, #30 and #31 propose that the RAN can trigger the switching between 5GC Shared and Individual MBS traffic delivery methods, due to trigger conditions including UE consumption reports, 5MBS capability of target gNB during handover, number of UEs/PDU Sessions for the same MBS service, etc.

Solution# (23, 24, 25, 27, 28, 31, and 40) propose the delivery method switch from the 5GC, i.e., the shared delivery method to the individual delivery method switch.

Solution#23, #25,  #27, #28, #31 and #40 propose that the (MB-)SMF, PCF, SMF, UDM or AMF triggers the switching between 5GC Shared and Individual MBS traffic delivery methods.

- Solution #23, #25, #28 and #40 proposes that the (MB-)SMF triggers the switching between 5GC Shared and Individual MBS traffic delivery methods based on number of the UEs receiving the multicast service in a multicast group or in a specific location area (i.e. a UE count threshold).

- Solution#27 introduces the SMF triggered individual delivery method to shared delivery method switch after Inter-RAN mobility. When UE move from one gNB not supporting MBS to a gNB supporting MBS, the SMF trigger the mode switch, i.e. change to the shared delivery method.

- Solution #31 proposes that switching between 5GC Shared and Individual MBS traffic delivery methods is done via (MB-)SMF initiated session modification procedure, triggered by the (MB-)SMF’s detection on changes of network status or performance, by the PCF based on its policy decision or upon AF requests, by the UDM based on change of delivery method for the subscribed service associated with the MBS session, or by the AMF upon detecting that the UE moves in or out of MBS service area.

- Triggered by the MSF/MBSF

- Solution #23 proposes that the MSF can trigger the switching between 5GC Shared and Individual MBS traffic delivery methods, based on the count of UE subscribed and utilizing the same content

- Solution #24 introduces the MBSF can do the delivery method switch based on different optimization criteria (e.g. UE population in certain area, etc.).

From the 5GC view the mode switch means the switch between the shared tunnel and unicast tunnel.

If the shared tunnel can be used, it is unclear why the delivery method need be changed to the unicast tunnel?

When the UE move from a source gNB not supporting MBS to a target gNB supporting MBS, after the mobility the MBS individual delivery is used. It is benefit in this case to switch to shared delivery method, which can be triggered by SMF.

- Triggered by the AF

Solution #24 proposes that the content/application server can send requests to the (MB-)SMF via the MBSF/NEF to trigger the switching between 5GC Shared and Individual MBS traffic delivery methods. Similarly, Solution #31 proposes that the PCF can, based on AF request, send requests to the (MB-)SMF to trigger such switching.

B) Switching between PTP and PTM delivery methods

All the three solutions, i.e. Sol #18, #22, #24, #25, and #30 propose that the NG-RAN decides on using PTP or PTM delivery method over radio interface for MBS data, and may dynamically switch between these two delivery methods.

- Solution#18 proposes the NG-RAN do the delivery method switch between PTP and PTM based on the MBS assistance information from 5GC.

- Solution#22 proposes the NG-RAN do the delivery method switch between PTP and PTM totally based on RAN internal decision and no need 5GC involved. In this case there are no change between the shared delivery and the individual delivery.

- Solution#24 propose the delivery method switch from RAN is performed with 5GC involved. In this case it also includes the change between the shared delivery and the individual delivery.

- Solution#25 propose when the 5GC decides the shared delivery method is used, the RAN can do the PTP or PTM independently from 5GC which has the same view with solution#22.

- Solution#30 introduces RAN initiated 5GC delivery method switch. When the RAN node detect multi same MBS session individual delivery, it trigger mode switch to shared delivery.

NOTE:  solution#28 mention the mode switch triggered by RAN. However it indeed is the UE trigger mode switch.

For solution#18, one LS (S2-2006044) has been sent to RAN. It is suggested to wait the RAN feedback on whether this information is useful or not.

For solution#30, it is unclear that why the 5GC does not use the shared delivery from the beginning, i.e. the 5GC shall try to use the shard tunnel if possible. If that, it is unclear whether this scenario exist.

The difference between the solution#22/25 and solution#24 is on whether the RAN trigger mode switch also includes the switch between the shared delivery and the individual delivery. As the NG-RAN node is the MBS capable, there are no reason why if the shared tunnel can be used, it still need switch to the unicast tunnel?

3) Inter-RAN handover / UE mobility related delivery method switching

A comparison between solutions for inter-RAN handover / UE mobility related delivery method switching is shown in Table 7.X-2.

Table 7.X-2: Comparison of solutions for Inter-RAN handover / UE mobility related delivery method switching

|  | Sol#11/#12 | Sol#26 | Sol#27 | Sol#29 | Sol#40 (dual RANs case) |
| --- | --- | --- | --- | --- | --- |
| Procedures for delivery method switching | Enhanced Xn HO: MB Session resource setup during Xn Handover preparation or completionphase Enhanced N2 HO: MB Session resource setup during N2 Handover preparation phase May involve switching between 5GC Shared and Individual MBS traffic delivery methods based on 5MBS capability of target gNB. | Enhanced Xn/N2 HO: - Forwarding multicast data via mapped QoS flow over PDU session from source to target RAN. - Associated PDU session is used for lossless handover. | Enhanced Xn/N2 HO: - Shared delivery or Individual delivery established in target RAN, depending on whether target RAN supports MBS. - Support forwarding MBS data to Target RAN - Associated PDU session is used for lossless handover or unicast fallback when source RAN or target RAN does not support MBS. May involve switching between 5GC Shared and Individual MBS traffic delivery methods based on 5MBS capability of target gNB. | Enhanced Xn/N2 HO with MB Session resource setup during Xn/N2 Handover completion phase via UE initiated NAS multicast session join. May involve switching between 5GC Shared and Individual MBS traffic delivery methods based on 5MBS capability of target gNB. | Enhanced Xn/N2 HO: - Multicast or unicast transport established in target RAN, based on UE request of unicast or multicast mode in application layer. - |
| Baseline architecture used | 2 | 2 | 1 | 1 | 1 |

NOTE:	The evaluation may depend on the feedback from RAN WGs and the conclusion of relevant solutions to other KIs (esp. KI#1).

***** End of Changes *****