3GPP TSG-WG SA2 Meeting #141E e-meeting	S2-2007114

October 12 – 23, 2020	(merge S2-2007246 and S2-2007114)

Source:	Ericsson

Title:	KI#4- Conclusion

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposed some high level conclusion for KI#4.

# Discussion

Proposal to add some high level conclusions on KI#4 with the aim to identify those aspects that require normative work in Rel-17.

# Proposal

It is proposed to include the conclusion in the eNA_ph2 TR

******************************** START OF CHANGES **************************************

## 8.4 	Key Issue #4: Remaining aspects on how to ensure that slice SLA is guaranteed

For KI#4, it is proposed that the following aspects in Solution#2 are considered as a baseline for the normative work for slice load distribution based on determination of slice service experience and slice load analytics by the NWDAF:

- In the requirements on how to leverage the Analytics Id on Observed Service Experience and Slice Load for the purpose to check if the Slice SLA is guaranteed, the SLA includes the following "Percentage of UEs that access Application(s) that report Service Experience between 4-5 should be at least X". The conclusion is that that existing Analytics on Observed Service Experience for a Network Slice are reused as defined in Rel-16 TS 23.288 [5] and that Analytics Id on Slice load needs enhancements to define the resource usage within the slice, and the number of UE registrations and PDU session established in the slice.

- The input data and output analytics for the Analytics Id on Observed Service Experience remain unchanged. The input data and output analytics for Slice Load are defined in Solution#2: input data retrieved from OAM and NRF contain the resource usage; the number of UE registrations are retrieved from AMF; the number of PDU session from SMF; and the traffic usage from UPF.

- The NWDAF service consumers are NSSF, AMF; leveraging the analytics information, NSSF, AMF and SMF check if the Slice SLA is guaranteed, and if not they take actions as instructed in the Network Slice or Network Slice instance Restrictions  not to accept any more registration or PDU sessions until this restriction is lifted.

- The NWDAF subscribes to OAM to be informed on the creation or modification of a slice or slice instance.

******************************** END OF CHANGES **************************************