SA WG2 Meeting #141-E	S2-2007277

Elbonia, October 12 - 23, 2020

Source:	Samsung, NTT DOCOMO

Title:	KI#4: Evaluation update

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution updates the KI#4 evaluation

# Proposal

It is proposed to include the following text in TR 23.700-91.

******************************** START OF CHANGES **************************************

### 7.4	Key Issue #4: Remaining aspects on how to ensure that slice SLA is guaranteed

The following is a description of solution #2 and the aspects that are covered, there are no evaluation of the different solutions, since only one solution is defined. However, this clause evaluates some aspects of solution#2.

-	How does NWDAF determine slice service experience and slice load information and which entity or entities subscribe(s) to/request(s) the analytics?

-	Using the above analytics, how is slice load distribution carried out to assist with SLA?

For the first aspect: NWDAF determines the slice service experience and slice load information as follows:

-	NWDAF performs input data collection for slice service experience analytics as described in clause 6.4 of TS 23.288 [5].

-	NWDAF performs input data collection for slice load analytics as described in clauses 6.2.1.3 and 6.2.1.4.3.

-	Based on the overall input data collection information, NWDAF determines the slice service experience and slice load statistics and predictions.

-	NWDAF provides the output to the service consumer e.g. NSSF or AMF,

For the second aspects: NWDAF  provides statistics and predictions on Slice load or Observe Service experience or both and then the consumer NF determines if the SLA is fulfilled and take actions if needed to help maintaining the SLA as follows:

- Based on the NWDAF statistics and predictions mentioned above the service consumer (i.e. NSSF) monitor the slice SLA and then may trigger and indication to the AMF  a slice restriction for UE registrations and/or PDU sessions. AMF enforces the slice restriction by accepting or rejecting new UE registrations and/or PDU session establishments on the restricted slice following the procedures in TS 23.502 [3].

******************************** END OF CHANGES **************************************