SA WG2 Meeting #141-e	S2-2007554r02

October 12th -23rd 2020, Online	(revision of )

Source:	Nokia, Nokia Shanghai Bell,

Title:	KI#7: new solution based on 17

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2

Abstract of the contribution: xxx

# Discussion

This is an update of solution 17 to remove some issues it may have and removes an editors note. Specifically, it provides visibility to the UE as to which S-NSSAI work in the band(s) the RAN provides in dedicated priorities for cell reselection. Also, it provides the ability to indicate whether certain bands require the UE to include or not the GUAMI and not include the S-TMSI so the UE can be directed to a dedicated AMF right away, as certain dedicated bands network slices also have an associated dedicated AMF.

# Proposal

The following clarifications are proposed to be added to TR 23.700-40

# Proposed text changes – all new text

## 6.x	Solution #x: Support RAN-Based UE redirection with enhanced RRC-level information

### 6.x.1	Introduction

This solution addresses KI#7.

The principles of the solution is to follow the current procedures as much as possible and if the network cannot accept the Requested NSSAI due to it is not allowed within current TAI, the 5GC provides a the Configured NSSAI, the Rejected S-NSSAIs in addition to the Allowed NSSAI to NG-RAN for NG-RAN to select a suitable Radio Spectrum for the UE. The AMF, also when the Requested NSSAI is fully allowed, provides to the RAN also the Configured NSSAI as part of the UE context so the NG-RAN, can indicate to the UE, if there are any band-specific slices that are supported in the frequencies provided to the UE, and which of the Configured NSSAI S-NSSAI(s) work in each of the bands in the CellReselectionPriorities list indicated to the UE. The network also should attempt to steer the UE only to bands where there are slices that the UE can potentially use, with priority to the bands in the Allowed NSSAI taking also into account the rejected S-NSSAIs if received from the AMF. The AMF decides whether to send the rejected S-NSSAIs to the UE based on policy, e.g. when it has to allocated a default S-NSSAI because the requested NSSAI was not supported in the TA where the UE is.

When the RAN receives a list of rejected S-NSSAIs for the UE, it should attempt to provide to the UE a CellReselectionPriorities in RRC Connection Release with bands that should include these S-NSSAIs, if such bands are available in neighbour cells of the cell where the UE currently is. These should be prioritised so the UE can attempt to use these. The NG-RAN also indicates for each frequency in the CellReselectionPriorities list which S-NSSAI(s) of the Configured NSSAI are supported in this frequency if the frequency supports band specific slices.

The NG-RAN should also always attempt to provide the UE with camping policies that allow the UE to camp on cells that can potentially serve the greatest possible number of S-NSSAIs of the Configured NSSAI S-NSSAIs, if possible, and also avoid that the UE is provided priorities in bands that support none of the S-NSSAIs the UE can potentially register with (i.e. the RAN shall not provide the UE with a frequency priority placing it where none of the slices in the configured NSSAI can work).

If the AMF cannot support certain S-NSSAI of the Configured NSSAI, it also indicates this information to the NG-RAN (as part of the UE context or in any N2 message causing the NG-RAN to release the UE RRC connection if no UE context is yet present in the RAN) so the NG-RAN informs the UE when releasing the RRC connection whether some S-NSSAI in the Configured NSSAI it sends to the UE in the CellReselectionPriorities in the RRC connection Release message require the UE to not send the GUAMI and S-TMSI so the UE is assigned to a more optimal AMF right away, or, if the NSSAI cannot be included, to a default AMF.

### 6.x.2	High-level Description

The following sequence of events are envisioned:

1.	 Band-specific Network Slices are created supporting specific radio spectrum, and other Network Slices, e.g. eMBB, are created supporting all the the radio spectrum available for the operator;

2.	The UE has a subscription for both the band-specific Network Slices and an eMBB Network Slice;

3.	The subscription for the UE includes at least a band-specific Network Slice and eMBB S-NSSAI and the eMBB S-NSSAI is marked as default;

4.	The UE has been registered in the network and been configured with a Configured NSSAI and provided an Allowed NSSAI including the eMBB but not the any band-specific Network Slice;

5.	The UE/user decides to use the band-specific Network Slice and therefore issues a Requested NSSAI at least a band-specific Network Slice..

6.	If at least one S-NSSAI in the Requested NSSAI is not allowed in the current TAI, and The AMF may provide to the RAN the rejected S-NSSAIs in addition to the Allowed NSSAI and the Configured NSSAI to the NG-RAN

to allow NG-RAN to steer the UE according to the received information.

The NG-RAN, provides the UE with the list of S-NSSAIs among the S-NSSAIs of the configured NSSAI which can be used in each band of the CellReselectionPriorities list which supports band-specific Network Slices (i.e. a band that supports all the S-NSSAIs of the configured NSSAI needs not be associated with a list of S-NSSAIs).

### 6.x.3	Procedures

Existing procedures are used with the changes shown (in bold) in the following Figure 1 showing the information flow for when a UE registers from a TA that is defined to not be supported by any of the S-NSSAIs in the Requested NSSAI.

Figure 6.x.3-1: UE requests to be registered to band-specific S-NSSAI while not using the radio spectrum defined for the band-specific Network Slice

The steps of Figure 6.x.3-1 are as follows:

1.	The UE decides to use Band-specific S-NSSAI and sends a Registration Request (Requested NSSAI=Band-specific S-NSSAI). NG-RAN selects AMF as per current procedures i.e. this can be done from CM-IDLE or CM-CONNECTED; if the UE was already CM-CONNECTED, the RAN already is assumed to have in the UE context the Allowed NSSAI, the Configured NSSAI and also the Direct AMF selection information.

2.	The AMF and, if necessary, NSSF performs Network Slice selection. As the Band-specific S-NSSAI is not defined for the current TAI, the AMF and, if applicable, the NSSF selects the eMBB S-NSSAI as Allowed NSSAI due to it is defined as the default Subscribed S-NSSAI.;

3.	The AMF sends an N2 message to the NG-RAN with , if the UE was CM-IDLE, Allowed NSSAI, Configured NSSAI, rejected S-NSSAI and any Direct AMF selection information, and the NAS message that includes the eMBB S-NSSAI as Allowed NSSAI and the Band-specific S-NSSAI as a rejected S-NSSAI for the RA; if the UE was CM-CONNECTED, the AMF just updates the NG-RAN UE context with rejected S-NSSAIs.

4.

The NG-RAN forwards the NAS message to the UE;

5.	The NG-RAN determines the Radio Spectrum based on the information received from the AMF;

6.	The NG-RAN issues an RRC Release and includes cell reselection priorities and the associated information of any band that only supports some Network Slices in the Configured NSSAI (whether in backlist of whitelist form is up to stage 3), and also any indication that certain S-NSSAIs are not supported by the current AMF so the UE can register without including the GUAMI and S-TMSI and include the NSSAI if required. ; Alternatively, if the UE has ongoing PDU sessions which can work in the target band (because the target band supports the eMBB network slice, in this case) that work in the current TA, the RAN can handover the UE to a suitable band directly outside the RA based on policy, and if so the step 7 is skipped. If the UE was handed over to a new TA as it was in CM-CONNECTED state, the NG-RAN removes the rejected S-NSSAIs form the UE context or, if it is handing over the UE to another NG-RAN node, it does not pass to the target NG-RAN the rejected S-NSSAIs. the NG-RAN adds in the RRC connection reconfiguration the list of S-NSSAIs available in the new TA.

NOTE:	Steps 4 and 6 can possibly be done at the same time, i.e. it is up to NG-RAN. When moving the UE the NG-RAN takes into account UE radio capabilities, as per current specifications.

7.	The UE performs cell reselection if it was caused to become idle by the NG-RAN.

8.	The UErealizes that the TAI of the selected cell is outside of the RA and issues a Registration Request and includes the Band-specific S-NSSAI in the Requested NSSAI as the UE still wants to register to that Network Slice and the Band-specific S-NSSAI was previously only rejected for the RA (i.e. as per existing logic);

9.	Network Slice selection may be performed as per current procedures if required.

10.	The AMF sends a Registration Accept as per current procedures and includes the Band-specific S-NSSAI in the Allowed NSSAI.

If the eMBB slice is not supported for the radio spectrum dedicated to the band-specific Network Slice the PDU Sessions for eMBB are released as per existing procedures e.g. using PDU Session status.

NOTE:	If the eMBB slice is not available on the radio spectrum dedicated to the band-specific Network Slice, as these Network Slices are to be isolated, when an application in the UE requires connectivity for eMBB, the whole procedure is repeated with the UE adding the eMBB slice in a Requested NSSAI.

NOTE:	in the event the UE requests multiple S-NSSAI (sets) that require separate frequency bands not possible to support at the same time, the solution may result in more registration attempts.

### 6.x.4	Impacts on services, entities and interfaces

The impacts to the 5GS entities are the following:

UE:

-	No impacts foreseen if the UE ignores the additional info in RRC connection release making it aware of whether certain bands only support certain s-NSSAIs, and also the "direct AMF selection information".

NG-RAN:

-	Support taking into account the Configured NSSAI and rejected S-NSSAIs when providing CellReselectionPriority list to the UE, storing Configured NSSAI and any direct AMF selection information, provision of additional information in RRC connection release or RRC connection reconfiguration as indicated above.

AMF:

-

providing configured NSSAI, rejected NSSAI and any Direct AMF selection info to the NG-RAN as indicated above.