3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-210xxxx

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | XXXX | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE On-boarding registration for NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the TR 23700-07, On-boarding registration and following remote provisioning for SNPN case should be supported in R17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support on-boarding registration and remote provisioning after successful on-boarding |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing new features as per conclusion of TR 23.700.07 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.X(NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.30.2.X	UE on-boarding for SNPN

SNPN or PLMN can act as an on-boarding network to support On-boarding registration for UEs that need provisioning of NPN credentials (i.e. for primary authentication) and other information to enable SNPN access.

In order to register to an SNPN for On-boarding, the UE sets the registration type as On-boarding Registration and provides an RRC indication to indicate that the RRC connection is for on-boarding. This RRC indication allows NG-RAN to select an appropriate AMF that supports on-boarding procedures. The On-boarding Registration type allows AMF to, e.g., select an appropriate SMF and perform other on-boarding-related configuration, e.g., start the timer configured for on-boarding service, restrict the UP connectively only for purpose of remote provisioning with allowed PS(s).

On-boarding network should support functionality to restrict usage to only on-boarding service.

-	When On-boarding network is O-SNPN, the information required to restrict the usage to only on-boarding service is locally configured in the AMF and SMF, and the AMF and SMF restrict the usage when the UE indicates that the registration type is On-boarding Registration or NG-RAN indicates that the RRC connection is for On-boarding.

-	When On-boarding network is a PLMN, the AMF restrict its functionality only for on-boarding usage based on received operator subscription from the UDM. Based on operator’s configuration, the subscription data shall contain a DNN, S-NSSAI or information (e.g., explicit indication) dedicated for on-boarding.

The UE shall initiate de-registration from the on-boarding network after finishing the remote provisioning or the on-boarding network shall initiate the de-registration after successful completion of on-boarding or based on timer configured for on-boarding service.

* * * * End of changes * * * *