SA WG2 Meeting #143E e-meeting	S2-2100387

Elbonia, February 24 – March 09, 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0202 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: Additional consumer NFs for service experience analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  |  | Date: |  | 2021-02-18 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FS_eNA_Ph2 concluded for KI#4 that service consumers for NWDAF service experience analytics are NSSF and AMF: “For KI#4, it is proposed that the following aspects from Solution #2 are considered for the normative work for slice load distribution mechanism based on determination by NWDAF of slice service experience and slice load analytics: “(…) The NWDAF service consumers are NSSF, AMF; (…)” |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR updates service consumers for observed service experience analytics, as concluded in FS_eNA_Ph2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Study conclusions would not be respected; defective/incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 23.501 CR2567 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS 23.502 CR2586 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

### 6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average of observed Service MoS and/or variance of observed Service MoS indicating service MOS distribution for services such as audio-visual streaming as well as services that are not audio-visual streaming such as V2X and Web Browsing services) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or both of the following:

-	Service Experience for a Network Slice: Service Experience for a UE or a group of UEs or any UE in a Network Slice;

-	Service Experience for an Application: Service Experience for a UE or a group of UEs or any UE in an Application or a set of Applications.

Therefore, Observed Service experience may be provided individually per UE or group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice.

The service consumer may be an NF (e.g. PCF, NSSF, AMF), or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in Table 6.4.1-1; and

-	optionally, maximum number of objects and maximum number of SUPIs;

Table 6.4.1-1: Analytics Filter Information related to the observed service experience

| Information | Description | Mandatory |  |
| --- | --- | --- | --- |
|  |  | Application | Network Slice |
| Application ID (0...max) | The identification of the application(s) for which the analytics information is subscribed or requested. | Y | N |
| S-NSSAI | Identifies the Network Slice for which analytics information is subscribed or requested. | N | Y |
| NSI ID(s) | Identifies the Network Slice instance(s) for which analytics information is subscribed or requested. | N | N |
| Area of Interest | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] for which the analytics information is subscribed or requested. | N | N |
| Media/application bandwidth | Identifies the Media/application bandwidth requirement of the application. | N | N |
| DNN | DNN to access the application. | N | N |
| DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. | N | N |

NOTE:	A service consumer may use the Area of Interest in order to reduce the amount of signalling that the analytics subscription or request generates.

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provide statistics and predictions on the observed service experience to a consumer NF or to OAM.

Based on the Analytics Filter information in Table 6.4.1-1 and the Target of Analytics reporting provided by the service consumer in the analytics subscription or request, NWDAF determines whether service experience analytics should be delivered for:

i)	Application(s);

ii)	Network Slice;

iii)	both Application(s) and Network Slice.

If NWDAF is unable to differentiate based on the analytics subscription or request, it provides service experience analytics for both Application(s) and Network Slice.

If service experience for both Application(s) and Network Slice is desired but the Target of Analytics reporting or Analytics Filter information values (e.g. Area of Interest) need to be different, separate subscriptions/requests may be provided by the service consumer.

* * * * End of Changes * * * *