SA WG2 Meeting #143E e-meeting	S2-2101337

Elbonia, February 24 – March 09, 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0201 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: Slice load level related network data analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, KDDI, CATT, NTT DOCOMO, Orange, Spirent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  |  | Date: |  | 2021-02-18 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces the definition of slice load analytics, which had been missing from Rel-16 specifications despite the existence of a related clause. The input data, output analytics and procedures are all taken from the conclusion of the study of KI#4 pointing to Sol#2, captured in clauses 8.4 and 6.2 of TR 23.700-91, respectively. In addition, a parameter to indicate the the subset of analytics that are requested is added, according to the following conclusion of TR 23.700-91, clause 8.11.3: “Additional Analytics filters shall be standardized on a per-analytics basis, in order to narrow down the scope of analytics, thus reducing signalling both for analytics provision and data collection.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change includes input data description, output analytics by NWDAF, and procedure required for delivery of analytics |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Study conclusion would not be respected; slice load analytics would continue un-specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.3.2A, 6.3.3A, 6.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

## 6.3	Slice load level related network data analytics

### 6.3.1	General

The NWDAF provides slice load level information to a consumer NF on a Network Slice level or a Network Slice instance level or both. The NWDAF is not required to be aware of the current subscribers using the slice. The NWDAF notifies slice specific network status analytics information to the consumer NF that is subscribed to it. A consumer NF may collect directly slice specific network status analytics information from NWDAF. This information is not subscriber specific.

The NWDAF services as defined in the clause 7.2 and clause 7.3 are used to expose slice load level analytics from the NWDAF to the consumer NF (e.g. PCF, NSSF or AMF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Load level information";

-	The following Analytics Filter Information:

-	S-NSSAI and NSI ID;

NOTE:	The use of NSI ID in the network is optional and depends on the deployment choices of the operator. If used, the NSI ID is associated with S-NSSAI.

-	an optional Area of Interest;

-	an optional list of NF types;

-	optionally, the list of analytics subsets that are requested among those specified in clause 6.3.3A.

-	optionally, Load Level Threshold value; and

- 	optionally, "maximum number of objects" indicating the maximum number of Network Slice instances expected in output, when the Analytics Filter Information does not indicate an NSI ID.

### 6.3.2	Void

### 6.3.2A	Input data

The detailed information collected by the NWDAF is listed in Table 6.3.2A-1.

Table 6.3.2A-1: Input data for slice load analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamps | 5GC NF | A time stamp associated with the collected information. |
| UE registered in a Network Slice/Network Slice instance | AMF, OAM | Current number of UEs registered in a NW slice or NW slice instance |
| PDU session established on a Network Slice/Network Slice instance | SMF, OAM | Current number of established PDU sessions in a NW slice or NW slice instance. |
| Load of NFs associated to Network Slice instance | OAM, NRF | Resource utilization information of a Network Slice instance obtained from its constituent NF instances. NF instance load input data collection is described in clause 6.5, Table 6.5.2-1. |

### 6.3.3	Void

### 6.3.3A	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the following analytics:

-	Network Slice load statistics information as defined in Table 6.3.3A-1.

-	Network Slice instance load statistics information as defined in Table 6.3.3A-2.

-	Network Slice load predictions information as defined in Table 6.3.3A-3.

-	Network Slice instance load predictions information as defined in Table 6.3.3A-4.

Table 6.3.3A-1: Network Slice instance load statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice |
| Network Slice instances (1,…,max) | List of Network Slice instance(s) within the S-NSSAI |
| > NSI ID | Identification of the Network Slice instance |
| > Number of UE Registrations (NOTE 1) | Number of UE registrations of the Network Slice instance (average, variance) |
| > Number of PDU Sessions establishment (NOTE 1) | Number of PDU Session establishments of the Network Slice instance (average, variance) |
| > Resource usage (NOTE 1) | The usage of assigned virtual resources currently in use for the NF instances (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [19] clause 5.7, belonging to a particular Network Slice instance. |
| > Resource usage threshold crossings (optional) (NOTE 1) | Number of resource usage threshold crossings on the Network Slice instance provided if threshold is provided by the consumer as Analytics Filter. |
| > Resource usage threshold crossings time period (1,…,max) (optional)(NOTE 1) | Resource usage threshold crossing vector including time elapsed between each threshold crossing on the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-2: Network Slice load statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice |
| > Number of UE Registrations (NOTE 2) | Number of UE registrations at the Network Slice (average, variance) |
| > Number of PDU sessions establishments (NOTE 2) | Number of PDU Session establishments at the Network Slice (average, variance) |
| NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-3: Network Slice instance load predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice |
| Network Slice instances (1,…,max) | List of Network Slice instance(s) within the S-NSSAI |
| > NSI ID | Identification of the Network Slice instance |
| > Number of UE Registrations (NOTE 3) | Number of predicted UE registrations at the Network Slice instance |
| > Number of PDU Sessions establishment (NOTE 3) | Number of predicted PDU Session establishments of the Network Slice instance |
| > Resource usage (NOTE 3) | The predicted usage of assigned virtual resources for the NF instances (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [19] clause 5.7, belonging to a particular Network Slice instance. |
| > Resource usage threshold crossings (optional) (NOTE 3) | Number of predicted resource usage threshold crossings at the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Resource usage threshold crossings time period (optional) (1,…,max) (NOTE 3) | Predicted Resource usage threshold crossing vector including predicted time elapsed between each threshold crossing on the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Probability assertion | Confidence of this prediction. |
| NOTE 3: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-4: Network Slice load predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice |
| > Number of UE Registrations (NOTE 4) | Predicted Number of UE registrations at the Network Slice |
| > Number of PDU sessions establishments (NOTE 4) | Predicted Number of PDU Session establishments at the Network Slice |
| > Probability assertion | Confidence of this prediction. |
| NOTE 4: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

NOTE 5:	If no NSI ID is provided as analytics filter, slice load level related output analytics are provided according to Tables 6.3.3A-2 and 6.3.3A-4. Otherwise slice load level related output analytics are provided according to Tables 6.3.3A-1 and 6.3.3A-3.

### 6.3.X	Procedures

Figure 6.3.X-1: Network Slice load analytics provided by NWDAF

Figure 6.3.X-1 shows the procedure for NWDAF to derive slice load analytics. The steps are described as follows:

1.	A consumer NF subscribes to/requests a NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation (Analytics ID = Load level information and a set of Analytics Filters (e.g. S-NSSAI, NSI ID, Area of Interest)).

2.	[OPTIONAL] If the NWDAF does not have already the slice information, it gains the slice information from OAM (as described in clause 6.2.3.x) and discovers from NRF the AMF, SMF and NSSF instance(s) relevant to the Analytics Filters provided in the analytics subscription.

3.	[OPTIONAL] If the NSI ID(s) are not provided in the analytics subscription by the consumer NF, the NWDAF invokes Nnssf_NSSelection_Get service operation from NSSF to obtain the NSI ID(s) corresponding to the S-NSSAI in the subscription.

4a.	[OPTIONAL] The NWDAF may subscribe to input data in Table 6.3.2A-1 from the OAM according to the data collection principles from the OAM described in clause 6.2.3.

4b.	[OPTIONAL] The NWDAF may collect input data from the NRF (see clause 6.5) to derive slice instance resource usage statistics and predictions for a Network Slice instance.

5.	The NWDAF subscribes to the AMF's event exposure service to collect data on the number of UEs currently registered on certain Network Slice and, if available, its constituent Network Slice instance(s). An UE access and mobility information event using event ID "UE moving in or out of Area of Interest" is used for that purpose as defined in TS 23.502 [3] using as an example Event Filters S-NSSAI and, if available, NSI ID(s), and optionally Area of Interest. If required, the NWDAF may also collect the corresponding UE IDs.

6.	The NWDAF subscribes to the SMF's event exposure service to collect data on the number of PDU sessions currently registered on certain Network Slice and, if available, its constituent Network Slice instance(s). A PDU Session establishment/release related event is used for that purpose as defined in TS 23.502 [3].

7.	The NWDAF derives slice load analytics.

8.	The NWDAF delivers analytics to the consumer NF by invoking Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response service operations

* * * * End of Changes * * * *