3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102576

Elbonia, 12 - 16 April, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	Mobility Procedures for 5MBS

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This document adds Mobility Procedure for 5MBS to the new TS. This include the mobility among the homogenous and non-homogenous 5MBS capability NG-RAN node.

# Background and Introduction

This contribution proposes the related procedures as per the conclusion in clause 8.7 of TR 23.757.

# Proposal

It is proposed to capture the following changes vs. TS23.247.

# * * * * First change* * * *

## 7.1.5	Mobility Procedures for 5MBS

## 7.1.5.1 General

UE may move from one NG-RAN node to another NG-RAN node after UE has joined the MB Session. Depending on whether the NG-RAN node support 5G MBS, the following scenarios are possible:

- Both source and target NG-RAN support 5G MBS.

- Source NG-RAN supports 5G MBS, target NG-RAN does not support 5G MBS.

- Source NG-RAN does not support 5G MBS, target NG-RAN supports 5G MBS.

- Both source and target NG-RAN don’t support 5G MBS.

When the UE moves from a NG-RAN node that supports 5G MBS to a NG-RAN node that does not support 5G MBS, the delivery method is switched from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method.

When the UE moves from a NG-RAN node that does not support 5G MBS to a NG-RAN node that supports 5G MBS, the delivery method is switched from the 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

## 7.1.5.2 Xn based handover with MBS Session

Editor’s Note: The content will be provided based on RAN WGs input.

## 7.1.5.3 N2 based handover with MBS Session

Editor’s Note: The content will be provided based on RAN WGs input.

# * * * * End of changes * * * *