# SA WG2 Meeting #148-e (e-meeting)	S2-210xxxx

# 15-22 November 2021, Elbonia

Title:		Draft LS on Alternative IMSI calculation in EPS

Reply to:	R2-2111329 (S2-2108997) Reply LS on RAN2 agreements for MUSIM

Release:	Release 17

Work Item:	MUSIM

Source:	SA2

To:		RAN2, CT1

Cc:		RAN3, SA3

Contact person:	Lars Nord

Lars dot Nord at sony dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	attach agreed CRs

# 1	Overall description

SA2 thanks RAN2 for the Reply LS on RAN2 agreements for MUSIM in R2-2111329/S2-2108997. In that LS SA2 noted the following:

RAN2 found inconsistent between CT1 and SA2 specification on alternative IMSI calculation in EPS. Based on the CT1 spec, NAS will forward the accepted IMSI offset to lower layers. This is aligned with RAN2 agreement on PF/PO calculation for EPS since the AS layer needs the IMSI offset to calculate the UE_ID. However, SA2 specifies that the IMSI offset is used by the NAS layer to calculate an alternative IMSI instead of the UE_ID, and it seems that the NAS layer needs to forward the alternative IMSI to the AS for the determination of paging occasions as specified in TS 36.304. So based on the SA2’s specification, SA2 is not aligned with both RAN2 and CT1’s agreements. RAN2 prefers that the alternative IMSI is to be calculated in AS layer (i.e. aligned with CT1 specification).

SA2 has discussed the inconsistency mentioned in the reply LS from RAN2 and sees no mismatch between SA2 and stage 3.

According to TS 23.401 clause 4.3.33.5, the UE and the MME use the Accepted IMSI Offset to determine an alternative IMSI value and use the alternative IMSI value instead of the IMSI to calculate to PO/PF timing for Paging Timing Collision Control.

For the network, the MME uses the Alternative IMSI value to determine the UE Identity Index Value (UE_ID) and provides the UE Identity Index Value (UE_ID) in the S1 PAGING message (as specified in TS 36.413) to the eNB. This means that the eNB does not require any additional parameters or information, and can use the provided UE Identity Index Value (UE_ID) to derive the PO/PF for Paging Timing Collision Control.

For the UE, the SA2 specifications describe the calculation for the Alternative IMSI value and refers to TS 36.304 for the rest of the calculation needed to determine the PO/PF. Importantly, SA2 does not specify which layer in the UE (NAS or AS) performs the calculation.

As the SA2 specification was not clear enough and created discussion in RAN2, SA2 has agreed to update the text to make it more explicit.

SA2 kindly ask RAN2 to take the SA2 response into consideration and to provide feedback if needed.

# 2	Actions

To RAN3

ACTION: 	SA2 kindly ask RAN2 to take the SA2 response into consideration and to provide feedback if needed.

# 3	Dates of next TSG SA WG2 meetings

3GPPSA2#149-e	14 - 25 February 2022	Electronic Meeting

3GPPSA2#150	4 - 8 April 2022	TBD