3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-220xxxxx

Elbonia, February 14th – 25th, 2022	(revision of S2-2200575, S2-2200836, S2-2200975)

Source:	vivo, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon

Title:	23.700-88: Architectural assumptions and principles

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: this paper proposes Architectural assumptions and principles for PIN.

1. Introduction

1.1 PIN element definitions

The PIN SA1 requirements refer PIN element, as reported below:

  - PIN Element: UE or non-3GPP device that can communicate within a PIN.

  - PIN direct connection: the connection between two PIN Elements without any 3GPP RAN or core network entity in the middle.

  - PIN Element with Gateway Capability (PEGC): a UE PIN Element that has the ability to provide connectivity to and from the 5G network for other PIN Elements.

  - PIN Element with Management Capability : A PIN Element with capability to manage the PIN.

However from SA2 point of view, we define functionalities which can be supported on different device, For instance a UE can be a PIN element, a PIN Element with Gateway Capability or Management capabilities, or any Combination of them, so it is more flexible and generic to define the functionalities related to Personal IoT Network allowing a device either a UE or a Non-3GPP device to support only 1 or more than 1 of them allowing, in case of UE to play different roles if needed.

Proposal #1: to define of Personal IoT Network functions rather then different “elements” as done by SA1

    - PIN Element: the PIN Element is a UE or Non-3GPP device supporting PIN Element Functions which enables the realization of a Personal IoT Network

    - PIN Element Function (PEF): The function enabling the PIN network communication and supporting the PIN specific procedure, e.g. PIN discovery, etc..

    - PIN Element Gateway Capability function (PEGC): the function providing the connectivity to and from the 5G network for other device supporting the PIN Elements Function.

    - PIN Element Management Capability functiony (PEMC): The function providing the capability to manage the Personal IoT Network .

    - Note: The solutions description may provide a more detailed list of functionalities supported by each of the above PIN functions

    - A PIN Element can support both PIN management capability and Gateway Capability functions

    - Editor’s note: the definitions will be revised in order to reflect the agreed conclusion

1.2 PIN Architecture assumptions

The SID description includes the note in the following which are proposed to be captured as working assumption

  - NOTE 3: In release 18, this study only addresses the case where PIN Elements other than PEGC and PEMC use non-3GPP access (e.g. WIFI, Bluetooth). Additionally, the study assumes to reuse procedures defined for ProSe without introducing new features to sidelink, and no change to underlying WIFI, Bluetooth and other underlying non-3GPP access standards.

  - NOTE 4: In release 18, only UE can act as PEGC and PEMC.

Proposal #2: to capture Note 3 and note 4 of PIN SID as architecture assumptions with the rewording to refer to the above definitions.

According to the SP-211643, the study on Personal IoT Networks, there’re some NOTEs that can also be used as the architecture assumption for this study.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change * * * *

# 4	Architectural requirements, assumptions and principles

## 4.1	Architectural Requirements

This study has following architectural requirements:

-	Reuse procedures defined for ProSe without introducing new features to sidelink.

-	There shall be no change to underlying WIFI, Bluetooth and non-3GPP access standards.

## 4.2	Architectural Assumptions

This study has following architectural assumptions:

-	Only a 3GPP UE can act as PEGC and/or PEMC.

-	There may be multiple PIN Element UEs within a PIN that have GW capability, but at any point in time only one UE acts as the GW for a specific PIN.

-	In Release 18 only a UE acting as PEGC and/or PEMC are identified by 3GPP network. A PIN element without 3GPP access capability is not known to the 3GPP network.

-	A PIN element without GW or Management capability only uses non-3GPP access (e.g. WiFi, Bluetooth)

-	The communication between PIN elements using WiFi, Bluetooth or other non-3GPP technologies with the PEGC and PEMC are out of scope of this study.

-	The PIN Elements other than PEGC and PEMC assumes to use non-3GPP access (e.g. WIFI, Bluetooth) for direct communication with PEGC, and no change to underlying WIFI, Bluetooth and other underlying non-3GPP access standards.

-	If PC5/sidelink is used for the direct communication, the procedures defined for ProSe without introducing new features to sidelink is assumed.

The Personal IoT Networks is assumed to be composed by following functions:

-	PIN Element: the PIN Element is a UE or Non-3GPP device supporting PIN Element Functions, which enables the realization of a Personal IoT Network

-	PIN Element Function (PEF): The function enabling the PIN network communication and supporting the PIN specific procedure, e.g. PIN discovery, etc..

-	PIN Element Gateway Capability functionality (PEGC): the function providing the connectivity to and from the 5G network for other device supporting the PIN Elements Function.

-	PIN Element Management Capability functionality (PEMC): The function providing the capability to manage the Personal Network .

Note:	The solutions description may provide a more detailed list of functionalities supported by each of the above PIN functions

A PIN Element can support have both PIN management capability and Gateway Capability functionalities

Editor’s note: the definitions will be revised in normative phase in order to reflect the agreed conclusion

This study only addresses the case where PIN Element Function uses non-3GPP transport technologies (e.g. WIFI, Bluetooth, 5G Prose Direct Communication) for communications.

This study assumes to reuse procedures defined for 5G Prose Direct Communication (i.e. UE-to-UE communication) without introducing new features to sidelink, and no change to underlying WIFI, Bluetooth and other underlying non-3GPP standards.

This study assumes that only UE can support PEGC function and PEMC function.

# * * * * End of changes * * * *