SA WG2 Meeting #149e	S2-220xxx

# February 14th – 25th, 2022; Elbonia

Source: 	China Mobile

Title: 	Key issue related with WT3.4

Document for: 	Approval

Agenda Item:

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution:

1 Discussion

In FS_XRM SID, the WT#3.5 is focus on how to coordinate the UL-DL transmission to meet the RTT requirement.

WT#3.4: Whether and how to support uplink-downlink transmission coordination to meet RTT (Round-Trip Time) latency requirements between UE and N6 termination point at the UPF.

he key issue about WT#3.4 is proposed in this paper.

2 Proposal

It is proposed to update TR 23.700-60 on FS_XRM as follows

# * * * * First change * * * *(all new texts)

5.X	Key Issue #X: Support uplink-downlink transmission coordination to meet RTT latency requirement

5.X.1	Description

This key issue studies how to support uplink-downlink transmission coordination to meet RTT latency requirement.

Specific scenario requirement is as following:

In TS 22.261 clause 7.6, in order to support VR environments with low motion-to-photon capabilities, the 5G system shall support:

-	motion-to-photon latency in the range of 7 ms to 15ms while maintaining the required resolution of up to 8k giving user data rate of up to [1Gbit/s] and

-	motion-to-sound latency of [< 20 ms].

NOTE: 	The motion-to-photon latency is defined as the latency between the physical movement of a user's head and the updated picture in the VR headset. The motion-to-sound latency is the latency between the physical movement of a user's head and updated sound waves from a head mounted speaker reaching their ears.

So the 5GS should support the motion-to-photon RTT latency in the range of 7-15ms, and motion-to-sound latency less than 20ms.

The following aspects should be studied to support uplink-downlink transmission coordination to meet RTT (Round-Trip Time) latency:

- Whether and how to support uplink-downlink transmission coordination to meet RTT (Round-Trip Time) latency requirements between UE and N6 termination point at the UPF.

# * * * *end of change * * * *