SA WG2 Meeting #150E e-meeting	S2-2202391

Elbonia, 06 – 12 April, 2022	(revision of S2-22xxxxx)

Source:	vivo

Title:	FS_AIMLsys, KI#2, New solution for 5GC exposuring network data analytics to UE

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys/Rel-18

Abstract: This contribution proposes a solution to address how the 5GC exposes data analytics to the UE.

1.	Introduction

The UE may need information including data or data analytics from the 5GC to facilitate its Application AI/ML operation (e.g. Model Training, Splitting and inference feedback etc.), but how to obtain this information from 5GC is not clear.

This paper proposes solution for KI#2 in TR 23.700-80 for the SID FS_AIMLsys, to address how the 5GC exposes data analytics to the UE.

2.	Proposal

It is proposed to adopt the following solution into 23.700-80.

# * * * * First change* * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 |  | x |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |

# * * * * Next Change (ALL NEW TEXT) * * * *

6.X	Solution #X: <5GC exposures analytics to the UE via AF >

6.X.1	Description

This solution is about KI# 2 to address how the 5GC exposes data analytics to the UE.

Reusing the AF (e.g. DCAF), which is used to collect data from UE client for the NWDAF, the UE can also request or subscribe to the AF for data analytics from NWDAF in the opposite direction.

Upon the request or subscription to analytics from the UE, AF may ask the NEF for authorization information to check if the UE is allowed to obtain analytics ID from the network, the NEF may further check with the UDM serving the UE.

Then the AF subscribes to analytics information for the UE via NEF, which is similar as the procedure defined in clause 6.1.12, TS 23.288.

After receiving analytics from NWDAF via NEF, the AF will forward it to the UE.

6.X.2	Procedures

- UE selects AF for data collection (e.g. DCAF) based on configuration or DNS discovery.

- UE requests or subscribes to the AF for analytics with Analytics ID(s), Analytics filter and Target of Analytics Reporting, etc. the other detailed content of request can refer to the clause 6.1.3, TS 23.288.

- Before requesting analytics from network for the UE, the AF may ask the NEF for authorization information to check if the UE is allowed to obtain the analytics ID from the network.

The NEF determines the authorization information for the UE based on local policy and the UE subscription data about whether the UE has subscribed to the service that obtaining analytics from network.

  - The AF subscribes to analytics information for the UE via NEF by invoking the Nnef_AnalyticsExposure_Subscribe service operation, including UE ID, Analytics ID(s), Analytics filter and Target of Analytics Reporting, etc. The other detailed content of subscription can refer to the clause 6.1.3, TS 23.288.

The following steps between NEF and NWDAF, or between NEF and AF are similar as the ones defined in clause 6.1.12, TS 23.288.

7. After receiving analytics from NWDAF via NEF, the AF will forward it the UE.

6.X.3	Impacts on services, entities and interfaces

UE：

- Requests or subscribes to analytics from 5GC via AF

AD(DCAF):

- Check with NEF about authorization information for the UE if it is allowed to obtain analytics ID from the network;

- Requests or subscribes to analytics from 5GC for the UE

- Forwards the requested the analytics to UE.

NEF

- Check about authorization information for the UE based on local policy, the UE subscription data about whether the UE has subscribed to the service that obtaining analytics from network.

# * * * * End of change * * * *