| 3GPP TR 23.700-17 V0.2.0 (2022-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on the support for 5WWC, Phase 2 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms and abbreviations	7

3.1	Terms	7

3.2	Abbreviations	8

4	Architectural Assumptions and Principles	8

5	Key Issues	8

5.1	Key Issue #1: Providing differentiated service for UE and Non-3GPP devices connected behind a 5G RG	8

5.1.1	Description	8

5.2	Key Issue #2: How to select a TNGF/N3IWF that supports the S-NSSAI(s) needed by the UE	9

5.2.1	Description	9

5.X	Key Issue #X: <Key Issue Title>	9

5.X.1	Description	9

6	Solutions	9

6.0	Mapping of Solutions to Key Issues	9

6.1	Solution 1: N3GPP device behind 5G-RG	10

6.1.1	Description	10

6.1.1.1	Overall architecture	10

6.1.1.2	Support for differentiation of e.g. charging and QoS	11

6.1.2	Procedures	11

6.1.3	Impacts on Existing Nodes and Functionality	11

6.2	Solution 2: UE behind 5G-RG and FN-RG	11

6.2.1	Description	11

6.2.1.1	Overall architecture	11

6.2.1.2	Support for differentiated charging and QoS	13

6.2.2	Procedures	13

6.2.3	Impacts on Existing Nodes and Functionality	14

6.3	Solution 3: Differentiated QoS for N5CW devices behind 5G-RG	14

6.3.1	Description	14

6.3.2	Procedures	15

6.3.3	Impacts on Existing Nodes and Functionality	15

6.4	Solution 4: Solution of providing differentiated service for Non-3GPP devices connected behind a 5G-RG	15

6.4.1	Description	15

6.4.2	Procedures	15

6.4.3	Impacts on Existing Nodes and Functionality	17

6.5	Solution 5: 5GC-capable UE behind 5G-RG accessing 5GC	17

6.5.1	General	17

6.5.2	Registration procedure	18

6.5.3	Impacts on existing Functions	19

6.6	Solution 6: New method for non-3GPP device connected behind a 5G-RG	20

6.6.1	Description	20

6.6.2	Procedures	21

6.6.3	Impacts on Existing Nodes and Functionality	22

6.7	Solution 7: Differentiated QoS for non-3GPP devices behind 5G-RG	22

6.7.1	Description	22

6.7.1.1	Introduction	22

6.7.1.2	Solution principles	22

6.7.2	Procedures	23

6.7.3	Impacts on Existing Nodes and Functionality	23

6.10	Solution 10: Registration via Trusted Non-3GPP Access with TNGF Relocation	24

6.10.1	Description	24

6.10.2	Procedures	25

6.10.2.1	Registration via trusted non-3GPP access with TNGF relocation	25

6.10.3	Impacts on Existing Nodes and Functionality	26

6.11	Solution 11: Registration via Untrusted Non-3GPP Access with N3IWF Relocation	27

6.11.1	Description	27

6.11.2	Procedures	28

6.11.2.1	Registration via untrusted non-3GPP access with N3IWF relocation	28

6.11.3	Impacts on Existing Nodes and Functionality	30

6.12	Solution 12: slice related TNGF selection for WLAN access	30

6.12.1	Description	30

6.12.2	Procedures	30

6.12.3	Impacts on Existing Nodes and Functionality	31

6.X	Solution #X: <Solution Title>	31

6.X.1	Description	31

6.X.2	Procedures	31

6.X.3	Impacts on Existing Nodes and Functionality	31

7	Overall Evaluation	31

8	Conclusions	31

Annex A: Change history	32

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The Technical Report studies and performs evaluations of potential architecture enhancements to enhance the support of Wireline-Wireless convergence in the 5G Core network (5GC). Specifically, two objectives are included:

-	Work task #2: Whether and how to improve the support of devices connecting behind 5G-RG including:

Providing differentiated service (e.g. QoS and charging) for UE and Non-3GPP devices connected behind a 5G RG. This may imply s studying ways for 5GC to identify Non-3GPP devices connected behind a 5G RG and/or their traffic. Conclusions may differ for UE and for Non-3GPP devices.

-	Work task #3: Trusted/untrusted Non-3GPP access network: How to select a TNGF/N3IWF that supports the S-NSSAI(s) needed by the UE.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[6]	WiFi Alliance Technical Committee, Hotspot 2.0 Technical Task Group: "Hotspot 2.0 (Release 2) Technical Specification".

[7]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[8]	3GPP TS 24.502: "Access to the 3GPP 5G Core Network (5GCN) via non-3GPP access networks".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[10]           3GPP TS 24.526: " User Equipment (UE) policies for 5G System (5GS): Stage 3".

[11]           3GPP TS 23.402: " Architecture enhancements for non-3GPP accesses".

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], in TS 23.501 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] or in TS 23.501 [2].

Authenticable Non-3GPP (AUN3) devices: A Non-3GPP device that the 5GC can authenticate.

Non Authenticable Non-3GPP (NAUN3) devices: A Non-3GPP device that the 5GC cannot authenticate.

Non-3GPP devices: A device that use Non-3GPP access technology to connect to the RG and does not support NAS over non 3GPP access.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 23.501 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] or in TS 23.501 [2].

# 4	Architectural Assumptions and Principles

Editor's note:	This clause will document any architectural assumptions and principles for the study

For all objectives, the architectural assumptions and principles include:

1.	The architecture of 5WWC (for Wireline, Trusted and Untrusted access to 5GC) as defined in TS 23.501 [2] and TS 23.316 [5] shall be used as a baseline,

2.	The support of S-NSSAIs as defined in TEI17_N3SLICE shall be taken as baseline,

3.	Before a solution that impacts the 5G RG or the wireline access network is selected, this solution shall be evaluated by BBF and/or CableLabs.

# 5	Key Issues

## 5.1	Key Issue #1: Providing differentiated service for UE and Non-3GPP devices connected behind a 5G RG

### 5.1.1	Description

This key issue will study:

-	How to support differentiated services (e.g. QoS and charging) for the same and different Non-3GPP devices, and UEs connected behind a 5G-RG.

NOTE 1:	A device capable of access to a N3IWF or to a TNGF is a UE; Non-3GPP devices can correspond to either AUN3 devices or NAUN3 devices defined in clause 3.1.

NOTE 2: The definitions in the clause 3.1 are considered as indication of kinds of Non-3GPP devices to be considered, but each solution may propose alternative kinds of devices with relative definition, if needed.

This may imply studying ways for 5GC to identify Non-3GPP devices connected behind a 5G RG and/or their traffic.

Conclusions to this key issue may differ for UE and for Non-3GPP devices. Different solutions may address different kinds of Non-3GPP devices, but as conclusion of this KI the kinds of Non-3GPP devices with related definitions considered applicable for the normative work will be selected.

This Key Issue will focus on 5G-RG as a first priority; No solution specific only to FN RG can be proposed, but common solutions for 5G RG and FN RG can be proposed.

## 5.2	Key Issue #2: How to select a TNGF/N3IWF that supports the S-NSSAI(s) needed by the UE

### 5.2.1	Description

As part of TEI17_N3SLICE it has not been possible to work on How to select a TNGF/N3IWF that supports the S-NSSAI(s) requested by the UE during registration via non-3GPP access network because TEI17_N3SLICE had the constraint not to impact the UE.

The key issue will study solutions allowing to select a TNGF/N3IWF that supports the S-NSSAI(s) needed by the UE.

Work on this KI may address roaming; Network (e.g. PLMN) selection shall not be impacted by solutions to this KI.

## 5.X	Key Issue #X: <Key Issue Title>

### 5.X.1	Description

Editor's note:	This clause provides a description of the key issue.

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | KI1 | KI2 |
| --- | --- | --- |
| Solution 1: N3GPP device behind 5G-RG | X |  |
| Solution 2: UE behind 5G-RG and FN-RG | X |  |
| Solution 3: Differentiated QoS for N5CW devices behind 5G-RG | X |  |
| Solution 4: Solution of providing differentiated service for Non-3GPP devices connected behind a 5G-RG | X |  |
| Solution 5: 5GC-capable UE behind 5G-RG accessing 5GC | X |  |
| Solution 6: New method for non-3GPP device connected behind a 5G-RG | X |  |
| Solution 7: Differentiated QoS for non-3GPP devices behind 5G-RG | X |  |
| Solution 8 - Support of "combo Ethernet + IP" service | X |  |
| Solution 9: 5GC-capable UE behind 5G-RG using untrusted Non-3GPP access | X |  |
| Solution 10: Registration via Trusted Non-3GPP Access with TNGF Relocation |  | X |
| Solution 11: Registration via Untrusted Non-3GPP Access with N3IWF Relocation |  | X |
| Solution 12: slice related TNGF selection for WLAN access |  | X |
| Solution 13 : UE getting the slices supported by N3IWF (s) |  | X |
| Solution 14: use of AN NRF |  | X |
| Solution 15: Selecting N3IWF supporting the S-NSSAI needed by UE |  | X |
| Solution 16: N3IWF selection taking supported slices into account |  | X |
| Solution 17: Untrusted Non-3GPP Access with N3IWF/TNGF Relocation due to network slicing |  | X |
| Solution 18: ANDSP based solution for slice related TNGF/N3IWF selection |  | X |
| Solution 20: providing differentiated service for UE and non-3GPP devices connected behind a 5G-RG | X |  |
| Solution 21: Non-3GPP device behind 5G-RG based on 5GS exposure | X |  |
| Solution 22: Support differentiated QoS for AUN3 devices | X |  |
| Solution 23: Delay budget for non-3GPP devices behind 5G-RG | X |  |
| Solution 24: Differentiated QoS for Non-Authenticable non-3GPP devices behind RG | X |  |
| Solution 25: Differentiated QoS for Authenticable non-3GPP devices behind RG. | X |  |

## 6.1	Solution 1: N3GPP device behind 5G-RG

### 6.1.1	Description

#### 6.1.1.1	Overall architecture

This solution addresses KI#1 and describes a solution for how to provide differentiated service (e.g. QoS and charging) for Non-3GPP devices connected behind a 5G RG.

The N3GPP devices are locally authenticated by the 5G-RG using such as pre-shared secret etc. As such, it is invisible to the 5G network though 5G-RG is subscribed to the 5G-Core. Differentiated QoS can still be provided by using local configuration in the 5G-RG and based on DNN / S-NSSAI for the 5G RG's PDU Sessions.

This solution allows for traffic segregation on both PDU session / S-NSSAI and per QoS flow basis. We assume that, when it is per PDU session basis, the default QoS associated with the PDU session can be different based on the DNN and S-NSSAI. 5G-RG can set up distinct PDU Session for each N3GPP device or a group of N3GPP devices based on local configuration in the 5G-RG. For e.g. consider admin settings on the 5G-RG where the admin can group N3GPP devices based on mac addresses and set different QoS levels/priority levels for a group of N3GPP devices. Based on these local configurations the 5G-RG can provide distinct PDU session for each N3GPP device. When these devices have common QoS sessions and DNN / S-NSSAI to be served then, these devices are classified into "connectivity groups" where each group connects to the 5G-RG using a separate physical or virtual port. These ports could for e.g. refer to a separate physical Ethernet ports and/or a separate WLAN SSIDs and the groups may also be separated on the LAN behind the RG using VLANs.

Each physical/virtual port (connectivity group) is then mapped to a separate PDU Session that is established by the 5G-RG based on existing procedures defined in TS 23.316 [5]. The overall architecture is illustrated in Figure 6.1.1-1. Only the case where 5G-RG connects via wireline access is shown in the Figure. The 5G-RG may also connect via 3GPP access (not shown).

Figure 6.1.1-1: Example scenario for Non-3GPP devices behind 5G-RG based on connectivity groups

The 5G-RG is configured with e.g. VLANs and SSIDs and the corresponding DNN/S-NSSAI.

Editor's note:	It is FFS if the configuration can be achieved by TR-69, or via PCF/URSP or both.

It is assumed that the non-3GPP devices are manually configured to use a specific SSDI or connect to a certain Ethernet port on the 5G-RG.

Editor's note:	It is FFS whether if/how the non-3GPP devices can be enforced/authorized to use a certain SSID or Ethernet port.

#### 6.1.1.2	Support for differentiation of e.g. charging and QoS

The connectivity groups are fully separated from each other and differentiation of charging and QoS is provided via existing mechanisms. The solution also allows isolation of the connectivity groups into separate network slices, i.e. with separate N-SSAIs for each PDU Session.

### 6.1.2	Procedures

The procedures for 5G-RG connecting via W-5GAN and/or NG-RAN are captured in TS 23.316 [5].

### 6.1.3	Impacts on Existing Nodes and Functionality

No impacts to 3GPP entities compared to 3GPP Rel-16.

5G-RG:

-	Be able to establish one PDU Session per connectivity group, or per non-3GPP device.

-	Be able to map one or multiple non-3GPP devices to a PDU Session of the 5G-RG. This could e.g. be done based on SSID or Ethernet port used by the non-3GPP device

Editor's note:	The BBF related functionalities, e.g. usage of TR-69, should be supported based on existing BBF specifications but this needs to be verified.

Editor's note:	Impacts to W-5GAN related entities and functionality may need to be verified with BBF and/or Cablelabs.

## 6.2	Solution 2: UE behind 5G-RG and FN-RG

### 6.2.1	Description

#### 6.2.1.1	Overall architecture

This solution addresses KI#1 and describes how a UE behind a 5G-RG/FN-RG can connect to 5GC. The solution is based on existing solutions described in TS 23.316 [5] clause 4.10 (UE connecting to TNGF via RG) and TS 23.316 [5] informative Annex A (UE connecting to N3IWG via RG). The solution assumes that the UE is capable of handling the 5G NAS protocol over WLAN.

An RG connecting via W-5GAN or NG-RAN access towards 5GC can provide connectivity for a UE behind the RG to access an N3IWF or TNGF. It is assumed that the UE is 5GC capable, i.e. supports untrusted non-3GPP access and/or trusted non-3GPP access. This allows the RG, W-5GAN and the RG's connectivity via 5GC to together act as untrusted/trusted N3GPP access to support UEs behind the RG.

When FN-RG/5G-RG is serving a UE, the control and user plane packets of the UE is transported using a FN-RG/5G-RG IP PDU session and then from PSA UPF of that PDU session to an IWF. A single FN-RG/5G-RG IP PDU session can be used to serve multiple UEs.

Figure 6.2.1-1 shows the non-roaming architecture for a UE, behind a 5G-RG, accessing the 5GC via N3IWF where the combination of 5G-RG, W-5GAN and UPF serving the 5G-RG/FN-RG is acting as an untrusted Non-3GPP access network.

Figure 6.2.1-2 shows the non-roaming architecture for a UE, behind a 5G-RG, accessing the 5GC via TNGF where the combination of 5G-RG, W-5GAN and UPF serving the 5G-RG is acting as a trusted Non-3GPP access network.

NOTE  1:	FN-RG and W-5GAN acting as trusted Non-3GPP access is not considered since it is assumed that FN-RG does not support EAP-based access control (e.g. 802.1X).

Figure 6.2.1-1: Architecture for UE behind 5G-RG using untrusted N3GPP access

The 5G-RG can be connected to 5GC via W-5GAN, NG-RAN or via both accesses. The UE can be connected to 5GC via untrusted non-3GPP access (via 5G-RG), NG-RAN or via both accesses.

Figure 6.2.1-2: Non-roaming architecture for UE behind 5G-RG using trusted N3GPP access

The TNGF and Ta reference point are defined in TS 23.501 [2].

NOTE 2:	The reference architecture in figure 6.2.1-1 and 6.2.1-2 only shows the architecture and the network functions directly connected to W-5GAN or TNGF/N3IWF, and other parts of the architecture are the same as defined in clause 4.2 of TS 23.501 [2].

The salient features of this solutions are the following:

-	For untrusted non-3GPP access:

-	UE connects to the overlay 5G network using the untrusted non-3GPP access approach as illustrated in Annex A of TS 23.316 [5].

-	For trusted non-3GPP access:

-	UE connects to the overlay 5G network using the trusted non-3GPP access approach as illustrated in clause 4.10 of TS 23.316 [5].

-	In addition to being connected to the underlay 5G network, the 5G-RG also acts as TNAP with respect to the TNGF in the overlay network i.e. it has an established Ta reference point with the TNGF.

-	For NWu scenario, the overlay and the underlay network as described in clause D.7 of TS 23.501 [2] may be administrated by different operator's domain, hence an SLA can be used (as defined clause D.7 of TS 23.501 [2]). If the same operator deploys both overlay and the underlay network, e.g. as in the case for trusted non-3GPP access (NWt), there is no need for SLA but that operator's network configuration determines how to handle the DSCP markings in the underlay network.

-	The N3IWF/TNGF in the overlay network translates the QoS request received over N2 into DSCP marking that is applied to the outer header of the IPsec tunnel encapsulation.

-	The N3IWF/TNGF may inform the UE about the DSCP marking to apply in the uplink as defined in clauses 4.12 and 4.12a of TS 23.502 [3].

#### 6.2.1.2	Support for differentiated charging and QoS

Support for QoS differentiation can be achieved in a similar way as it is handled when a UE connects to a PLMN via SNPN (clause 5.30.2.7 and clause D.7 of TS 23.501 [2],). Also differentiated charging, both in the RG's PLMN and in the UE's PLMN, can be achieved based on existing mechanisms. This is further described below.

QoS differentiation in the RG's PDU Session can be provided on per-IPsec Child Security Association basis. The UE's N3IWF/TNGF determines the IPsec child SAs as defined in clauses 4.12 and 4.12a of TS 23.502 [3]. The N3IWF/TNGF is preconfigured by the UE's PLMN to allocate different IPsec child SAs for QoS Flows with different QoS profiles.

To support QoS differentiation in the W-5GAN, the mapping rules between the RG's 5GC and the UE's 5GC (where N3IWF is located) are assumed to be governed by an SLA (or network configuration in case of single operator) including:

1)	mapping between the DSCP markings for the IPsec child SAs on NWu and the corresponding QoS, and

2)	N3IWF IP address(es) in the UE's 5GC.

The non-alteration of the DSCP field on NWu is also assumed to be governed by an SLA and by transport-level arrangements that are outside of 3GPP scope. The packet detection filters in the RG's UPF can be based on the N3IWF IP address and the DSCP markings on NWu.

It is assumed that the same set of DSCP values and corresponding QoS are applicable independent of whether UE-requested or network-initiated QoS is used.

To enable differentiated charging similar mechanisms are applied. It is assumed that the awareness of N3IWG/TNGF IP addresses as well as RG PDU Session IP addresses are governed by SLA between UE's 5GC and RG's 5GC (or network configuration in case of single operator).

In the RG's PLMN, the IP addresses belonging to N3IWF/TNGF in UE's PLMN can be provided in packet filters to the RG's UPF together with a separate Rating Group. In addition, to enable service-based differentiation, packet detection filters in the RG's UPF can also include the DSCP markings on NWu. This is similar to how QoS differentiation is done and would enable differentiated charging in the RG's PDU Session on a per-IPSec Child Security Association basis.

In the UE's PLMN, the N6 IP addresses available on the DNN to which the RG establishes its PDU Session are assumed to be known based on SLA (or network configuration in case of single operator). When the UE connects to N3IWF/TNGF, one of these IP addresses will be used as the UE's local IP address (i.e. N6 address of the RG's PDU Session). This local UE IP address will be known to the UE's SMF since it is part of the ULI provided from N3IWF/TNGF to AMF and forwarded to SMF. The ULI is also provided to the UE's PCF. It is thus possible for SMF/PCF to apply differentiated charging (e.g. different Rating Groups) when the UE is accessing N3IWF/TNGF via W-5GAN.

QoS differentiation in the underlying 5G-RG's PDU Session can be provided on per-IPsec Child Security Association basis by using the network requested PDU Session Modification procedure described in clause 4.3.3.2 of TS 23.502 [3].

To support QoS differentiation in the underlying 5G-RG's PDU Session, the PCF can generate separate PCC rules based on the DSCP values and the corresponding QoS. Based on this, the SMF initiates PDU Session modification procedure for an IPSec SA based on mapping between the DSCP markings for the IPsec child SAs and the corresponding QoS in the PLMN. The packet detection filters in the underlying 5G-RG's PDU Session can be based on the N3IWF/TNGF IP address and the DSCP markings.

Editor's note:	Whether and how 5G-RG can initiate PDU Session modification procedure for an IPSec SA is FFS.

### 6.2.2	Procedures

The procedures for 5G-RG connecting via W-5GAN and/or NG_RAN are captured in TS 23.316 [5].

The procedures for FN-RG connecting via W-5GAN are captured in TS 23.316 [5].

The procedures for a UE connecting via N3IWG and TNGF are captured in TS 23.502 [3].

### 6.2.3	Impacts on Existing Nodes and Functionality

No impacts compared to 3GPP Rel-17. It is assumed that an SLA exist between the RG's PLMN and the UE's PLMN (or appropriate network configuration, in case of same operator).

Editor's note:	It is FFS if (5G/RG+TNAP) and TNGF can be administrated by different operator.

It is assumed that the SMF initiates PDU Session modification procedure for each IPSec SA based on PCC rules with the DSCP values and the corresponding QoS in the PLMN (supported by existing standards).

## 6.3	Solution 3: Differentiated QoS for N5CW devices behind 5G-RG

### 6.3.1	Description

The solution describes how differentiated QoS can be provided to the following type of devices residing behind a 5G-RG:

-	Non-5G Capable WLAN (N5CW) device, which is not capable of handling the 5G NAS protocol over WLAN, but is equipped with a USIM.

The architecture for providing differentiated services to a N5CW device behind 5G-RG is illustrated in Figure 6.3.1-1.

Figure 6.3.1-1: N5CW device behind 5G-RG using trusted WLAN access for N5CW device

The salient features of this solutions are the following:

-	N5CW device connects to the overlay 5G network using the trusted WLAN access for N5CW device approach.

-	In addition to being connected to the underlay 5G network, the 5G-RG also acts as TWAP with respect to the TWIF in the overlay network i.e. it has an established Yw reference point.

-	The traffic between TWAP and TWIF is carried in IPsec tunnels, each IPsec tunnel corresponding to a pre-defined QoS level. Each IPsec tunnel can carry traffic from more than one N5CW device. Each N5GW device is associated with only one QoS level.

-	Provided Yw has such a signalling capability, each IPsec tunnel establishment can be initiated by the TWIF on the fly when the QoS associated with a N5CW device does not correspond to the QoS of any of the established IPsec tunnels. Otherwise, all the IPsec tunnels corresponding to all supported QoS levels need to be pre-established ahead of time (i.e. before any N5CW device associates with the 5G-RG).

-	The TWIF in the overlay network translates the QoS request received over N2 into DSCP marking that is applied to the outer header of the IPsec tunnel encapsulation. The IPsec tunnel is selected based on the QoS level only (i.e. the IPsec tunnels are not specific to a N5CW device).

-	The TWAP extracts the inner IP packet and forwards the decapsulated IP packet to the N5CW device using the DSCP marking in the outer IPsec header for local QoS handling on the Yt' interface. The TWAP also keeps track of the QoS associated with this N5CW device and applies the same QoS in the uplink i.e. the TWAP receives the uplink packets from specific N5CW device over Yt' and forwards them via the IPsec tunnel associated with the same QoS level.

-	There is an SLA between the overlay and the underlay network as described in clause D.7 of TS 23.501 [2] defining how the DSCP marking in the outer IP header of the IPsec Child SA (of the IPsec tunnel(s) established between TWAP and TWIF) is to be handled by the underlay network for differentiated QoS.

### 6.3.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution. To be completed.

Editor's note:	define ways how the 5G RG can discover a TWIF in another network.

Editor's note:	The establishment of IPsec tunnels for handling QoS between TWAP and TWIF requires further clarifications (e.g., based on operator configurations, etc.).

### 6.3.3	Impacts on Existing Nodes and Functionality

Specify (if needed) Yw as a standard inter operator interface (in R17 The Yw reference points is outside the scope of the 3GPP specifications) and define ways how the 5G RG can discover a TWIF in another network.

The solution largely re-uses existing functionality.

Impact to 5G-RG (in the role of TWAP):

-	Ability to pre-establish QoS-specific IPsec tunnels with the TWIF. Optionally use Yw signalling to establish IPsec tunnels with the TWIF on the fly.

-	Ability to keep track of QoS level associated with a N5CW device based on DSCP marking in downlink packets and apply the same QoS to uplink packets by forwarding them to the corresponding IPsec tunnel.

Impact to TWIF:

-	Optionally use Yw signalling to establish IPsec tunnels with the TWAP on the fly.

## 6.4	Solution 4: Solution of providing differentiated service for Non-3GPP devices connected behind a 5G-RG

### 6.4.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub) clause(s) may be added to capture details.

This solution provides differentiated service for non-3GPP devices connected behind a 5G-RG. In this solution, the 5G-RG receives QoS rules for the non-3GPP devices behind the 5G-RG. Then the 5G-RG uses the corresponding data transfer tunnel to transfer the packets of the non-3GPP devices according to the QoS rules.

This solution is used for Non-Authenticable Non-3GPP (NAUN3) devices.

The 5G-RG is pre-configured or receives session policy for non-3GPP devices from the network during its PDU Session Establishment procedure. The session policy for non-3GPP devices describes the following information:

a) 	to establish a QoS flow for one category of non-3GPP devices;

b) 	the DNN / S-NSSAI corresponding to the PDU session of the QoS flow for non-3GPP devices;

c) 	the non-3GPP device category identifier. The non-3GPP device category identifier is used to identify one or a group of non-3GPP devices.

When the 5G-RG detects the connection of a non-3GPP device, the 5G-RG requests to establish a QoS flow for the non-3GPP device according to the session policy. If there is already a corresponding PDU Session (i.e. a PDU sessions with the same DNN/S-NSSAI), the 5G-RG performs PDU Session Modification procedure. If there is no corresponding PDU Sessions (i.e. no PDU sessions with the same DNN/S-NSSAI), the 5G-RG performs PDU Session Establishment procedure.

### 6.4.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

Figure 6.4.2-1 5G-RG establishes a QoS flow for one category of non-3GPP devices

1.	non-3GPP device connects to 5G-RG with non-3GPP access network (e.g. establishes WiFi association).

During this procedure, 5G-RG obtains non-3GPP device category identifier. The 5G-RG may need to map the non-3GPP device MAC address, WLAN SSID or physical Ethernet port identifier used by the non-3GPP device to non-3GPP device category identifier. The 5G-RG performs such mapping based on local configuration.

Editor's note:	It is FFS how the 5G-RG obtains the local configuration for mapping non-3GPP device to non-3GPP device category identifier, e.g. operator configuration or other kinds of configurations.

Editor's note:	It is FFS whether and how to standardize the non-3GPP device category identifier.

The 5G-RG may configure IP address (i.e. using IPv6 prefix delegation or local IPv4 address allocation mechanism) to the non-3GPP device.

2.	5G-RG sends NAS message to the AMF. The NAS message includes a PDU Session Modification Request, including PDU Session ID, 5G-RG ID and non-3GPP device category identifier. Optionally, non-3GPP indication is also included.

The 5G-RG determines to request to establish a QoS flow for the category of non-3GPP devices according to the session policy for non-3GPP devices.

NOTE:	It is assumed that the 5G-RG already establishes a PDU session.

3.	The AMF sends N11 message to the SMF and forwards the PDU Session Modification Request.

4.	The SMF reports non-3GPP device category identifier to the PCF by initiating the SM Association Modification procedure.

The PCF provides QoS rules for the category of non-3GPP devices to the SMF.

5.

For Ethernet PDU Session type, the SMF updates the UPF with N4 Rules by indicating that this PDU session is also used to transfer the traffic related to the non-3GPP device's MAC address.

6.	The SMF generates PDU Session Modification message including the QoS rules for the non-3GPP device.

The SMF sends N11 message to the AMF, including the PDU Session Modification message, 5G-RG ID and non-3GPP device category identifier.

7.	The AMF sends NAS message to the 5G-RG, forwarding the PDU Session Modification message to the 5G-RG.

The 5G-RG maps the traffic of the non-3GPP device to the QoS flow of this PDU session according to the received QoS rule.

When the 5G-RG detects disconnection of the non-3GPP device, if there is no traffic of other non-3GPP devices maps to the QoS flow, the 5G-RG initiates PDU Session Modification procedure to release such QoS flow.

Figure 6.4.2-2 5G-RG establishes a PDU session for one category of non-3GPP devices

If the 5G-RG determines to establish a PDU session for the non-3GPP device according to the session policy for non-3GPP devices, the 5G-RG performs PDU session establishment procedure according to TS 23.502 clause 4.3.2.1 with the following differences:

In step 1, 5G-RG includes 5G-RG ID and non-3GPP device category identifier in PDU Session Establishment Request.

During SM Policy Association Establishment procedure, in step 3, the SMF reports non-3GPP device category identifier to the PCF, and in step 4, the PCF sends PCC policy corresponding to the non-3GPP device category identifier.

In step 6 and 7, the SMF includes the non-3GPP device category identifier and QoS rule of the non-3GPP device in PDU Session Establishment Response.

The 5G-RG maps the traffic of the non-3GPP device to this PDU session according to the received QoS rule.

Figure 6.4.2-3 5G-RG receives session policy for non-3GPP devices

The 5G-RG receives the session policy for non-3GPP devices during its own PDU Session establishment procedure. The 5G-RG performs PDU session establishment procedure according to TS 23.502 clause 4.3.2.1 with the following differences:

In step 1, 5G-RG includes non-3GPP access indication in PDU Session Establishment Request to requests the session policy for non-3GPP devices behind it.

During SM Policy Association Establishment procedure, in step 3, the SMF reports non-3GPP access indication to the PCF and in Step 4, the PCF sends session policy for non-3GPP devices to the SMF.

In step 6 and 7, the SMF sends the session policy for non-3GPP devices to the UE in PDU Session Establishment Response.

Editor's note:	It is FFS whether the session policy for non-3GPP devices could be provided via URSP.

Editor's note:	The RG impacts in this solution needs to be checked by BBF.

### 6.4.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

5G-RG:

-	5G-RG requests for QoS rule  for the non-3GPP device behind it, by sending PDU session modification message to the network and receives the requested information from the network.

-	5G-RG receives session policy for non-3GPP devices from the network and requests to establish PDU Session or QoS flow based on the session policy for non-3GPP devices.

-	5G-RG includes non-3GPP access indication in PDU Session Establishment Request to requests the session policy for non-3GPP devices behind it.

-	5G-RG maps the MAC address, WLAN SSID or physical Ethernet port identifier used by the non-3GPP device to non-3GPP device category identifier based on local configuration.

SMF:

-	The SMF requests QoS rule for non-3GPP device from the PCF.

-	The SMF sends the non-3GPP access indication to the PCF and receives session policy for non-3GPP devices.

-	The SMF sends the session policy for non-3GPP devices to the UE in PDU Session Establishment Response.

PCF:

-	The PCF provides QoS rules for non-3GPP devices.

-	The PCF provides session policy for non-3GPP devices to the SMF.

## 6.5	Solution 5: 5GC-capable UE behind 5G-RG using trusted Non-3GPP access

### 6.5.1	General

This solution is applicable when 802.1x (EAP over LAN) and 3GPP credentials are used to associate with the WLAN. In this option, the solution for trusted N3GPP is used as baseline but with the additional property that the service is offered via a wireline access and a 5G-RG. This solution can distinguish the 5GC-capable UE role and apply the different QoS and charging policy for the different categories of devices, for example for 5GC capable UE considered as host or guest of the local network of 5G-RG/HA 5G-RG. The member of the family decides on which 5GC capable UE considered as host (i.e. subscriber of the 5G-RG) and reports these 5GC capable UE IDs (e.g. MSISDN) to the operator via online or offline service. This is out of the scope of 3GPP. After that, it is the 5GC's responsibility to maintain such information and decides the 5GC capable UE role based on it.

The solution is based on the following architecture as defined in clause 4.10 of TS 23.316 [5]. The Ta interface is a non-roaming interface, which means the 5G-RG and TNGF belongs to the same PLMN. Then the UE and the 5G-RG belong to the same PLMN too.

NOTE:	For the UE accessing 5GC, the 5G-RG is treated as WLAN AP connecting to TNGF, so the same slice is supported by the 5G-RG and the TNGF.

Figure 6.5.1-1: Non-roaming architecture for UE behind 5G-RG using trusted N3GPP access

### 6.5.2	Registration procedure

In this solution, the 5G-RG acts as TNAP and is behaved as authenticator, the interface between the 5G-RG and W-5GAN is assumed an AAA interface.

Figure 6.5.2-1: Registration procedure

0.	5G-RG has successfully registered to the AMF1. 5G-RG obtained the 5G-RG GUTI from 5GC.

A layer-2 connection is established between the UE and the 5G-RG. In case of IEEE 802.11 this step corresponds to an 802.11 Association. Optionally, the 5G-RG may send 5G-RG 5G-GUTI to the UE via ANQP message.

Editor's note: Security aspects of sending 5G-RG 5G-GUTI in ANQP should be verified by SA3.

1a-1c.	An EAP authentication procedure is initiated. EAP messages are encapsulated into layer-2 packets, i.e. into Ethernet-over-LAN packets. The UE provides a NAI in the EAP-Res/Identity message. This EAP-Res/Identity is encapsulated in the AAA message as defined in clause 4.12a.2.2 step 3 of TS 23.502 [3] by 5G-RG. If the 5G-RG does not transport the 5G-GUTI via ANQP message, the 5G-RG also includes 5G-RG 5G-GUTI in this AAA message. The AAA message is encapsulated over IP layer, the source IP address is the 5G-RG's IP address allocated by the 5GC in the PDU session established procedure via wireline or wireless access, the destination IP address is the TNGF IP address configured on the 5G-RG side or queried through the DNS procedure.

1d.	An EAP-5G procedure is executed as specified in TS 23.502 [3] and TS 33.501 [9]. The W-5GAN sends the EAP-5G start message including the 5G-RG GUTI to the UE.

1e.	The UE sends the AN parameters (e.g. UE SUCI or the 5G-GUTI, Requested NSSAI, etc.) and NAS Registration request message (e.g. Registration type, UE SUCI or 5G-GUTI, Security parameters, Requested NSSAI, etc.) in the EAP-5G. In addition, in the NAS Registration request message, the UE includes the 5G-RG 5G-GUTI received from the EAP-5G start message. The 5G-RG 5G-GUTI is used by AMF to find the 5G-RG context and obtain 5G-RG SUPI.

2.	W-5GAN selects the AMF2 for UE as specified in TS 23.501 [2] clause 4.2.2.2.2 with the UE 5G-GUTI or Requested NSSAI. This AMF2 may be the same or different from the AMF1.

3-4.	The AMF2 received the Registration request message and the UE authentication procedure is performed based on the UE 3GPP credential as defined in TS 23.502 [3] and TS 33.501 [9].

5-6.	With the existing of 5G-RG GUTI, the AMF2 knows that the UE is accessing 5GC via 5G-RG. In order to obtain the 5G-RG SUPI, the AMF2 finds the 5G-RG context based on the 5G-RG 5G-GUTI. If there is no 5G-RG context in the AMF2, AMF2 requests the 5G-RG SUPI from AMF1. To be more specific, the AMF2 selects the AMF1 based on the 5G-RG 5G-GUTI and sends the request message to the AMF1 including the 5G-RG 5G-GUTI and SUPI requested indication. AMF1 finds the 5G-RG context based on the 5G-RG 5G-GUTI and replies the 5G-RG SUPI to the AMF2.

7-8.	The AMF2 selects the UDM based on 5G-RG SUPI, and requests 5G-RG's subscribed UE ID list from the UDM.. The 5G-RG's subscribed UE ID list includes the UE IDs, e.g. UE SUPI or UE MSISDN. If the UE ID sent by AMF2 to UDM belongs to the subscribed UE ID list, the UE is treated as the subscriber of the 5G-RG. If the UE ID does not belong to the subscribed UE ID list and the 5G-RG only allows the subscribed UE, the UDM shall reject the UE. The UDM stores the serving AMF identity and will notify the AMF when the 5G-RG's subscribed UE ID list is updated.

9.	The UDM sends the subscribed UE ID list to the AMF2. AMF2 stored the subscribed UE ID list and decides if the UE is the subscribed UE of the 5G-RG. If the UE is not the subscriber of 5G-RG and the 5G-RG only allows the subscribed UE, the AMF shall reject the UE.

10-11.	The AMF2 requests the policy from PCF.

12-16.	The same steps as defined in clause TS 23.502 [3] clause 4.12a.2 steps 10a to 15b.

During the PDU session establishment procedure triggered by the remote UE, the procedure defined in TS 23.502 subclause 4.12a.5 is applied, with the following difference:

Step 2a: If the AMF2 identifies the UE as the subscribed UE of the 5G-RG, it sends the subscribed UE indication to the SMF2. The SMF2 forwards this information to the PCF, the PCF may create the different QoS or/and charging rules based on the UE role, i.e. subscribed UE or unsubscribed UE. For example, the PCF will apply the higher priority 5QI value for the subscribed UE traffic than the unsubscribed UE. The SMF2 creates the DSCP value based on 5QI and the UE role.

Step 2b: The SMF2 sends N2 SM information to the TNGF, including the mapping between the QFI and the DSCP value. The TNGF can provide DSCP value to UE for the User Plane IPsec Child SA at PDU Session Establishment as defined in clause 4.12.5, step 4a and 4c.

In the UE’s PLMN, for the downlink data, the TNGF marks the DSCP in the outer IP header based on the mapping between the QFI and DSCP received from SMF2. For the uplink, the UE marks the DSCP using the DSCP value associated with the Child SA.

In the 5G-RG’s PLMN, the QoS is controlled per description for the PCF, SMF and UPF in the Annex D.7 of the TS 23.501 [2].

### 6.5.3	Impacts on existing Functions

UDM:

-   The 5G-RG subscription data is extended to include the subscribed UE list.

AMF:

-   Obtain the 5G-RG GUTI from NAS message, and then request the 5G-RG SUPI from the 5G-RG’s serving AMF.

-   Request the 5G-RG’s subscribed UE list from UDM based on the 5G-RG SUPI.

PCF:

-   Decide the 5QI based on the UE role, i.e. the subscribed UE or unsubscribed UE indicated by the SMF.

SMF:

-   Transfer to the PCF the UE role, i.e. the subscribed UE or unsubscribed UE received from the AMF.

5G-RG:

-   Send the 5G-RG GUTI to the UE. There are two options, the impacts on each one is as follows.

Option 1: The 5G-RG sends the 5G-RG GUTI to TNGF via the Ta interface. As the 5G-RG and TNGF are defined by 3GPP, the Ta interface can also be defined by 3GPP.

Option 2: The 5G-RG sends the 5G-RG GUTI to the UE via ANQP message.

UE:

-   Send the 5G-RG GUTI to the AMF via the NAS message.

## 6.6	Solution 6: New method for non-3GPP device connected behind a 5G-RG

### 6.6.1	Description

This solution addresses Key Issue #1"Providing differentiated service for UE and Non-3GPP devices connected behind a 5G RG".

Figure 6.6.1-1 depicts the architecture for devices connected behind a 5G-RG. In this solution, the AF, which supports the functionality of assigning virtual identifier (e.g. virtual SUCI) can preconfigure a list of the virtual identifier to the UDM through NEF. After the 5G-RG registers to the 5GC, a virtual identifier would be assigned to it as the identifier of the device which has been connected to 5G-RG. Then the 5G-RG could use the virtual identifier to initial registration procedure and PDU session establishment procedure via W-AGF on behalf of the device. Different QoS profiles may be applied per device behind the 5G-RG by 5GC.

NOTE 1:	AF in figure 6.6.1-1 is managed by operator.

NOTE 2:	The interaction between the AF and 5G-RG may be out of 3GPP scope.

NOTE 3:	The virtual identifier corresponds to 5G-RG's subscription information in UDM.

Editor's note:	How to resolve randomized MAC address is FFS.

Figure 6.6.1-1: architecture for non-3GPP devices connected behind a 5G-RG

### 6.6.2	Procedures

Figure 6.6.2-1: Procedure for 5G RG and Device Registration

0.	The AF preconfigures the list of virtual identifier (e.g. virtual SUPI) to UDM through the NEF, so that the virtual identifier can be a legal identifier of the non-3GPP device behind the 5G-RG for which the 5G-RG is doing the authentication procedure on behalf.

1.	The 5G-RG registration procedure, as descibed in clause 7.2.1.1 of TS 23.316 [5].

2.	The UDM informs the AF that the 5G-RG has been successfully registered with 5GC. AF should save a list of SUCI of registered 5G-RGs.

3.	The device connects to the 5G-RG via a layer-2 (L2) connection.

4.	The 5G-RG applies to the AF for a virtual identifier by carrying the SUCI of the 5G RG and the MAC address of the device.

5.	The AF checks if the 5G-RG is in the list of registered 5G-RGs. If it has registered successfully, the AF will inform the UDM of the mapping between the virtual identifier and the MAC address of the non-3GPP device.

6.	If the 5G-RG is in the list of registered 5G-RGs, the AF responds to the 5G-RG by assigning a virtual identifier to the device. The AF saves the timestamp and the mapping information between the virtual identifier and the MAC address of the device.

NOTE:	In steps 4 and 6, the AF and 5G-RG communicate directly without going through the 5GC. Step 4 and 6 may be out of the 3GPP scope.

7.	The 5G-RG uses the virtual identifier to initial registration procedure on behalf of the device, as described in clause 7.2.1.1 of TS 23.316 [5].

Editor's note:	Modification of registration procedure in clause 7.2.1.1 of TS 23.316 [5] is FFS.

Editor's note:	How the device is authenticated, e.g. based on which credentials is FFS.

8.	5G-RG initates PDU Session Establishment procedures on behalf of device. And differentiated QoS can be provided to non-3GPP device according to the virtual identifier.

Editor's note:	The detail of how to provide differentiated QoS to non-3GPP device is FFS.

### 6.6.3	Impacts on Existing Nodes and Functionality

UDM:

-	Inform AF that a 5G-RG has been successfully registered.

-	Store the mapping between the virtual identifier and the MAC Address of the non-3GPP device.

5G-RG:

-	Apply to the AF for a virtual identifier.

-	Store the mapping information between the virtual identifier and the MAC address of the non-3GPP device.

-	Support performing registration and PDU Session related procedure on behalf of non-3GPP device behind.

Editor's note:	Whether any impact on other NF is FFS and it depends on the detail of how to provide differentiated QoS to non-3GPP device behind And may depend on Modification of registration and authentication procedure in clause 7.2.1.1 of TS 23.316 [5].

## 6.7	Solution 7: Differentiated QoS for non-3GPP devices behind 5G-RG

### 6.7.1	Description

#### 6.7.1.1	Introduction

QoS experienced by non-3GPP devices connected behind a 5G-RG depends on the end-to-end path between a non-3GPP device and the application server, i.e. depends on the QoS differentiation in both the 3GPP network and the non-3GPP network attached to the 5G-RG.

For example, a certain Packet Error Ratio (PER) can only be successfully ensured if it is enforced in both the 3GPP network and the non-3GPP network attached to the 5G-RG.

To address this shortcoming, this solution proposes to indicate Non-3GPP QoS assistance information to the 5G-RG to enable the 5G-RG to perform QoS differentiation for the non-3GPP devices in the non-3GPP network behind the 5G-RG.

Note that this is conceptually similar as providing Additional QoS Information to a UE for trusted/untrusted access to 5GC as defined in TS 23.502 [3], which enables the UE to reserve resources in the non-3GPP network. The difference is that the Non-3GPP QoS assistance information is used to reserve resource in the non-3GPP network attached to the 5G-RG. In line with the definition of Additional QoS information, the details of how to enforce QoS in the non-3GPP network based on the Non-3GPP QoS assistance information are beyond the scope of 3GPP.

The solution applies to 5G-RG connected via W-5GAN or connected via FWA as defined in TS 23.316 [5].

#### 6.7.1.2	Solution principles

The solution is based on the following principles:

-	During PDU session establishment and PDU session modification, if the SMF provides the 5G-RG with QoS flow descriptions, the SMF may, based on subscription, additionally signal non-3GPP QoS assistance information for each QoS flow to the 5G-RG. The non-3GPP QoS assistance information consists of the following QoS information (if available at the SMF): QoS characteristics, GFBR/MFBR (if applicable), ARP, Periodicity.

Editor's note:	Details of which non-3GPP QoS assistance information to provide, e.g. whether Periodicity needs to be part of the non-3GPP QoS assistance information for the non-3GPP devices behind 5G-RG scenario, are FFS.

-	Based on the non-3GPP QoS assistance information together with QoS rule information, the 5G-RG may reserve resources in the non-3GPP network.

NOTE:	The details of how to enforce QoS based on the Non-3GPP QoS assistance information in the non-3GPP network are not subject of this solution and are considered to be beyond the scope of 3GPP.

### 6.7.2	Procedures

Figure 1: Providing Non-3GPP QoS assistance information to the 5G-RG to enable the 5G-RG to ensure QoS in the non-3GPP network attached to the 5G-RG

1.	PDU Session is established

2.	Application layer signalling is exchanged between an application on a non-3GPP device connected to the 5G-RG and an application function (AF)

3.	Based on the application layer signalling, the AF requests QoS for application flows.

NOTE 1:	If the AF is not considered trusted, then the AF interacts with the NEF instead. If the AF provides explicit QoS parameters, then AF (or NEF in case the AF is not trusted) interacts with PCF as defined in clause 4.15.6.6 of TS 23.502 [3].

4.	PCF sends PCC rules to the SMF.

NOTE 2:	Steps 1-4 are following existing Rel-17 specifications.

5.	SMF performs PDU Session modification as defined in clause 4.3.2 of TS 23.502 [3] and may include, based on subscription, in the N1 SM container per QoS-flow Non-3GPP QoS assistance information. The Non-3GPP QoS assistance information contains (if available at the SMF): QoS characteristics, GFBR/MFBR (if applicable), ARP, Periodicity.

Editor's note:	Details of which non-3GPP QoS assistance information to provide are FFS.

6.	Based on the non-3GPP QoS assistance information and QoS rule information, the 5G-RG may reserve resources in the non-3GPP network.

### 6.7.3	Impacts on Existing Nodes and Functionality

-	SMF:

-	to provide per-QoS flow Non-3GPP QoS assistance information

-	5G-RG:

-	to receive additional Non-3GPP QoS assistance information in NAS-	Optionally enforce QoS in the non-3GPP network The details of how to enforce QoS in the non-3GPP network are not subject of this solution and are considered to be beyond the scope of 3GPP.

Editor's note:	Whether further considerations on 5G-RG would be brought to BBF and/or CableLabs attention is FFS.

## 6.8	Solution 8 - Support of "combo Ethernet + IP" service

### 6.8.1	Overview

This solution addresses KI # 1; it relates to How to improve the support of L2 Bridge 5G-RG scenario for providing differentiated connectivity to devices behind the RG based on BBF requirements as expressed in their LS BBF-291/S2-1903875

This solution is meant to apply to both 5G-RG (connected over FWA and/or via a Wireline access) and FN-RG. The same solution applies to both 5G RG and FN RG.

This solution aims at serving RG working in Bridged and Bridged/Routed mode (at least for some services), where the 5GC may need to:

-	provide an IP service to the Customer premises devices e.g. allocate IPv4/v6 address / Prefix to devices behind the RG.

-	identify each device / terminal behind the RG via its MAC address: for this purpose, Ethernet frames are exchanged between the RG and the 5GC so that the network (UPF) is capable to identify traffic from different individual devices in the Customer Premises. In this solution, a device / terminal behind the RG may be a UE or a non 3GPP device

NOTE 1:	This is e.g. to apply specific policies related with the terminal (e.g. parental control or Enhanced home office service where a work laptop gets bandwidth and prioritization).

This solution is only meant to be used in deployments with RG working in Bridged and Bridged/Routed mode.

The Ethernet PDU Session Type does not support an IP service over N6. For example, TS 23.501 [2] states that "Neither a MAC nor an IP address is allocated by the 5GC to the UE for a PDU Session". Furthermore, with the Ethernet PDU Session the Layer 2 header received from the access side is copied on the N6 side of the PSA while it is not meant to be copied over N6 in the case of an IP service provided to RG working in Bridged and Bridged/Routed mode. In case of Ethernet PDU Session Type DL forwarding is basically done based on the Destination MAC address of the UE while here DL forwarding is to be made based on the IP address of the target device ; in case an IP PDU Session type would be used to support an RG working in Routed mode with no NAT, as many PDU Sessions as there are devices in the Customer Premises requiring an IP address would be required.

A dedicated PDU Session Type called "Combo Ethernet + IP" is used for that purpose. It basically supports an access Router in the PSA while an Ethernet PDU Session supports a bridge in the PSA;

In case of "combo Ethernet + IP" PDU Session Type:

1.	Ethernet frames are exchanged between the RG and the UPF acting as PSA (PDU Session Anchor).

2.	Ethernet frames exchanged between the RG and the UPF acting as PSA carry IP traffic (and IP auxiliary protocols: ARP, ND).

3.	An IP service is delivered at N6 where for example IP addresses are allocated to individual devices (devices in the Customer Premises) behind the RG. Any IP address delivery mechanism may be used to deliver an IP address to the devices behind the RG i.e. SLACC, DHCPV4 or DHCPV6. The SMF related subscription data associated with the RG apply to a whole PDU Session regardless of whether multiple IP addresses are allocated on this PDU Session.

NOTE 2:	IP address(es) allocated to devices can correspond to public or to private addresses.

4.	In the UL direction the UPF acting as PSA receives (as a payload of the GTP-u tunnel coming from the AGF) IP traffic within a Layer 2 framing; This layer 2 framing below IP is called "access side layer 2 header" below. The PSA maps between the IPv4/v6 address / Prefix of a device behind the RG and the " access side layer 2 header" as follows:

-	in UL the "access side layer 2 header" framing coming from a device in the Customer Premises is removed by the PSA. The UPF keeps track of the association between the IP address of the device and the corresponding "access side layer 2 header".

-	in DL the " access side layer 2 header" is added by the UPF before sending the traffic towards a device in the Customer Premises. Necessary MAC address swapping (source/destination) is enforced by the PSA on the "access side layer 2 header" stored from UL traffic.

The "access side layer 2 header" handled by the PSA contains the IEEE 802.3 MAC header possibly augmented by IEEE 802.1q header or by a IEEE 802.1ad header (also known as « QinQ »).

NOTE 5:  It is up to BBF access to determine whether this "access side layer 2 header" includes any VLAN tagging (possibly multiple levels of VLAN tagging) e.g. VLAN tagging added by the BBF access on top of the UL Ethernet frames received from the RG. The PSA performs the mapping above regardless of whether the " access side layer 2 header" contains 0, 1 or 2 level of VLAN tagging. Potential VLAN tagging addition by 5G RG is defined by existing BBF specifications.

For a Combo Ethernet and IP PDU session type the RG exchanges with the 5GC IP packets each carried within an Ethernet frame. The 5GC delivers an IP service to the RG and to devices locally connected to the RG while it is able to identify traffic of these devices by the MAC address they are using.

NOTE 6:	No new traffic filters (for N4 PDR or PCC rule or N1 SM QoS related signalling) need to be defined.

Figure 6.8.1-1: Protocol stack for the combo PDU Session Type

For UL traffic, the Ethernet layer 2 framing depicted in Figure 6.8.1-1 above is subject of following processing:

-	The RG may add a VLAN header (*) to the traffic received from the devices in the customer premises.

-	The BBF access may add a VLAN header (**) (e.g. S-tag) to the traffic received from the RG.

For DL traffic, the Ethernet layer 2 framing depicted in Figure 6.8.1-1 above is subject of following processing:

-	The BBF access may strip off a VLAN header (**) (e.g. S-tag) to the traffic received from the network.

-	The RG may strip off a VLAN header (*) to the traffic received from the network.

In this release of the specification, SSC mode 3, UL CL and IPV6 multihoming do not apply to a PDU Session of the Combo type.

In this release of the specification, the Combo PDU Session type applies only to the Wireline access.

A RG may use simultaneously a PDU Session with the "combo Ethernet + IP"  PDU Session type and PDU Session(s) with another PDU Session type.

A 5G-RG configured to work in Bridged and Bridged/Routed mode requests PDU Sessions with the PDU Session Type "combo IP + Ethernet" or may be associated in its subscription with this default PDU Session Type.

NOTE 3:	This PDU Session Type has many similarities with the Ethernet PDU Session Type but services described in bullets 3 and 4 above are not supported by the Ethernet PDU Session Type.

The support of "combo IP + Ethernet" PDU Session type assume that multiple devices served by the PDU Session may request an IP address at any time during the lifetime of the PDU Session. The release of an IP address to such device (e.g. when the DHCP lease has expired) does not release the PDU Session. The SMF may allocate multiple IPv4 addresses in one PDU session of "combo IP + Ethernet" PDU Session type (when each device has its own IP address). IP address allocation takes place as in R17, i.e. options for SMF and UPF to allocate IP address, SMF handling the DHCP signalling, SLAAC etc are supported

There is an unique UDM subscription for the RG regardless of the number of devices in the customer premises that are subject of a specific handling.

When some devices are subject of a specific service, the identity of these devices (MAC address) and the parameters of their specific service (e.g. Access restriction for parental control, dedicated QoS) are configured in the policy (PCF) data related with the subscription (UDR). Other devices benefit of a baseline service and do not require to be individually identified in the policy data related with the subscription (UDR).

### 6.8.2	Description of the solution

The "Combo Ethernet + IP" PDU Session Type supports following features:

-	An IP service is delivered at N6 where for example IP addresses are allocated to individual devices behind the RG. The same IP address allocation mechanisms (including deferred IPv4 and DHCPV6 IP address allocation) are supported than in case of RG supported by the IP PDU Session Type.

-	The SMF indicates to the PSA that a PDU Session is of "Combo Ethernet + IP" type at the N4 Session Establishment. Based on this the PSA (UPF) acts as the first hop router of the devices in the customer premises with regard to the handling of the layers below IP. The SMF remains responsible of the RA/RS/ND handling. Especially it supports the behaviour described in bullet 4 of clause 6.8.1 related with " access side layer 2 header".

Editor's note: It is FFS whether signaling a new " 'combo Ethernet + IP' service"  PDU Session type can be avoided to be signalled by/to the 5G-RG (i.e. Ethernet PDU Session type can still be signaled by/to 5G-RG).

-	N4 FAR and PDR are those used for an IP PDU Session Type.

-	based on the PDU Session type, the UPF handles the Ethernet layer towards the UE on its own without instructions from SMF

-	The QER is updated for the SMF to be able to control the Priority Code Point of the VLAN Headers within the "access side layer 2 header".

-	When an IP address/prefix has been allocated to the PDU Session the SMF reports it together with the corresponding MAC address (of the RG or of a device behind the RG) to the PCF and the PCF may use this MAC address to derive policies dedicated to a given device (PCF policy subscription data stored in UDR for the 5G RG subscription may contain a device category that is to be handled in a similar way as Subscriber category already defined in 23.503)  .  For a "Combo Ethernet + IP" PCC rules (sent to the SMF) need only to refer to the IP header. No PCF policy differentiation is defined that would use the VLAN header received by the UPF over a PDU Session.

-	PCF binding using BSF is based on the IP address of the devices. Service differentiation upon application (AF) request is also possible using the same mechanisms (e.g. Npcf_authorization service) as in the case of an IP PDU Session type: the AF provides requests identifying the IP flow of a terminal it wants to act upon, session binding with the IP address of the terminal applies as described in clause 6.1.1.2 of TS 23.503 [8] and the PCF provides corresponding updated PCC rule. The BSF need not to be aware of "access side layer 2 header".

-	There is no authentication of the devices behind the RG that are identified by their MAC address.

The following figure shows the message flow for a PDU Session establishment (copied from TS 23.502 [3] Figure 4.3.2.2.1-1: "UE-requested PDU Session Establishment for non-roaming and roaming with local breakout"). The changes to existing procedure introduced for the support of the solution are described in the text below the figure while otherwise the PDU Session Establishment proceeds as described in existing specifications.

Figure 6.8.2-1: impacts of Combo IP + Ethernet PDU Session Type to 23.502 [3] Figure 4.3.2.2.1-1: UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

Step 1:	The 5G RG or in case of FN RG the AGF may be configured (e.g. URSP) to request a PDU Session of the Combo IP + Ethernet PDU Session Type. Other parameters of the PDU Session may be the same than in IP PDU Session Type (E.g. the PDU Session establishment request may contain PCO asking for a P-CSCF address, ACS address, etc.).

Step 4:	The subscription data retrieved from the UDM may contain Combo IP + Ethernet PDU Session Type as allowed PDU Session Type for a subscription.

Step 6 is optional (as in R17 TS 23.502)

Step 7.	The SMF may indicate to the PCF that the PDU Session is of Combo IP + Ethernet PDU Session Type.
Per PDU Session policies sent by the PCF may contain a control on the Maximum Number of IP address to allocate as part of the PDU Session to devices in the customer premises (MNIP).

Step 10a.	The N4 session between the SMF and an UPF acting as PSA for the PDU Session may be associated by the SMF with a Combo IP + Ethernet PDU Session Type; the SMF controls the UPF to report MAC addresses detected in UL traffic of the PDU Session.
If the SMF requests the UPF to forward DHCP/RS (Router Solicitation) traffic from devices using the PDU Session, the SMF may request the UPF to report the "access side layer 2 header" on which the DHCP/RS has been received.

Access to ACS (Autoconfiguration server used in BBF TR 069 procedures) is supported as for IP or Ethernet PDU Session types (e.g. as described in 23.316).

Figure 6.8.2-2: Allocation of an IP address to a device behind the RG

For Combo PDU Session Type, the SMF requests the UPF to forward DHCP / RS signalling over N4; the reporting from UPF shall contain the MAC address of the device.

When during the life time of a PDU Session, an IP address is allocated to a device supported by a PDU Session established with the Combo IP + Ethernet PDU Session Type, the following applies:

1)	In band IP address allocation procedure (RS/RA, DHCPv4, DHCPv6). The SMF / UPF uses policies received in step 7 of Figure 6.8.2-1 (IP index, max number of IP addresses for the PDU Session) to determine whether to grant an IP address to the device and if yes to determine the IP pool / allocation method to use.

NOTE :	The usage of DHCPV4 signalling to allocate IP address to devices behind the RG does not require sending dedicated NAS signalling with a request of deferred IP address allocation. ; it is possible that the IP address allocated to the RG itself is provided via NAS to the RG while in band IP address allocation applies for the devices behind the RG.

2)	if this corresponds to a PCRT (Policy Control Request Trigger) received previously from the PCF, the SMF initiates the SM Policy Association Modification procedure described in TS 23.502 [3] clause 4.16.5.1.

The reception of new policies from the PCF in step 2 may trigger the network initiated PDU Session modification procedure per TS 23.502 [3] clause 4.3.3 which may include a corresponding N4 session modification procedure to create or update the corresponding PDR/FAR/QER/URR on the UPF and SMF sending updates of QOS rules to the 5G-RG to apply  new QoS flows

### 6.8.3	Impacts on existing functions

The impact on existing network function is the following:

-	SMF / UPF(N4), UDM and PCF (Npcf) need to support a new PDU Session type.

-	Support of Multiple IP address for a PDU session. The SMF may allocate multiple IPv4 addresses  in one PDU session of "combo IP + Ethernet" PDU Session type

-	When the N4 session is associated by the SMF with a Combo IP + Ethernet PDU Session Type, the UPF enforces the behaviour in bullet 4 of clause 6.8.1.

-	Npcf Policy Request Control Trigger mechanism is upgraded for the SMF to be able to report the allocation of an IP address to a device behind the RG together with the MAC address of that device.

-	the PCF may apply policies (possibly related with IP level traffic) related with the MAC address of a device (as reported by the SMF via a Policy Request Control Trigger). PCF policy subscription data stored in UDR for the 5G RG subscription may contain a device category that is to be handled in a similar way as Subscriber category already defined in 23.503.

-	The CHF and LI may receive an indication of a new IP address having been allocated to a PDU Session together with the indication of the corresponding MAC Address.

-	5G-RG: Support the new "combo IP + Ethernet" PDU Session type value in NAS signaling

### 6.9 	Solution 9: 5GC-capable UE behind 5G-RG using untrusted Non-3GPP access

#### 6.9.1	General

The solution is based on the UE behind RG using untrusted Non-3GPP access architecture as defined in TS 23.316 Annex A. The 5G-RG and 5GC-capable UE may belong to the same PLMN or the different PLMN.

Figure 6.9.1: Non-roaming architecture for UE behind 5G-RG using untrusted N3GPP access

#### 6.9.2	Registration procedure

The UE register to the 5GC based on the procedure as defined in TS 23.502 subclaus 4.12.2 with the following difference.

- Before the Step 1a, the UE obtains the 5G-RG GUTI from ANQP message, as defined in subclause 6.5.2 step 0.

- Step 5: the UE sends the Registration Request message to the UE’s AMF, in which the 5G-RG GUTI is included in this message as defined in subclause 6.5.2 step 1e.

- After the step 9d, the UE’s AMF identify the UE role by using the same solution as defined in subclause 6.5.2 from step 5 to step 9.

During the PDU session establishment procedure of the UE, the same procedure defined in subclause 6.5.2 is applied.

### 6.9.3	Impacts on existing Functions

UDM, AMF, PCF, SMF, UE:

-   Same as described in the subclause 6.5.3.

5G-RG:

-   The 5G-RG sends the 5G-RG GUTI to the UE via ANQP message.

## 6.10	Solution 10: Registration via Trusted Non-3GPP Access with TNGF Relocation

### 6.10.1	Description

The solution in this clause defines extensions to the "5GC registration via trusted non-3GPP access" procedure, which enable the AMF to select a different TNGF to serve the UE, when the originally selected TNGF cannot support the slices needed by the UE.

The solution follows the principles agreed for TEI17_N3SLICE, i.e., that each TNGF may be locally configured with one TAI value and a corresponding set of S-NSSAIs.

The key steps of the solution are summarized as follows:

a)	The UE selects a PLMN and a trusted non-3GPP access that supports 5G connectivity with this PLMN, as specified in clause 6.3.12 of TS 23.501 [2].

b)	An EAP procedure is initiated between the UE and the selected trusted non-3GPP access. The realm provided by the UE in NAI ("<any_username>@nai.5gc.mnc<MNC>.mcc<MCC>.3gppnetwork.org") is used to select a TNGF that supports 5G connectivity with the selected PLMN. This TNGF is selected without considering any slice information.

c)	The UE sends a Registration Request message via the selected TNGF, as specified in clause 4.12a.2.2 of TS 23.502 [3]. The selected TNGF selects an AMF and forwards to this AMF the Registration Request message. If the Requested NSSAI is received by the selected TNGF (according to the conditions defined in clause 5.15.9 of TS 23.501 [2]), then the TNGF selects an AMF by considering also the Requested NSSAI (as defined in clause 6.3.5 of TS 23.501 [2]).

d)	The AMF (either the one selected by TNGF, or the one selected after AMF re-allocation, as defined in clause 5.15.5.2.3 of TS 23.501 [2]) identifies whether the selected TNGF can or cannot support the slices requested by the UE. The AMF knows the slices supported by each TNGF connected with the AMF because (as defined in TS 38.413 [7]), the N2 Setup Request message sent from TNGF to AMF includes the supported TAI and the supported S-NSSAIs for this TAI.

If the selected TNGF cannot support the slices requested by the UE, and if there is another TNGF that can serve this UE and can support the slices requested by the UE, then the AMF triggers TNGF relocation, i.e., requests to substitute the selected TNGF with a new TNGF. This TNGF relocation does not interrupt the ongoing registration procedure. The registration procedure continues but the UE is instructed to establish a NWt connection with the new TNGF.

The AMF can decide to trigger TNGF relocation based on its own criteria. Typically, the AMF considers the Requested NSSAI provided by the UE, the Subscribed NSSAI and the S-NSSAIs supported by all TNGFs that can serve the UE.

Editor's note:	Many TNGFs can be connected to the AMF. How the AMF can determine which of these TNGFs can serve the UE is FFS.

Editor's note:	When the NSSF is used for selecting the serving AMF, it is FFS whether and how the NSSF has knowledge of the TAI supported by TNGF.

e)	As a result, the 5G registration procedure is completed and a new TNGF is selected by AMF, which can support all (or the most) S-NSSAIs requested by the UE.

More details can be found in the next clause. No changes are made to the slice selection procedure and to the AMF selection for serving AMF.

### 6.10.2	Procedures

#### 6.10.2.1	Registration via trusted non-3GPP access with TNGF relocation

The procedure specified in this clause extends the registration procedure for trusted non-3GPP access specified in clause 4.12a.2.2 of TS 23.502 [3], , in order to support TNGF relocation. During the registration via trusted non-3GPP access, the AMF may trigger TNGF relocation as defined in the previous clause.

Figure 6.10.2.1-1: Registration via trusted non-3GPP access with TNGF relocation

The steps in Figure 6.10.2.1-1 are the same steps as in TS 23.502 [3], Figure 4.12a.2.2-1 (registration via trusted non-3GPP access) with the following additions and clarifications:

3.	The TNAP selects a TNGF, referred to as TNGF-1. The TNGF-1 is selected by using the NAI provided by the UE and, optionally, the non-3GPP access network identity (e.g. SSID) requested by the UE.

6b.	If the AMF determines that the Registration Request should be served by another AMF (e.g., as defined in TS 23.501 [2], clause 5.15.5.2), the AMF initiates AMF re-allocation and forwards the Registration Request to another AMF (see TS 23.501 [2], clause 5.15.5.2.3).

10a.	The AMF decides to trigger TNGF relocation and use TNGF-2 instead of TNGF-1. For taking this decision, the AMF may consider the Requested NSSAI provided by the UE, the Subscribed NSSAI for this UE, the S-NSSAIs supported by other TNGFs connected to AMFs, and other information. The AMF knows the slices supported by each TNGF connected with the AMF because (as defined in TS 38.413 [7]), the N2 Setup Request message sent from TNGF to AMF includes the supported TAI and the supported S-NSSAIs for this TAI.

If the AMF decides to trigger TNGF relocation, then the AMF shall include in the N2 Initial Ctx Setup Request the TNGF-2 IP address, indicating that the AMF wants to relocate the UE to TNGF-2. When TNGF-1 receives message 10a, it recognizes that the TNGF Address contains an IP address of another TNGF, hence, it recognizes that the AMF requests TNGF relocation.

Editor's note:	It is FFS whether N2 Initial Ctx Setup Request or a N2 handover related message is used for this purpose.

Editor's note:	Instead of sending an N2 Initial Ctx Setup Request in step 10a, the AMF could send a new N2 message used for TNGF relocation, e.g., a TNGF Relocation Command. In this case, the TNGF-1 would respond with a TNGF Relocation Response in step A1. It is FFS whether such a new N2 message is preferable.

10b.	The TNGF Address sent to UE is the TNGF-2 IP address provided by AMF. After receiving this message, the UE does not switch to a different SSID, if the non-3GPP access network supports SSIDs (such as WLAN access networks). It is assumed that the same SSID can support multiple S-NSSAIs.

Editor's note:	Whether there is need for the UE to switch to a different SSID after receiving step 10b is FFS.

A1.	After receiving an EAP-Response/5G-Notification packet from the UE in step 10c, the TNGF-1 shall send message 10d containing the EAP-Success packet. In addition, TNGF-1 shall respond to AMF with an N2 Initial Context Setup Failure message (see TS 38.413 [7]) with a Cause value indicating that the failure is because of the requested TNGF relocation. If needed, this message may also contain some information that shall be transparently forwarded to TNGF-2 (via the AMF).

A2.	The N2 Initial Context Setup Failure message triggers the AMF to send an N2 Initial Context Setup Request to TNGF-2 including the TNGF key, the UE identity and, if received in step A1, the information that shall be transparently forwarded to TNGF-2.

13.	The UE sets up a secure NWt connection using the TNGF address received in step 10b, i.e., with TNGF-2. All further communication between the UE and the AMF takes place via TNGF-2.

NOTE:	In the above registration procedure, the EAP-5G protocol runs between the UE and TNGF-1, while the NWt connection is established between the UE and TNGF-2.

### 6.10.3	Impacts on Existing Nodes and Functionality

UE:

-	No impact.

TNGF-1 (used for EAP-5G):

-	Shall be able to recognize when the AMF requests relocation (by inspecting the TNGF Address in step 10a) and shall respond to AMF with N2 Initial Context Setup Failure after receiving an EAP-Response/5G-Notification packet from the UE (which indicates that the UE has received the IP address of TNGF-2).

TNGF-2 (used for NWt connection establishment):

-	Shall be able to receive N2 Initial Context Setup Request from an AMF without having an EAP-5G session with a UE and shall be able to establish an NWt connection with the UE by using the UE identity and the TNGF key included in the N2 Initial Context Setup Request.

AMF:

-	Shall be able to trigger TNGF relocation, e.g., based on the Requested NSSAI provided by the UE, the Subscribed NSSAI for this UE, the S-NSSAIs supported by all TNGFs that can serve the UE, and other information. The AMF shall include in the N2 Initial Ctx Setup Request (step 10a) the IP address of the new TNGF.

-	The AMF triggers the establishment of an N2 association for the UE via N2 Initial Context Setup Request. After receiving the N2 Initial Context Setup Failure from TNGF-1 with a cause value indicating TNGF relocation, the AMF shall send and N2 Initial Context Setup Request to TNGF-2 including the TNGF key and the UE identity. This N2 Initial Context Setup Request establishes the N2 association for the UE.

## 6.11	Solution 11: Registration via Untrusted Non-3GPP Access with N3IWF Relocation

### 6.11.1	Description

The solution in this clause defines extensions to the "5GC registration via untrusted non-3GPP access" procedure, which enable the AMF to select a different N3IWF to serve the UE, when the originally selected N3IWF cannot support the slices needed by the UE.

The solution follows the principles agreed for TEI17_N3SLICE, i.e., that each N3IWF may be locally configured with one TAI value and a corresponding set of S-NSSAIs.

The key steps of the solution are summarized as follows:

a)	The UE selects a PLMN and an N3IWF in this PLMN as specified in clause 6.3.6.2 of TS 23.501 [2], "Stand-alone N3IWF selection". This N3IWF is selected without considering any slice information.

b)	The UE initiates an IKE_AUTH exchange with the selected N3IWF and sends a Registration Request message via this N3IWF, as specified in clause 4.12.2.2 of TS 23.502 [3]. The selected N3IWF selects an AMF by using information provided by the UE, including the Requested NSSAI, and forwards to this AMF the Registration Request message.

c)	The AMF (either the one selected by N3IWF, or the one selected after AMF re-allocation, as defined in clause 5.15.5.2.3 of TS 23.501 [2]) identifies whether the selected N3IWF can or cannot support the slices requested by the UE. The AMF knows the slices supported by each N3IWF connected with the AMF because (as defined in TS 38.413 [7]) the N2 Setup Request message sent from N3IWF to AMF includes the supported TAI and the supported S-NSSAIs for this TAI.

If the selected N3IWF cannot support the slices requested by the UE, and if there is another N3IWF that can serve this UE and can support the slices requested by the UE, then the AMF triggers N3IWF relocation, i.e. requests to substitute the selected N3IWF with a new N3IWF. This N3IWF relocation does not interrupt the ongoing registration procedure. The registration procedure continues but the UE is instructed to establish a NWu connection with the new N3IWF.

The AMF can decide to trigger N3IWF relocation based on its own criteria. Typically, the AMF considers the Requested NSSAI provided by the UE, the Subscribed NSSAI and the S-NSSAIs supported by all N3IWFs connected to this AMF.

Editor's note:	When the NSSF is used for selecting the serving AMF, it is FFS whether and how the NSSF has knowledge of the TAI supported by N3IWF.

d)	As a result, the 5G registration procedure is completed and a new N3IWF is selected by AMF, which can support all (or the most) S-NSSAIs requested by the UE.

More details can be found in the next clause. No changes are made to the slice selection procedure and to the AMF selection for serving AMF.

### 6.11.2	Procedures

#### 6.11.2.1	Registration via untrusted non-3GPP access with N3IWF relocation

The procedure specified in this clause extends the registration procedure for untrusted non-3GPP access specified in clause 4.12.2.2 of TS 23.502 [3], , in order to support N3IWF relocation. During the registration via untrusted non-3GPP access, the AMF may trigger N3IWF relocation as defined in the previous clause.

Fig. 6.11.2.1-1: Registration via untrusted non-3GPP access with N3IWF relocation

1-7.	Same as steps 1-8 in TS 23.502 [3], figure 4.12.2.2-1 (Registration via untrusted non-3GPP access) with the following clarification:

-	If the AMF determines that the Registration Request received in step 6b should be served by another AMF (e.g. as defined in clause 5.15.5.2 of TS 23.501 [2]), the AMF initiates AMF re-allocation and forwards the Registration Request to another AMF (see clause 5.15.5.2.3 of TS 23.501 [2]).

9a.	The AMF decides to trigger N3IWF relocation and use N3IWF-2 instead of N3IWF-1. For taking this decision, the AMF may consider the Requested NSSAI provided by the UE, the Subscribed NSSAI for this UE, the S-NSSAIs supported by all N3IWFs connected to AMF, and other information. The AMF knows the slices supported by each N3IWF connected with AMF because (as defined in TS 38.413 [7]) the N2 Setup Request message sent from N3IWF to AMF includes the supported TAI and the supported S-NSSAIs for this TAI.

The AMF sends a N3IWF Relocation Command to N3IWF-1 that contains a Security Mode Control (SMC) Request message, in order to establish a NAS security context for this UE and protect further NAS messages. In this message, the AMF also includes the address of N3IWF-2 and an AMF identity, e.g., a Globally Unique AMF Identifier (GUAMI) or an IP address of AMF. The N3IWF-1 forwards to UE the received SMC Request message, the N3IWF-2 address and the AMF identity, inside an EAP 5G-NAS packet.

Editor's note:	It is FFS whether the AMF sends a N3IWF Relocation Command or an existing NGAP handover related message is used for this purpose.

10.	Since the UE receives an N3IWF address in step 9b, the UE determines that it should select another N3IWF. Therefore, the UE sends an EAP 5G-Stop packet, which (as specified in TS 24.502 [8]) triggers the N3IWF-1 to terminate the ongoing IKE procedure by sending an IKE_INFORMATIONAL request message containing an EAP-Failure and an appropriated error cause.

After this step, the N3IWF-1 could release the N2 connection with the AMF. However, since the release of the N2 connection may affect the ongoing 5G registration procedure, the N3IWF-1 may delay the release of the N2 connection with the AMF and wait for AMF to release the N2 connection.

11-12.	The UE starts the establishment of a new NWu connection with the N3IWF address received in step 9b (i.e., N3IWF-2).

13.	The UE sends an IKE_AUTH request to N3IWF-2, which includes an EAP-Response/5G-NAS packet that contains the AN-Params and a SMC Complete message, which is a response to the SMC Request message received in step 9b. The AN-Params contains a UE identity (e.g., SUCI or 5G-GUTI), an Establishment cause, (optionally) a Requested NSSAI, and the AMF identity received in step 9b. The presence of the AMF identity in this message indicates to N3IWF-2 that this message is sent due to N3IWF relocation. Alternatively, the Establishment cause may contain a value that indicates to N3IWF-2 that this message is sent due to N3IWF relocation.

NOTE:	Although the UE reconnected to a new N3IWF, the NAS registration procedure between the UE and the AMF is resumed via the new N3IWF. The registration procedure is not interrupted due to the N3IWF relocation.

14.	The N3IWF-2 selects the same AMF based on the received AMF identity and forwards the SMC Complete message to the AMF inside a N3IWF Relocation Notify. This message also contains the UE identity so that the AMF can associate the received SMC Complete message with the appropriate UE context and resume the ongoing registration procedure for this UE. The N3IWF Relocation Notify message creates a new N2 connection associated with the UE. Note that the N3IWF-2 decides to send a N3IWF Relocation Notify message to AMF (and not an Initial UE Message) because it determines that the message in step 13 is sent to due to N3IWF relocation.

After the AMF receives the N3IWF Relocation Notify from N3IWF-2, the AMF may have two different N2 connections associated with the same UE: one with N3IWF-1 setup in step 6b and another with N3IWF-2 setup in step 14b. Therefore, the AMF is expected to release the N2 connection with N3IWF-1, which is not required anymore. The messages exchanged for releasing this N2 connection are not shown in the above figure.

Editor's note:	Whether the UE Id is needed in message 14b is FFS. It should be studied whether the SMC Complete message is enough for AMF to associate message 14b with an existing UE context.

15-21.	Same as steps 10-13 in TS 23.502 [3], figure 4.12.2.2-1 (Registration via untrusted non-3GPP access).

After the above signalling flow the UE registration to 5GC is completed and the initially selected N3IWF-1 is relocated to N3IWF-2, which supports the S-NSSAI allowed for the UE.

### 6.11.3	Impacts on Existing Nodes and Functionality

Editor's note:	The impacts on existing nodes will be provided in a future update of this solution.

## 6.12	Solution 12: slice related TNGF selection for WLAN access

### 6.12.1	Description

The solution addresses slice related TNGF selection for WLAN access. Other types of non-3GPP access are not covered by this solution.

Current (Rel-17) clause 6.3.12 of TS 23.501 [2] specifies that a non-3GPP access network may advertise "3GPP Cellular Network" information, by using the ANQP protocol, defined in the HS2.0 specification [6]. A non-3GPP access network (WLAN) may via ANQP "3GPP Cellular Network" information advertise (per Current (Rel-17) clause 6.3.12 of TS 23.501 [2]):

-	A PLMN List-3, which includes PLMNs with which "5G connectivity" is supported. A non-3GPP access network supports "5G connectivity" with a PLMN when it deploys a TNGF function that can connect with an AMF function and an UPF function in this PLMN via N2 and N3 interfaces, respectively.

The solution proposes to extend this capability in order for the UE to be able to select a SSID mapping at best to the NSSAI the UE wishes to register to over TNGF access. The solution assumes all TNGF that may be reached behind a SSID and TNAP support the same set of S-NSSAI(s).

The ANQP "3GPP Cellular Network" information when indicating "5G connectivity" may also advertise:

a)	the list of S-NSSAI(s) that the TNGF(s) reachable by the corresponding SSID on the TNAP support; or

b)	the list of Network Slice Access Stratum Groups (NSASGs) that the TNGF(s) reachable by the corresponding SSID on the TNAP support.

Editor's note: Whether the ANQP "3GPP Cellular Network" information contains a list of S-NSSAIs or a list of NSASGs will be decided at the TR conclusion phase.

Editor's note: It is FFS how the UE is configured with a mapping from S-NSSAIs to NSASGs that apply for TNGF selection (the NSASG configuration for NG-RAN may not fit to TNGF selection).

### 6.12.2	Procedures

The UE runs the TNGF selection process described in clause 6.3.12.2 of TS 23.501 [2] up to step 3.

Step 4 of clause 6.3.12.2 of TS 23.501 [2] is modified as follows to take into account the NSSAI the UE wishes to register to over TNGF access, as well as the ANQP "3GPP Cellular Network" information advertisement of the list of S-NSSAI(s) or the list of Network Slice Access Stratum Groups (NSASGs) that the TNGF(s) reachable by the corresponding SSID may support.

NOTE:	The list of S-NSSAI(s) that the TNAP advertises over ANQP for a SSID is a subset of the list of S-NSSAI(s) associated with the TAI that the TNGF has provided to the AMF in the N2 Setup Request message (TS 38.413 [7]).

Step 4:	The UE selects a non-3GPP access network to connect to, as follows (only step a is modified):

a.	The UE puts the available non-3GPP access networks in priority order. For WLAN access, the UE constructs a prioritized list of WLAN access networks by using the WLANSP rules (if provided) and the procedure specified in clause 6.6.1.3 of TS 23.503 [4], as well as by using the Requested NSSAI of the UE and the list of S-NSSAI(s) or the list of NSASGs advertised with the ANQP "3GPP Cellular Network" information.

Editor's note: The relative priority between the WLANSP rules and the slice-based selection is FFS.

b.	From the prioritized list of non-3GPP access networks, the UE selects the highest priority non-3GPP access network that supports the selected type of trusted connectivity to the selected PLMN.

d.	Over the selected non-3GPP access network, the UE starts the 5GC registration procedure specified in clause 4.12a.2.2 of TS 23.502 [3].

If the UE registers with a requested NSSAI equal to the Default Configured NSSAI (first registration in the serving PLMN), it may happen that the UE receives a Configured-NSSAI for the serving PLMN and an allowed NSSAI that contains more S-NSSAI(s) where there are more S-NSSAI(s) in the Configured-NSSAI for the serving PLMN than in the Allowed NSSAI. In that case the UE can retry TNGF selection in that PLMN using the Configured NSSAI for the Serving PLMN as Requested NSSAI.

### 6.12.3	Impacts on Existing Nodes and Functionality

Editor's note: This clause captures impacts on existing 3GPP nodes and functional elements.

-	TNAP is to advertise per SSID the list of S-NSSAI(s) or the list of Network Slice Access Stratum Groups (NSASGs) that the TNGF(s) reachable by the corresponding SSID support, via ANQP "3GPP Cellular Network" information that is also indicating "5G connectivity".

-	UE is to leverage this information to determine the priority order of available non-3GPP access networks per a modified clause 6.3.12 of TS 23.501 [2] process..

## 6.13	Solution 13: UE getting the slices supported by N3IWF (s)

### 6.13.1	Description

The solution assumes that the UE(s) can themselves select the N3IWF that supports the set of slices they wish to use. As part of the N3IWF selection procedure defined in 23.501 clause 6.3.6, when a UE has selected a PLMN and has a set of candidate N3IWF(s), the UE may issue a Slice Support Get request to each of these N3IWF. Upon receiving a Slice Support Get request, a N3IWF answers with the list of slices this N3IWF supports. Then, the UE takes into account the set of slices it wishes to use and the slices supported by the candidate N3IWF as indicated in Slice Support Get responses to select a N3IWF that best supports the slices the UE wishes to use.

- The set of slices the UE wishes to use maps to Requested NSSAI if the N3IWF selection is part of a UE registration request over N3IWF, and maps to Allowed NSSAI if the N3IWF selection is part of a UE service request over N3IWF

### 6.13.2	Procedures

Figure 6-1: 	Baseline N3IWF discovery using slice support get

- UE discovery of candidate N3IWF per 3GPP R17 23.501 clause 6.3.6 including PLMN selection (possibly taking into account the need to select a N3IWF in the local country) and including choice between N3IWF and epdg and up to the point where the UE has received from DNS the IP addresses of candidate N3IWF; Using S-NAPTR DNS procedure the UE may discover which N3IWF supports the Slice Support Get request procedure, based on a dedicated S-NAPTR "Service Parameter" (equivalent to "x-3gpp-sgw:x-s5-gtp” defined in 3GPP 29.303)

- of

2.and 4.	The UE sends a Slice Support Get request to each N3IWF.

A Slice Support Get request may optionally contain the set of slices (S-NSSAI) the UE wishes to use (indicating to N3IWF that there is no need to answer with slices outside this set).

3. And 5.	The N3IWF sends a Slice Support Get response that contains the set of slices (S-NSSAI) the N3IWF supports. This set of slices (S-NSSAI) is restricted to contain only a subset of the set of slices (S-NSSAI) the UE has indicated in the Slice Support Get request. It may also contain a caching information telling the Ue how long the UE should consider this information valid for

Editor's note: How many N3IWFs the UE may need to query as part of this is FFS.

Editor's note: Whether the UE needs to perform the discovery each time it needs to connect to an N3IWF is FFS. If not, how to handle the case that the UE has outdated information about supported slices by the different N3IWFS is FFS.

This procedure is stateless: the N3IWF does not need to store any information related with a Slice Support Get request.

The Slice Support Get service may be designed as a new (HTTPs) API exposed by the N3IWF to the UE and defined by 3GPP.

TLS connections may be used between the UE and the N3IWF from step 2 to 5. The Slice Support Get service is not meant to change the state of the N3IWF (as the UE just gets information). The N3IWF is assumed to provide a server certificate as it does over IKE within R17 specifications.

Security aspects for this new HTTP API are to be specified by SA3.

6.	based on the information received from N3IWF in slice support get response, the UE selects the N3IWF it will contact to access to the 5GC.

7. The UE connects to 5GC over the selected N3IWF, here N3IWF1, per R17 specifications

NOTE:	 The procedure requires to run, as per R17 specifications, only once the IKE_SA_Init procedure which requires heavy computation

NOTE:	 selection of a N3IWF also based on tracking area is possible as the UE contacts N3IWF based on currently configured information.

If within the set of candidate N3IWF(s) there are N3IWF(s) that support the Slice Support Get procedure and other that don’t, the UE selects the N3IWF to use in the following descending order of priority:

- N3IWF(s) that support the Slice Support Get procedure and that have indicated they support the set of slices (S-NSSAI) the UE wishes to use

- N3IWF(s) that don’t support the Slice Support Get procedure

- N3IWF(s) that support the Slice Support Get procedure and that have indicated they don’t support some set of slices (S-NSSAI) the UE wishes to use.

It is left implementation specific how UE considers N3IWF(s) that support the Slice Support Get procedure and that have indicated they support only a subset of the slices (S-NSSAI) the UE wishes to use.

### 6.13.3	Impacts on Existing Nodes and Functionality

This new solution impacts

- the UE:

- discover which N3IWF supports the Slice Support Get request procedure

- support new Slice Support Get procedure

- N3IFW selection based on additional information received from N3IWF,

- the N3IWF

- : support new Slice Support Get procedure)

## 6.14	Solution Y: use of AN NRF

### 6.14.1	Description

The UE performs a N3IWF discovery procedure to a new “AN NRF” (similar to 5GC NRF defined in TS 23.501), to which N3IWFs register the set of Slices (S-NSSAI(s)) they support (reusing Nnrf_NFManagement_NFRegister service operation as defined in TS 23.502 or a new similar service operation supported by a new NF) .

The AN NRF is for security reason assumed to be a different NF hosted by a different platform than the 5GC NRF;

UE(s) only needs to perform ONE "AN NRF" query (reusing Nnrf_NFDiscovery_NFDiscover service operation as defined in TS 23.502), not as many queries as there are N3IWFs.

UEs discover the “AN NRF” node via a DNS query as the AN NRF has a well known FQDN (to be defined in TS 23.003).

NOTE: As an example the well known FQDN pattern could be something like

“an-nrf.5gc.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org" where the <MNC> and <MCC> identify the PLMN where the N3IWF is to be located

### 6.14.2	Procedures

The procedure of Figure 6.14.2-1 and/or Figure 6.14.2-2 are run independently of any UE attempt to get N3IWF access. The N3IWF information newly configured on the AN NRF may contain one or multiple S-NSSAI(s) (with possible wildcarding) and one or multiple N3IWF public IP addresses for use by UEs issuing connecting to the N3IWF (Nwu N3IWF IP address).

Figure 6.14.2-1: N3IWF registering onto AN NRF

Figure 6.14.2-2: N3IWF updating an existing registration onto AN NRF

Figure 6.14.2-4: UE discovering suitable N3IWF using AN NRF

The AN NRF may take into account both the list of target S-NSSAI requested by the UE and the UE IP address (source address of the AN NRF request) to determine a best matching N3IWF that serves the list of target S-NSSAI requested by the UE  and whose Nwu N3IWF IP address is IP close to the UE IP address. The AN NRF may also take into account 3GPP TA information provided by the UE

The Discovery of the AN NRF is not meant to change the state of the AN NRF (as the UE just gets information). The AN NRF is assumed to provide a server certificate.

Security aspects for usage of AN NRF are to be specified by SA3.

### 6.14.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

- Addition of a new AN NRF

- N3IWF register onto the AN NRF

- UE issue a discovery request to AN NRF

## 6.15	Solution #15: Selecting N3IWF supporting the S-NSSAI needed by UE

### 6.15.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub) clause(s) may be added to capture details.

This solution aims at addressing Key Issue #2 about how to select a TNGF/N3IWF that supports the S-NSSAI(s) needed by the UE. In particular, this solution mainly focuses on how to select a N3IWF that supports the S-NSSAI(s) needed by the UE.

The solution provides two methods to select a N3IWF that supports the S-NSSAI(s) needed by the UE. One option is to enhance the N3IWF identifier configuration or the non-3GPP access node selection information that includes the supported S-NSSAI(s) per N3IWF. Access Network Discovery & Selection policy (ANDSP) was designed to be provided to UE by the network for selection of N3IWF. Adding the supported S-NSSAI(s) for the N3IWF in ANDSP allows UE to determine an appropriate N3IWF that supports the S-NSSAI(s) needed by the UE during N3IWF selection procedure. Another option is that when UE connects with a N3IWF that does not support the S-NSSAI(s) needed by the UE, AMF can determine the target N3IWF which supports the S-NSSAI(s) needed by the UE and provides the target N3IWF information to UE via Registration Accept/Reject message so that UE can use the target N3IWF information to connect with the target N3IWF in order to meet the requirement of the UE. The second option can be applied when the UE has not been updated with the enhanced N3IWF identifier configuration or non-3GPP access node selection information.

### 6.15.2 N3IWF selection solution

The UE can be configured by the HPLMN with N3IWF identifier configuration which contains the FQDN or IP address of the N3IWF in the HPLMN. As described in current (Rel-17) clause 5.3.3.3 of 3GPP TS 24.526 [10], the content of home N3IWF identifier configuration contains a list of home N3IWF identifier entries. The HPLMN can configure a UE with N3IWF identifier configuration so that the list of N3IWFs in N3IWF identifier configuration cover all the UE's subscribed S-NSSAI(s).

Editor's note: How to enable selection of an N3IWF that supports the slices that the UE intends to access while also considering the tracking area (e.g. if the HPLMN or VPLMN prefers the UE to use the Tracking Area FQDN format) is FFS.

Editor's note: It is FFS on how to select a N3IWF supporting the needed S-NSSAI(s)in VPLMN when roaming.

As defined in TS 23.503 subclause 6.6.1, the 5GC provides the Access Network Discovery & Selection Policy (ANDSP) to the UE, in which the N3IWF selection information is included for the UE to select the N3IWF in the HPLMN. In order to support the slice information applied in the N3IWF selection procedure, the ANDSP is extended as follows:

- N3IWF identifier configuration: It contains the FQDN or IP address of the N3IWF in the HPLMN and the S-NSSAIs slice information supported by this N3IWF. This S-NSSAIs slice information is also included in the UE’s Subscribed S-NSSAIs for non-3GPP access defined in the UE subscription data.

Editor's note: It is FFS how the PCF obtains the UE’s Subscribed S-NSSAIs information, e.g. from AMF or from the OSS system.

If the UE does not receive the N3IWF identifier configuration from ANDSP, or the UE cannot select the N3IWF based on the N3IWF identifier configuration, e.g. in the roaming case, the N3IWF identifier configuration information configured in the UE can be applied as the default N3IWF selection policy.

Editor's note: It is FFS on how the N3IWF selection procedure is impacted with the extension of the ANDSP.

### 6.15.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

Figure 6.15.3-1: Registration via untrusted non-3GPP access

1.	The UE connects to an untrusted non-3GPP Access Network with any appropriate authentication procedure and it is assigned an IP address. For example, a non-3GPP authentication method can be used, e.g. no authentication (in the case of a free WLAN), EAP with pre-shared key, username/password, etc. When the UE decides to attach to 5GC network, the UE selects an N3IWF in a 5G PLMN as described in 6.15.1.

2.	The UE proceeds with the establishment of an IPsec Security Association (SA) with the selected N3IWF by initiating an IKE initial exchange according to RFC 7296 [3]. After step 2, all subsequent IKE messages are encrypted and integrity protected by using the IKE SA established in this step.

3.	The UE shall initiate an IKE_AUTH exchange by sending an IKE_AUTH request message. The AUTH payload is not included in the IKE_AUTH request message, which indicates that the IKE_AUTH exchange shall use EAP signalling (in this case EAP-5G signalling). If the UE supports MOBIKE, it shall include a Notify payload in the IKE_AUTH request, as specified in RFC 4555 [40], indicating that MOBIKE is supported. In addition, as specified in TS 33.501 [15], if the UE is provisioned with the N3IWF root certificate, it shall include the CERTREQ payload within the IKE_AUTH request message to request the N3IWF's certificate.

4.	The N3IWF responds with an IKE_AUTH response message, which includes an EAP-Request/5G-Start packet. The EAP-Request/5G-Start packet informs the UE to initiate an EAP-5G session, i.e. to start sending NAS messages encapsulated within EAP-5G packets. If the N3IWF has received a CERTREQ payload from the UE, the N3IWF shall include the CERT payload in the IKE_AUTH response message containing the N3IWF's certificate. How the UE uses the N3IWF's certificate is specified in TS 33.501 [15].

5.	The UE shall send an IKE_AUTH request, which includes an EAP-Response/5G-NAS packet that contains the Access Network parameters (AN parameters) and a Registration Request message. The AN parameters contain information that is used by the N3IWF for selecting an AMF in the 5G core network. This information includes e.g. the GUAMI, the Selected PLMN ID (or PLMN ID and NID, see clause 5.30 of TS 23.501 [2]), the Requested NSSAI and the Establishment cause. The Establishment cause provides the reason for requesting a signalling connection with 5GC. Whether and how the UE includes the Requested NSSAI as part of the AN parameters is dependent on the value of the Access Stratum Connection Establishment NSSAI Inclusion Mode parameter, as specified in clause 5.15.9 of TS 23.501 [2]. The Registration Request message may optionally contain an indication indicating that the N3IWF shall support the requested NSSAI.

NOTE 1:	The N3IWF does not send an EAP-Identity request because the UE includes its identity in the first IKE_AUTH. This is in line with RFC 7296 [3] clause 3.16.

6.	The N3IWF shall select an AMF based on the received AN parameters and local policy, as specified in clause 6.3.5 of TS 23.501 [2]. The N3IWF shall then forward the Registration Request received from the UE to the selected AMF within an N2 message. This message contains N2 parameters that include the Selected PLMN ID and the Establishment cause.

7.	The selected AMF determines whether the N3IWF connected with UE currently (named S-N3IWF) support the request NSSAI needed by UE if the indication of supporting the requested NSSAI is included in the Registration Request. If the selected N3IWF does not support the request NSSAI, e.g. when UE does not select the N3IWF based on S-NSSAI configuration information per N3IWF, the AMF may determine a target N3IWF that supports the request NSSAI based on the list of supported TAs and the corresponding list of supported slices for each TA obtained in RAN Configuration Update procedure as specified in TS 38.413 [7].

8a.	Optionally, the AMF may provide the target N3IWF information (e.g. FQDN and/or IP address) to UE within Registration Reject message so that UE can use the target N3IWF information to select the target N3IWF to register to 5GC.

8b.	Alternatively, if the selected N3IWF cannot support the requested NSSAI, AMF may continue the registration procedure as defined in clause 4.12.2.2 steps 7 to 11 and provides the target N3IWF information (e.g. FQDN and/or IP address) to UE within Registration Accept message.

Editor’s note: It is FFS whether and how the AMF can accept the Registration with the initial N3IWF/TNGF if that N3IWF/TNGF does not support the set of slices requested by the UE.

If the S-N3IWF supports the requested NSSAI, AMF continue the registration procedure as defined in clause 4.12.2.2 steps 7 to 10.

9.	The AMF sends the NAS Registration Accept message to the N3IWF. The N2 Message includes the Allowed NSSAI for the access type for the UE. If AMF determines that the S-N3IWF cannot support the requested NSSAI in step 7 and continue the registration procedure, the NAS Registration Accept message contains the target N3IWF information.

10.	The N3IWF forwards the NAS Registration Accept message to UE via the established signalling IPsec SA. If the NAS Registration Accept message is received by the N3IWF before the IPsec SA is established, the N3IWF shall store it and forward it to the UE only after the establishment of the signalling IPsec SA.

11.	If UE is rejected, UE uses the target N3IWF information to select T-N3IWF and registers to 5GC via T-N3IWF. If UE is not rejected and obtains the target N3IWF information in step 10, UE may decide to re-register to 5GC via T-N3IWF.

### 6.15.4	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

UE impacts:

-	select the N3IWF based on N3IWF identifier configuration that includes the supported S-NSSAI(s) configuration per FQDN or IP address of N3IWF

-	re-select N3IWF based on the N3IWF information provided by the AMF via NAS Registration Accept or Registration Reject message.

AMF impacts:

-	Ability to determine target N3IWF information based on the list of supported TAs and the corresponding list of supported slices for each TA obtained in RAN Configuration Update procedure.

-	provide UE with the N3IWF information via NAS Registration Accept or Registration Reject message.

## 6.16	Solution #16: N3IWF selection taking supported slices into account

### 6.16.1	Description

#### 6.16.1.1	Introduction

Key idea of this solution is to provide additional information to a UE to enable the UE to select an N3IWF that supports the network slices (identified by S-NSSAIs) that the UE is intending to register for, i.e., to select an N3IWF that supports the S-NSSAIs that the UE wants to include in the Requested NSSAI during the subsequent Registration procedure.

#### 6.16.1.2	Solution principles

##### 6.16.1.2.1	General

-	PLMN selection prior to N3IWF selection is performed as defined in TS 23.501 [2]

##### 6.16.1.2.2	N3IWF selection when the UE is located in the home country and when the UE is not located in the home country

-	UE configuration

-	AMF may provide the UE with the following information for the current PLMN (which the UE stores and applies as described further below) as part of the Registration or the UE Configuration Update procedure:

-	Slice-specific N3IWF prefix information: one or multiple entries, each consisting of:

-	List of supported S-NSSAIs

-	FQDN format: Tracking Area (TA) FDQN format or Operator Identifier (OI) FQDN format

-	Prefix

Editor's Note: It is FFS how this information fits with N3AN node selection information and Home N3IWF identifier configuration defined in 24.526.

-	N3IWF selection by the UE

-	If the UE has Slice-specific N3IWF prefix information for the PLMN that the UE wants to register with:

-	The UE selects a Slice-specific N3IWF prefix information entry that supports all (or most, in case there is no full match) of the S-NSSAIs that the UE intends to request in the Requested NSSAI, and creates an FQDN using the prefix from the selected prefix information as follows:

-	<Prefix>.tac-lb<TAC-low-byte>.tac-hb<TAC-high-byte>.tac.n3iwf.5gc.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org (if the FQDN format indicates TA FQDN format)

-	<Prefix>.n3iwf.5gc.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org (if the FQDN format indicates OI FQDN format)

-	If the UE has not received Slice-specific N3IWF prefix information from the AMF, then the UE selects an N3IWF using Rel-17 N3IWF selection procedures.

-	If the DNS returns an IP address for the FQDN and the UE has established the connection to the N3IWF but the AMF detects that the selected N3IWF does not support the S-NSSAIs requested by the UE in the Requested NSSAI, then the AMF may indicate updated Slice-specific N3IWF prefix information to the UE and may reject the UE’s registration request. Based on this, the UE performs N3IWF selection again.

NOTE:	Alternatively, the AMF can, after updating the UE with new Slice-specific N3IWF prefix information, redirect the UE to a different N3IWF as proposed by solution 11.

### 6.16.2	Procedures

Existing procedures are reused.

### 6.16.3	Impacts on Existing Nodes and Functionality

-	UE:

-	Support N3IWF selection based on Slice-specific N3IWF prefix information from AMF

-	AMF:

-	Indicate Slice-specific N3IWF prefix information to the UE

## 6.17	Solution 17: Untrusted Non-3GPP Access with N3IWF/TNGF Relocation due to network slicing

### 6.17.1	Description

The solution addresses KI#2 and enables the AMF to select a different N3IWF to serve the UE, when the originally selected N3IWF cannot support the slices needed by the UE.

Each N3IWF/TNGF may be locally configured with one TAI value and a corresponding set of S-NSSAIs (based on rel-17 enhancements done as part of TEI17_N3SLICE).

The key steps of the solution are summarized as follows:

1. 	UE Registers with initial N3IWF/TNGF (i.e. full initial registration procedure).

2. 	AMF informs the UE about target N3IWF/TNGF and releases the N2 UE connection

3. 	UE preforms Service Request via target N3IWF/TNGF.

### 6.17.2	Procedures

Figure 6.17.1-1 Registration and N3IWF/TNGF relocation to enable UE to be served by N3IWF/TNGF supporting the requested S-NSSAIs

1. The UE selects a PLMN and an N3IWF in this PLMN. This N3IWF is selected as currently specified in 23.501, i.e. without considering any slice information.

The UE initiates the initial Registration procedure which takes place as described in TS 23.502 [3] clause 4.12.2.2, steps 1-11 for untrusted non-3GPP access and clause 4.12a.2.2, steps 1-14 for trusted non-3GPP access, with the differences and clarifications described below.

The AMF (either the one selected by N3IWF, or the one selected after AMF re-allocation, as defined in clause 5.15.5.2.3 of TS 23.501 [2]) identifies whether the selected N3IWF can or cannot support the slices requested by the UE. The AMF knows the slices supported by each N3IWF connected with the AMF because (as defined in TS 38.413 [7]) the N2 Setup Request message sent from N3IWF to AMF includes the supported TAI and the supported S-NSSAIs for this TAI.

2. 	The AMF sends Registration Accept to the UE and includes information about target N3IWF(s). The N3IWF information may contain information about N3IWFs and what S-NSSAIs each N3IWF supports.

Editor’s note: it is FFS whether the AMF may provide information about target TNGF(s). How the AMF is configured with TNGF(s) available via the SSID+BSSID that the UE is using is also FFS.

Editor’s note: The details on what N3IWF/TNGF information is provided to the UE is FFS. It is e.g. FFS whether the 5GC selects a N3IWF/TNGF on behalf of the UE, or provides information about multiple N3IWFs/TNGFs and allow the UE to select N3IWF/TNGF. It is also FFS for how long the information is intended to be valid in the UE.

Editor’s note: It is FFS whether and how the AMF can accept the Registration with the initial N3IWF/TNGF if that N3IWF/TNGF does not support the set of slices requested by the UE.

Editor’s note: In case 5GC or provides information about multiple N3IWFs/TNGFs and allowing the UE to select N3IWF/TNGF, how to enable N3IWF selection taking also the current tracking area of the UE in 3GPP access into account (i.e. how to support N3IWF selection based on the TA FQDN) is FFS.

3. 	The AMF triggers N2 connection release, as described in TS 23.502 clause 4.12.4.2 (for untrusted non-3GPP access) or clause 4.12a.4.2 (fur trusted non-3GPP access). As part of this procedure, the N3IWF performs the release of the IPsec tunnel. The AMF includes information about target N3IWF in step 3a. The N3IWF provides this information to the UE in step 3b.

Editor’s note: It is FFS whether target N3IWF/TNGF information should be provided in NAS Registration Accept (step 2) or via AS parameters (step 3a-3b). Both options are described in this solution, but only one is to be selected.

4. 	The UE selects a N3IWF/TNGF from the information provided in step 2 (for NAS) or step 3 (for AS) and triggers Service Request procedure as described in TS 23.502, clause 4.12.4.1 (for untrusted non-3GPP access) or clause 4.12a.4.1 (for trusted non-3GPP access).

Editor’s note: It is FFS how a UE can select a TNGF (the TNGF is selected by the WLAN AN).

### 6.17.3	Impacts on Existing Nodes and Functionality

UE:

-	Able to receive a new N3IWF/TNGF information via NAS (NAS option) or IKE (AS option)

N3IWF-1/TNGF-1:

-	AS option: Able to receive new N3IWF/TNGF information from the AMF and provide this to the UE

N3IWF-2/TNGF-2:

-	No impact.

AMF:

-	Able to trigger N3IWF/TNGF relocation, e.g., based on the Requested NSSAI provided by the UE, the Subscribed NSSAI for this UE, the S-NSSAIs supported by all TNGFs that can serve the UE, and other information.

Editor’s note: it is FFS whether the AMF may provide information about target TNGF(s). How the AMF is configured with TNGF(s) available via the SSID+BSSID that the UE is using is also FFS.

-	The AMF includes new N3IWF/TNGF information in NAS Registration Accept (NAS option) or N2 UE Context Release Command (AS option).

## 6.18 Solution 18: ANDSP based solution for slice related TNGF selection

### 6.18.1	Description

This solution addresses the Key Issue 2 on How to select a TNGF that supports the S-NSSAI(s) needed by the UE.

ANDSP (Access Network Discovery & Selection policy) was designed to be provided to UE by the network for selection of TNGF/N3IWF. This solution proposes to add the supported S-NSSAI(s) for the TNGF in ANDSP to aid the UE in the selection of the TNGF that supports the S-NSSAI(s) needed by the UE.

### 6.18.2	Procedures

Editor's note:	This clause describes services and related procedures for the solution.

#### 6.18.2.1 Procedures for TNGF selection

When the UE initiates trusted non-3GPP access to 5GC, it can selects a WLAN access network based on WLANSP rules, as described in current(Rel-17) clause 6.6.1.3 of 3GPP TS 23.503[4]. The UE can be provided by the HPLMN and by the VPLMN with WLANSP rules to assist UE the selection of WLAN in home network or in visited network. WLANSP rule can include PreferredSSIDList which is prioritized list of SSIDs preferred for selection, as described in 3GPP TS 23.402[Y]. PreferredSSIDList can be used as a selection criteria for the UE to constructs a prioritized list of the available WLANs by discovering the available WLANs and comparing their SSIDs against the PreferredSSIDList. From the prioritized list of the available WLANs, the UE selects the highest priority WLAN network and starts the 5GC registration procedure. Above current(Rel-17) procedures dose not consider the needed S-NSSAI(s).

To aid the UE in the selection of the WLANs that supports the S-NSSAI(s) needed by the UE, it is proposed to configure the supported S-NSSAI(s) for the SSID in the PreferredSSIDList. Here the supported S-NSSAI(s) information is not used as selection criteria which the UE compares the attributes/capabilities of the available WLANs against. It is designd for the UE to select the WLAN supporting the needed S-NSSAI(s) from prioritized list of WLANs. After the UE constructs a prioritized list of the WLANs following the existing procedures of WLAN access selection based on WLANSP rules, the UE can know the supported S-NSSAI(s) for the SSID/WLAN in the list, if these SSID match the PreferredSSIDList and the matched SSID is configured with supported S-NSSAI(s). When the UE desides to select a WLAN supporting S-NSSAI(s) needed by the UE for registration, the UE select the SSID/WLAN from the prioritized list of the WLANs.

The HPLMN or VPLMN can provide a UE with PreferredSSIDList in WLANSP rules so that the list of SSIDs in PreferredSSIDList cover all the UE's subscribed S-NSSAI(s).

After the UE selects the WLAN/TNAP, the UE follows the existing registration procedure specified in clause 4.12a.2.2 of TS 23.502[3], and connects to the TNGF supporting the needed S-NSSAI(s). It is assumed the TNAP selects a TNGF based on the realm provided by the UE and based on the SSID selected by the UE and the TNGF supports all the S-NSSAIs for this SSID.

NOTE:	the solution configures the UE with the supported S-NSSAI(s) on a per SSID basis so assumes that the a SSID gives access to TNGF(s) that support the same set of slices all over the country.

### 6.18.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

The following impacts are foreseen by this solution:

UE:

-

-  select the SSID/WLAN which supports the needed S-NSSAI(s) from the prioritized list of the WLANs.

PCF in HPLMN:

-

-  provide the supported S-NSSAI(s) for the SSID in the PreferredSSIDList in WLANAP

PCF in VPLMN:

- provide the supported S-NSSAI(s) for the SSID in the PreferredSSIDList which is in VPLMN.

## 6.20	Solution 20: providing differentiated service for UE and non-3GPP devices connected behind a 5G-RG

### 6.20.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub) clause(s) may be added to capture details.

In Rel-18 FS_5WWC_Ph2, the KI#1 aims to provide differentiated service (e.g. QoS and charging) for UE and Non-3GPP devices connected behind a 5G RG. Guided by the existing knowledge in 3GPP (e.g. ProSe), the study for L-3 UE-to-Network Relay without N3IWF in 5G can also be applied to UE connects to the 5G-RG for UE approach.

In this solution, 5G-RG obtains User ID and UE info of devices connected behind a 5G-RG. And 5G-RG sends these information to the SMF, in turn delivered to the PCF. Then the PCF or SMF can make corresponding policy decisions related to e.g. QoS control or charging.

### 6.20.2	Procedures

#### 6.20.2.1	providing differentiated service for non-3GPP devices connected behind a 5G-RG

Editor's note:	This clause describes high-level procedures and information flows for the solution.

Figure 6.20.2.1-1

1. Non-3GPP device connects to 5G-RG with non-3GPP access network (e.g. establishes WiFi association).

During this procedure, 5G-RG obtains non-3GPP device’s User ID and Non-3GPP device info. The User ID is an identity of the non-3GPP device (e.g. MAC address, or SUCI which could be acquired by Authentication procedure for NSWOF in TS 33.501[9] v17.5.0 Annex S, or SUPI). The non-3GPP device info is used to assist identifying the non-3GPP device in the 5GC. For IP PDU Session Type, the non-3GPP device info is non-3GPP device IP info. For IPv4, 5G-RG as a NAT shall report TCP/UDP port ranges assigned to individual non-3GPP device (s). For Ethernet PDU Session Type, the non-3GPP device info is non-3GPP device Ethernet address.

Editor's note: Whether and how the SUPI could be acquired is FFS.

2. 5G-RG sends Remote UE Report message to the SMF through the AMF. The Remote UE Report message includes Non-3GPP device’s User ID and Non-3GPP device info.

NOTE:	It is assumed that the 5G-RG already establishes a PDU session.

Editor's note: Whether use the Remote UE Report message or define a new message is FFS.

3. The SMF sends non-3GPP device’s User ID and Non-3GPP device info to the PCF by initiating the SM Association Modification procedure. The SMF may store the Non-3GPP device’s User ID and Non-3GPP device info.

4. The PCF stores the Non-3GPP device’s User ID and Non-3GPP device info.

Editor's note: The PCF can get policy information for Non-3GPP device from local configuration or from UDR. The detail on how to configure and store and manage the policy information for the non-3GPP device may need further study, considering these operations may relate to some services.

5. The PCF shall authorize policy for the non-3GPP device based on policy information at step 4.

6. The PCF generates and sends the PCC rules related to e.g. QoS or charging for non-3GPP devices connected behind a 5G-RG.

7. The SMF performs charging and QoS provision via existing mechanisms. And the SMF provides the charging information with corresponding User ID to the CHF.

Editor's note: whether use the same CHF session deals with the 5R RG traffic and with the traffic of different devices is FFS.

#### 6.20.2.2	differentiated QoS for 5GC-capable UE connected behind a 5G-RG

Figure 6.20.2.2 -1

The solution is based on the architecture as defined in clause 4.10 of TS 23.316[5] which UE connects to the overlay 5G network using the trusted N3GPP for UE approach.

In step 1, UE completes authentication and receives IP configuration during Registration procedure for trusted non-3GPP access in clause 4.12a of TS 23.502[3]. And 5G-RG obtains UE info (e.g. MAC address, or SUCI of the UE).

In step 2 and 3, the 5G-RG (acts as a TNAP) performs step 2 and 3 as in clause 6.20.2.1 with the modification where the UE takes the role of "non-3GPP devices", to report User ID (e.g. MAC address, or SUCI of the UE) and UE info to 5GC.

In step 4, PCF shall authorize QoS to UE, e.g. with the configured default QoS for the UE as described from step 4 to 7 in clause 6.20.2.1. (by configuration) If needed, the 5GC at 5G-RG side can perform differentiated charging for the data flow of this UE following the procedures in 6.20.2.1, although the 5GC at the UE side will perform charging for the UE. This approach can handle the issue of repeat billing at 5G-RG, by excluding the amount of data generated by 5GC-capable UEs behind the 5G-RG.

In step 5 and 6, once the TNGF acquires the UE’s QoS information e.g. UE-AMBR or session-AMBR from 5GC when UE setup PDU session, it sends the corresponding QoS information to 5G-RG through the Ta interface. Then 5G-RG stores the QoS information for the UE and performs PDU session modification procedure to request the QoS for the UE’s data flow identified by UE info. For example, the 5G-RG can request MBR for the data flow of this UE in the PDU session of 5G-RG. The SMF and PCF perform subsequent procedures for PDU Session Modification to authorize and enforce the QoS for the UE’s data flow in the PDU session.

### 6.20.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

5G-RG:

-	 5G-RG sends User ID and UE info to the SMF through the AMF, e.g. by Remote UE Report message

-	 5G-RG receives Qos information of the UE from TGNF and performs PDU session modification procedure to request the QoS for data flow of the UE.

SMF:

-  The SMF sends User ID and UE info to the PCF by initiating the SM Association Modification procedure.

-	The SMF provides the charging information with User ID to the CHF.

-  The SMF may store the User ID and UE info of devices connected behind a 5G-RG.

PCF:

-  The PCF can get policy related to e.g. QoS control, or charging of Non-3GPP device.

-  The PCF stores the User ID and UE info of devices connected behind a 5G-RG.

-  PCF authorizes policy for non-3GPP devices or UE behind the 5G-RG.

TNGF:

-  The TNGF sends the corresponding QoS information of the UE to 5G-RG through the Ta interface.

## 6.21	Solution 21: Non-3GPP device behind 5G-RG based on 5GS exposure

### 6.21.1	Description

#### Introduction

This solution addresses KI#1 for providing differentiated services (e.g. QoS and charging) for Non-3GPP devices behind a 5G-RG.

The solution consists of three parts that are used to provide a working e2e solution:

1. 	Example for how non-3GPP device information can be created in an AF

2. 	Enhancements to the NEF exposure services to provide the non-3GPP device information to 5GC

3. 	Description for how the traffic from non-3GPP devices can be identified in the 5GC to provide differentiated charging and QoS.

#### Overall Architecture

The overall architecture is shown in Figure 6.21.1-1. Only NFs relevant for the solution are shown.

Figure 6.21.1-1. Overall architecture

#### Functional Description

###### Providing non-3GPP device information to AF

In this solution, the AF is assumed to have access to information about the non-3GPP devices that are or have been connected behind the RG. An example for how this can be done is described here. The details are however out of 3GPP scope

Based on existing BBF specifications TR-69/369 and TR-181, the Auto-Configuration Server (ACS) can retrieve information about the non-3GPP devices from the 5G-RG. This information can e.g. contain the host table from the DHCP server in the RG, or device list gathered by other means, and typically includes for each device: a host name, the MAC address of the device and the IP address allocated to the device. An example of such a table for IPv6 is shown in Figure 6.21.1-2.

Figure 6.21.1-2. Example of IPv6 host table from an RG

In case of IPv4 traffic, the routed RG typically has NAT functionality. The IPv4 addresses in the list of non-3GPP devices received from the RG would thus correspond to the private IPv4 addresses. These private addresses are not useful for traffic detection in the 5GC. The solution is therefore only applicable to IPv6.

Editor’s note: If and how the solution can work also for IPv4 is FFS.

The non-3GPP device information is then provided from the ACS to the AF. How this is done is out of scope of 3GPP. For reference, the BBF has specified requirements for such an interface in TR-131 ACS Northbound Interface Requirements – Amendment 1, 2015.  The BBF has also done work on a RESTful API in a TR-369 controller (https://github.com/BroadbandForum/usp/tree/master/api).

It is further assumed that the operator may integrate a web portal with the AF. The end-user (e.g. the person that owns the subscription for the RG) can login to this web portal and manage the list of devices. The end-user could e.g. select a device profile for each device from a drop-down menu. The device profile may e.g. indicate what type of non-3GPP device it is (set top box, printer, tablet, PC etc), or be a reference to a QoS category. How this Device profile ID is to be interpreted is up to operator configuration.

In summary:

1)	It is assumed that the AF has a table containing a complete list of non-3GPP devices in the home. The table includes a host name (which may be informative but is not guaranteed to be unique), MAC address, assigned IPv4 address (local to the home LAN) and IPv6 address.  The combination of host name and MAC address is considered to be a unique and persistent identification of a specific device connected to the home LAN.

2)	It is assumed that the AF may be directed by an authorized subscriber by means out of scope of 3GPP to request differentiated service for specific devices in the AF’s list of non-3GPP devices.

It should be noted that similar systems and procedures are employed in wireline networks today, but these would benefit from the enhanced differentiation capabilities of the 5GS

###### Provisioning of non-3GPP device information to 5GC

The existing NEF Service Parameter service is enhanced to allow an AF to provide the non-3GPP device information to 5GC. This information will be used by 5GC to detect the traffic to/from a non-3GPP device and also to provide differentiated QoS and/or charging.

The information provided by the AF via the Nef_ServiceParameter service contains:

- 	GPSI of the RG

-	List of non-3GPP devices, containing for each device:

o	IPv6 address of the device

o	Device profile ID

The NEF maps the RG’s GPSI to the RG’s SUPI and stores the non-3GPP device information in UDR as Application Data, as currently defined for Nnef_ServiceParameter service in TS 23.502 [3], clause 4.15.6.7.

###### Differentiated services per non-3GPP device

When a PDU Session for an RG is established, the PCF contacts the UDR to subscribe to Application Data that may be available, as per existing procedure for Service specific parameter provisioning in TS 23.502 [3], clause 4.15.6.7. The PCF thus receives the non-3GPP device information from UDR corresponding to the RG’s SUPI.

The PCF takes the Device profile ID as well as other information (e.g. RG’s subscribed QoS and RG’s policy subscription data in UDR) into account for policy decisions, e.g. to determine QoS and charging parameters for the non-3GPP device’s traffic. The PCF may provide PCC rules to SMF that are specific for individual non-3GPP devices, containing SDF filter with the IPv6 address of the device, and corresponding QoS and charging related parameters. The PCF may provide different PCC rules for different services, as per existing standards.

Editor's note: Whether the device profile ID is needed or if the existing parameters (QoS reference or explicit QoS parameters) are sufficient is FFS.

### 6.21.2	Procedures

The procedures below are based on the existing Service specific parameter provisioning procedure in TS 23.502, clause 4.15.6.7, but adapted to fit the use case.

The procedure for the provisioning of non-3GPP device information is shown below:

Figure 6.21.2-1. Provisioning of non-3GPP device information

1-5. 	The end-user logs in to customer portal and manages the list of non-3GPP devices associated with the RG.

6. 		The AF invokes Nnef_ServiceParameter service to provide the non-3GPP device information for the RG’s GPSI to the NEF.

7. 		The NEF maps the RG’s GPSI to a SUPI and stores the information in UDR.

8. 		The NEF sends a Nnef_ServiceParameter Response to the AF.

The procedure for providing differentiated services for the traffic of a non-3GPP device is shown below.

Figure 6.21.2-2. Provisioning of providing differentiated services to non-3GPP devices

1. 	The 5G-RG establishes a PDU Session.

2. 	The PCF subscribes to non-3GPP device information from the UDR. PCF may e.g. use the DNN to determine whether to request such data from UDR.

3. 	In case the non-3GPP device information is updated during the lifetime of the 5G-RG’s PDU Session, the UDR notifies the PCF about the updated information.

4. 	The PCF takes the non-3GPP device information into account for its policy decision. The non-3GPP Device profile ID may map to a set of QoS parameters or charging related parameters.

5. 	The PCF provides updated policy rules to the SMF.

6. 	The SMF initiates corresponding updates of the N4 rules.

### 6.21.3	Impacts on Existing Nodes and Functionality

NEF: Extensions to the NEF Service Parameter Service to allow an AF to provision non-3GPP device information.

UDR: Extensions to the Application Data in UDR to store non-3GPP device information (new Data Type).

PCF: Ability to retrieve non-3GPP device information from UDR and take it into account for policy decisions.

There are no impacts to SMF and UPF since existing capabilities are re-used.

It is assumed that the 5G-RG can provide a list of non-3GPP devices to the ACS, with host name, MAC address and IP address for each device.

Impacts to and assumptions on the 5G-RG and ACS are out of 3GPP scope and are to be verified by BBF/Cablelabs.

## 6.22	Solution 22: Support differentiated QoS for AUN3 devices

### 6.22.1	Description

This clause specifies a solution for KI#1. It specifies how to support traffic identification and differentiated QoS for Authenticable Non-3GPP (AUN3) devices connected behind a 5G-RG. As defined in clause 3.1, an AUN3 device is a Non-3GPP device which does not support NAS over non-3GPP access but it can be authenticated by 5GC (so, it possesses 5G credentials).

In short, the solution supports the following capabilities:

a)	It enables an AUN3 device to connect to a 5G-RG after being authenticated and authorized by the HPLMN of this device. The authentication procedure does not require 5GS registration because it is based on the NSWO authentication procedure specified in TS 33.501, Annex S.

b)	It enables the AUN3 device to exchange data packets via the PDU Session of 5G-RG, each one marked with a unique Traffic Identifier (thus, it enables per-device traffic identification and charging); and

c)	It enables the AUN3 device to exchange data packets via the PDU Session of 5G-RG, all of them using a specific QoS flow (thus, it enables per-device QoS handling).

### 6.22.2	Procedures

The main steps of the solution are illustrated in the following figure and are described below. It is assumed that the SMF supports an SWa interface and can interact with a Non-Seamless WLAN Offload Function (NSWOF) defined in TS 23.501 [2].

Figure 6.22.2-1

0.	The 5G-RG registers with a 5G core network (PLMN (5G-RG)) using its own 5G credentials and establishes a PDU Session, which can be used by devices operating "behind" the 5G-RG, such as, the AUN3 device. This PDU Session typically supports one or more QoS flows, each one providing different QoS handling.

The PDU Session of the 5G-RG must be controlled by an SMF that supports the SWa interface and, therefore, it can communicate with one or more NSWOFs. If not all SMFs in the PLMN (5G-RG) support this capability, then, during this PDU Session establishment, the AMF must select an SMF that supports this capability (i.e., the SWa interface). The AMF may decide to select an SMF that supports the SWa interface, either because:

a)	It receives a new indication from the 5G-RG, e.g., a new indication in the NAS message sent from the 5G-RG to AMF; or

b)	It receives a new indication in the subscription information of 5G-RG (retrieved from UDM), which indicates that all PDU Sessions of this 5G-RG must use an SMF that support the SWa interface.

The alternative a) is preferable because it allows the 5G-RG to send the new indication to AMF only for the PDU Sessions which can be shared by devices "behind" the 5G-RG.

1.	The AUN3 device requests to connect to 5G-RG, e.g., it performs a WLAN association with the 5G-RG. This triggers the 5G-RG to initiate an EAP-based authentication procedure, hence, it sends an EAP-Request/Identity to AUN3, which responds with an EAP-Response/Identity containing its Subscriber Concealed Identity (SUCI) in a form of a Network Access Identifier (NAI). The realm part of NAI contains the MCC/MNC of the HPLMN of the AUN3 device.

2.	The 5G-RG requests from SMF to authenticate the AUN3 device and to determine whether the AUN3 device is authorized to connect to the 5G-RG and share its PDU Session. For this purpose, the 5G-RG sends a new 5GSM message to SMF, called PDU Session Third-Party Authentication Request message. The "Third-Party" signifies that the authentication request is not for the 5G-RG but for another device operating behind the 5G-RG. The PDU Session Third-Party Authentication Request message includes the EAP-Response/Identity received by the AUN3 device.

Editor’s note: The solution assumes that the SMF supports an SWa interface and interacts with NSWOF. However, it is possible that the SWa interface is supported by the 5G-RG and the 5G-RG interacts directly with NSWOF. In this case, the SMF is not involved in the authentication procedure of AUN3 device and the new 5GSM messages are not needed. The details of this case are FFS.

3-6. A NSWO mutual authentication procedure takes places between the AUN3 device and AUSF, using EAP-AKA' authentication. These steps are the same as those specified in TS 33.501, Annex S, with the following differences:

a)	In step 4a, the UDM provides not only the Subscriber Permanent Identity (SUPI) of the AUN3 device, but also "QoS Info" for the data traffic of the AUN3 device. This "QoS Info" is part of the subscription data of the AUN3 device and identifies the QoS that should be applied to handle the data traffic of the AUN3 device when carried over the PDU Session of the 5G-RG. All traffic associated with the same AUN3 device has the same QoS treatment.

For example, the "QoS Info" may contain QoS parameters such as a 5QI value and/or a packet delay budget and/or a packet error rate. The "QoS Info" is later used by SMF to determine the QoS flow on which the data traffic of AUN3 device should be sent over the PDU Session of the 5G-RG.

The UDM decides to provide the QoS Info based on the information received in step 3c, for example, based on the value of the Access Network Identity, or another parameter. For example, if the Access Network Identity is different from "5G:NSWO" (which is specified in TS 33.501, Annex S, for NSWO), then the UDM may decide to provide the QoS Info.

Editor’s note: Instead of sending the QoS Info in step 4a, the SMF (after step 6b) may request from UDM subscription data for the AUN3, including the QoS Info. Whether this alternative is better is FFS.

b)	In step 6a, if the authentication procedure is successful, the AUSF provides the QoS Info received from UDM and also the SUPI of the AUN3 device.

c)	In step 6b, the SUPI and the QoS Info are forwarded to SMF. This way the SMF receives both the permanent subscriber identity of the AUN3, which can be used for charging, and information indicating the QoS that should be used to handle the data traffic of the AUN3 device.

d)	In step 6c, the SMF responds to 5G-RG with a PDU Session Third-Party Authentication Result message. If the authentication procedure is successful, this message contains, not only the EAP-Success packet and the Master Session Key (MSK), but also the following parameters:

-	A Traffic Identifier, which will be carried in all GTP-U or GRE datagrams exchanged between the 5G-RG and UPF that encapsulate data packets to/from the AUN3 device. Via this identifier, the UPF can identify the traffic of each AUN3 devices behind the 5G-RG.

Editor’s note: The 5G-RG encapsulates all traffic of the AUN3 device into either GTP-U or GRE packets, which carry the Traffic Identifier in their headers. It is FFS whether GTP-U or GRE is used. It is also FFS whether the Traffic Identifier needs to be sent over the user plane.

-	The QoS flow on which the data traffic of AUN3 device should be sent when carried over the PDU Session of the 5G-RG.

8.	After the EAP-Success message is sent, the AUN3 device and the 5G-RG establish a security context to secure the air-interface traffic. In case of WLAN access, this is typically accomplished by applying the 4-way handshake procedure specified in IEEE 802.11. In addition, the AUN3 device obtains IP configuration data, including an IPv4/IPv6 address/prefix.

9.	The SMF sends to UPF the SUPI of the AUN3 device and the associated Traffic Identifier, so that the UPF can identify and, possibly, charge the data traffic of the AUN3 device.

Editor’s note: The details of how and if the Traffic Identifier can be used for charging the data traffic of the AUN3 device are FFS.

10.	If the QoS flow provided by SMF in step 6c does not exist in the PDU Session of the 5G-RG, the 5G-RG initiates a PDU Session Modification procedure to establish a new QoS flow.

12.	The 5G-RG sends the data traffic received from the AUN3 device to the QoS flow corresponding to this AUN3 device. Similarly, the UPF sends the data traffic received from the data network and is destined to the AUN3 device to the QoS flow corresponding to this AUN3 device. Between the 5G-RG and the UPF, the data traffic of the AUN3 device is encapsulated within GTP-U or GRE datagrams, each one containing the Traffic Identifier that corresponds to this AUN3 device (the one provided by SMF in step 6c).

### 6.22.3	Impacts on Existing Nodes and Functionality

HPLMN (AUN3 device):

Shall be able to support NSWO authentication (as defined in TS 33.501, Annex S) and shall be able to provide QoS Info for the AUN3 device. The QoS Info could be part of the subscription data of the AUN3 device.

AMF:

Shall be able to select an SMF that supports the SWa interface.

Editor’s note: It is FFS how determines to select an SMF that supports the SWa interface.

NRF:

Manage a new SMF registration and discovery capability related with SWa support.

SMF:

Shall be able to support the SWa interface and to initiate an NSWO authentication procedure with the HPLMN of the AUN3 device. It shall also be able to create a Traffic Identifier for the AUN3 device and to determine the QoS flow that should be used for the AUN3 device. In addition, it shall support the new 5GSM messages "PDU Session Third-Party Authentication Request/Response".

5G-RG:

Shall encapsulate the data traffic of the AUN3 device within GTP-U or GRE datagrams, each one containing the Traffic Identifier that corresponds to this AUN3 device. In addition, it shall support the new 5GSM messages "PDU Session Third-Party Authentication Request/Response".

UPF:

Shall encapsulate the data traffic of the AUN3 device within GTP-U or GRE datagrams, each one containing the Traffic Identifier that corresponds to this AUN3 device.

Editor’s note: It is FFS whether the Traffic Identifier needs to be sent over the user plane.

NAS protocol:

New 5GSM messages are introduced: "PDU Session Third-Party Authentication Request/Response".

UDM:

If the UDM provides the QoS Info in step 4a (see relevant Editor’s note in clause 6.22.2), the UDM shall provide the QoS Info when specific information is received from AUSF, e.g., based on the value of the received Access Network Identity.

PCF:

Editor’s note: The impact on PCF is FFS.

## 6.23	Solution 23: Delay budget for non-3GPP devices behind 5G-RG

### 6.23.1	Description

QoS experienced by non-3GPP devices behind a 5G-RG depends on the end-to-end path between a non-3GPP device and the application server, i.e. depends on the QoS differentiation in both the 3GPP network and the non-3GPP network attached to the 5G-RG.

The packet delay budget (PDB) currently only considers the delay between the UPF and the 5G-RG; the additional delay between the 5G-RG and the non-3GPP devices is not accounted for.

To address this shortcoming, this solution proposes to make 5GS aware of the additional delay that can occur in the non-3GPP network attached to the 5G-RG and to compensate for this delay in 5GS. The compensation is achieved by reducing the PDB for the 3GPP network by the delay that can occur in the non-3GPP network attached to the 5G-RG (referred to as non-3GPP delay hereafter).

This solution proposes two enhancements to make 5GS aware of the non-3GPP delay:

-	A Default non-3GPP network delay budget is configured in the UDR as part of the PDU Session policy control subscription information. The PCF applies the Default non-3GPP delay budget when taking policy decisions, i.e., the PCF reduces the PDB for a given flow by the Default non-3GPP delay budget.

-	If supported by the 5G-RG and if the 5G-RG wants to change the default non-3GPP delay budget for a specific flow, the 5G-RG may use the UE requested PDU Session Modification procedure to request a non-3GPP delay budget for a set of packet filters. The SMF provides the Requested non-3GPP delay budget received from the 5G-RG to the PCF, which takes it into account for the subsequent policy decision.

The solution applies to 5G-RG connected via FWA as defined in TS 23.316 [5]. The solution addresses Non authenticable non-3GPP devices and Authenticable non-3GPP devices.

### 6.23.2	Procedures

Figure 6.23.2-1: Taking delays in the non-3GPP network attached to the 5G-RG into account in 5GS

1.	PDU Session is established

2.	Application layer signalling is exchanged between an application on a non-3GPP device connected to the 5G-RG and an application function (AF)

3.	Based on the application layer signalling, the AF requests QoS for application flows.

NOTE 1:	If the AF is not considered trusted, then the AF interacts with the NEF instead. If the AF provides explicit QoS parameters, then AF (or NEF in case the AF is not trusted) interacts with PCF as defined in clause 4.15.6.6 of TS 23.502 [3].

NOTE 2:	Steps 1-3 are following existing Rel-17 specifications.

4.	PCF creates/updates PCC rules based on the AF request. If the PDU Session policy control subscription information in the UDR contains a Default non-3GPP delay budget, then the PCF takes it into account by reducing the PDB for the application flows by the Default non-3GPP delay budget.

5.	PCF sends the PCC rules to the SMF and also includes the Non-3GPP delay budget.

6.	SMF completes the PDU Session modification based on the PCC rules received from the PCF and includes in the N1 SM container the Non-3GPP delay budget provided by the PCF. A Pre-Rel-18 5G-RG or a Rel-18 5G-RG that does not support the Non-3GPP delay budget information, will ignore the Non-3GPP delay budget included in the N1-SM container.

7.	Application traffic is exchanged between the application on a non-3GPP device connected to the 5G-RG and an application function (AF).

8.	If the 5G-RG supports signaling of Non-3GPP delay budget information and determines the need to request a different Non-3GPP delay budget for a specific flow of a non-3GPP device, then the 5G-RG initiates the UE requested PDU Session Modification procedure and includes the Requested Non-3GPP delay budget. The packets filters that the 5G-RG includes in the UE requested PDU Session Modification Request include at least the non-3GPP devices’ IP address and optionally port numbers. In case 5G-RG has assigned a private IP address to the non-3GPP device, then the 5G-RG includes its own IP address and the port numbers used for the non-3GPP device.

NOTE 3:	How the 5G-RG determines the need to request a different Non-3GPP delay budget for a specific flow (e.g. by monitoring the delays in the non-3GPP network) is up to 5G-RG implementation.

9.	The SMF provides the Requested Non-3GPP delay budget to the PCF.

10.	PCF creates/updates PCC rules based on the Requested Non-3GPP delay budget received from the SMF by reducing the PDB for the application flows by the Requested Non-3GPP delay budget received from the SMF. The PCF may also alternatively apply a different Non-3GPP delay budget, based on operator policy. PCF uses the packet filters as signaled by the 5G-RG for the PCC rules as per existing procedures.

11.	PCF sends updated PCC rules to the SMF and also includes the Non-3GPP network budget.

12.	SMF completes the PDU Session modification based on the PCC rules received from the PCF and includes in the N1 SM container the Non-3GPP delay budget provided by the PCF.

### 6.23.3	Impacts on Existing Nodes and Functionality

-	SMF:

-	Support signaling of non-3GPP delay budget between UE and PCF

-	PCF:

-	Receive Default non-3GPP delay budget from UDR

-	Take Default non-3GPP delay budget from UDR into account for policy decisions (specifically for calculating PDB)

-	Provide Non-3GPP delay budget to SMF

-	Optionally, support receiving Requested non-3GPP delay budget from SMF

-	Optionally, take Requested non-3GPP delay budget from SMF into account for policy decisions (specifically for calculating PDB)

-	UDR:

-	Support Default non-3GPP delay budget as part of the PDU Session policy control subscription information

-	5G-RG:

-	Optionally, support receiving Non-3GPP delay budget from SMF

-	Optionally, support signaling Requested Non-3GPP delay budget to SMF

## 6.24	Solution 24: Differentiated QoS for Non-Authenticable non-3GPP devices behind RG.

### 6.24.1	Description

The solution addresses KI#1 and describes how differentiated QoS can be provided to non-authenticable non-3GPP devices (NAUN3) connected behind the RG (5G-RG and FN-RG). A NAUN3 device is a non-3GPP device that the 5GC can not authenticate.

The NAUN3 device is locally authenticated by the 5G-RG using such as pre-shared secret etc and is invisible to the 5G network. Differentiated QoS can still be provided by using 5G-RG’s local configuration (based on DNN / S-NSSAI for the 5G RG's PDU Sessions).

Using local-configuration, the 5G-RG can perform Traffic separation to isolate traffic from the NAUN3 devices and map them to appropriate PDU Session/QoS flows. The overall architecture is illustrated in Figure 6.24.1-1

Figure 6.24.1-1 Architecture for NAUN3 devices behind 5G-RG

### 6.24.2	Procedures

1. The 5G-RG registers to 5GC as specified in clause 7.2.1.1 of TS 23.316[5].

2. The 5G-RG PDU session establishment is done as specified in 7.3.1.1 of TS TS 23.316[5].

3. Using local-configuration, the 5G-RG can perform Traffic separation to isolate traffic from the NAUN3 devices and map them to appropriate PDU Session/QoS flows.

### 6.24.3	Impacts on Existing Nodes and Functionality

The solution largely re-uses existing functionality.

5G-RG:

- Be able to map the traffic from NAUN3 devices to a PDU session/QoS Flow of the 5G-RG.

- Further considerations on 5G-RG would be brought to BBF and/or CableLabs attention is FFS

## 6.25	Solution 25: Differentiated QoS for Authenticable non-3GPP devices behind RG.

### 6.25.1	Description

The solution addresses KI#1 and describes how differentiated QoS can be provided to authenticable non-3GPP devices (AUN3) connected behind the RG (5G-RG and FN-RG). An AUN3 device can be authenticated by the 5GC based on a permanent identifier (e.g. NAI, physical Mac address).

NOTE: A N3GPP device using a randomized MAC address can still be identified based on EAP Identity (NAI) if configured on the device. If a N3GPP device uses a randomized MAC address for authentication, it will be treated as NAUN3 device. It is up to SA3 to study how to identify and authenticate a device using randomized MAC address. In principle, the registration procedure of N5GC devices connecting via W-5GAN described in clause 4.10a of Rel-16 TS23.316 [5] can be followed by AUN3 devices. The authentication procedure of N5GC device as described in Rel-16 TS 33.501 [5] can be used to authenticate AUN3 devices.

The architecture for providing differentiated services to an AUN3 device behind 5G-RG is illustrated in Figure 6.25.1-1.

Figure 6.25.1-1: AUN3 device behind 5G-RG

The salient features of this solution are the following:

- The AUN3 device is authenticated by the 5GC.

- 5GC can setup a distinct PDU session for each AUN3 device.

- 5GC can setup distinct QoS Flows for each AUN3 device.

NOTE: FWA deployment is NOT applicable to AUN3 devices.

### 6.25.2	Procedures

The high-level procedure for this solution is described in Figure 6.25.2-1.

NOTE: Each AUN3 device is registered to 5GC and has a unique subscription record in UDM/UDR that is separate from that of the RG.

Figure 6.25.2-1. 5GC registration of AUN3 device

1. The 5G-RG registers to 5GC as specified in clause 7.2.1.1 of TS 23.316 [5].

2a. The AUN3 device connects to the 5G-RG with non-3GPP access network (e.g to establish WiFi connection).

2b. An authentication procedure is triggered (e.g.local authentication or 802.1x authentication). This can be done either by AUN3 device sending a EAPOL-start frame to 5G-RG or 5G-RG receives a frame from an unknown MAC address. 5G-RG receives a permanent identifier from the AUN3 device (e.g. an NAI in form of username@realm or a physical MAC address).

NOTE:	 how the 5G-RG is triggered to apply procedures for AUN3 devices is defined by BBF and/or CableLabs. Authentication procedures for AUN3 devices is to be defined by SA3.

NOTE: 	For FN-RG scenario, Step 2b involving the FN-RG and W-AGF is to be defined by BBF and/or CableLabs.

3. The 5G-RG, on behalf of the AUN3 devices, sends NAS Registration Request message to the AMF, including SUCI (or GUTI) of the AUN3 device and a device capability indicator that the device is AUN3. 5G-RG selects the serving AMF for the AUN3 device based on the received AN parameters and local policy as described in TS 23.316.

NOTE:	 Each AUN3 device has its UDM/UDR subscription. AUN3 PDU Session parameters are checked wrt AUN3 subscription.

4. AMF selects a suitable AUSF as specified in TS 23.501 [2] clause 6.3.4.

5. EAP based authentication defined in TS 33.501[X] is performed between the AUSF and AUN3 device.

NOTE: 	Each AUN3 device is registered to 5GC with its own unique SUPI.

6. The AMF performs other registration procedures as required (TS 23.502 [3] clause 4.2.2.2.2).

7. The AMF sends the Registration Accept message to the 5G-RG.

8. The 5G-RG sends the Registration Complete message back to the AMF, when the procedure is completed. The 5G-RG shall store the 5G-GUTI of AUN3 deviceto be able to use it potential later NAS procedures. The 5G-RG shall maintain a separate N2 connection for the 5G-RG and AUN3 device. Each AUN3 device has its own NAS connection (own NAS security context in the AMF and with the 5G-RG) and its own NGAP connection separate from that of the 5G-RG.

NOTE:	 Mobility restrictions as defined in TS 23.316[5] clause 4.3.3 are applicable to 5G-RG shall be applicable to the AUN3 device also.

9. The AMF performed step 23-24 in TS 23.501[2] clause 4.2.2.2.2.

10. The 5G-RG continues by requesting the establishment of PDU Session(s) on behalf of the AUN3 device. A distinct PDU session and QoS flows maybe established for each AUN3 device based on the subscription of the AUN3 device in the UDM/UDR. The PDU Session establishment/modification/release procedure is specified in TS 23.316[5] clause 7.3.1, 7.3.2, 7.3.3.

Distinct RG Level Wireline Access Characteristics (RG-LWAC), which provide appropriate mapping of QoS characteristics of the 5G QoS flows to the wireline technology specific QOS parameters to both the W-AGF and 5G-RG are configured for the 5G-RG and the AUN3 device and specified in TS 23.316[5] clause 4.5.1.2.

In case of FN-RG, Authentication, Registration and PDU session handling is performed by the W-AGF instead of the   5G-RG as specified in TS 23.316[5] clause 7.3.4, 7.3.6, 7.3.7.

### 6.25.3	Impacts on Existing Nodes and Functionality

The solution largely re-uses existing functionality.

5G-RG:

- The 5G-RG is configured to perform registration and PDU Session related procedures on behalf of the AUN3 device.

- Further considerations on 5G-RG would be brought to BBF and/or CableLabs attention is FFS.

W-AGF:

- In case of FN-RG, W-AGF is configured to perform registration and PDU session related procedures on behalf of the AUN3 device.

- The W-AGF supports multiple PDU sessions, at least one PDU session for each AUN3 device behind the FN-RG.

Editor’s Note: Further considerations on W-AGF would be brought to BBF and/or CableLabs attention is FFS.

## 6.X	Solution #X: <Solution Title>

### 6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub) clause(s) may be added to capture details.

### 6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.3	Impacts on Existing Nodes and Functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-02 | SA2#149E | S2-2200102 | - | - | - | Proposed skeleton approved at SA2#149E | 0.0.0 |
| 2022-03 | SA2#149E | - | - | - | - | Includes S2-2200104, S2-2201780, S2-2201781, S2-2200106, S2-2201784, S2-2201785, S2-2201786, S2-2201787, S2-2201782, S2-2201783, S2-2201791, S2-2201788, S2-2201789, S2-2201790 | 0.1.0 |
| 2022-03 | SA2#149E |  |  |  |  | Includes S2-2203503, S2-2203512, S2-2203510, S2-2203509, S2-2203511, S2-2203507, S2-2203508, S2-2203501, S2-2203505, S2-2203504, S2-2203506, S2-2202234, S2-2203513, S2-2203514, S2-2203502 | 0.2.0 |