SA WG2 Meeting #S2-151e(e-meeting)	S2-2203898r03

16 - 20 May 2022, Electronic Meeting	(revision of S2-2202218)

Source:	Ericsson, Deutsche Telekom, AT&T, Charter Communications Inc., MediaTek Inc., TMO USA Inc., Telstra, Apple, Verizon, Nokia, Nokia Shanghai Bell, DISH Network, Google, BT, CableLabs, Meta USA, Intel

Title:	KI#3, New sol: 5GS information exposure on XR/media Enhancements

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This contribution provides solution to enable codec/rate adaptation to meet requirements for services as described by KI#3.

# 1. Introduction

This paper proposes solution for KI#3 as agreed by SA2#149E:

“This Key Issue will study mechanisms that enable codec/rate adaptation to meet requirements for services.

-	Study the use cases and whether enhancements to the exposure framework are needed for such use cases;

-	What 5GS information needs to be exposed to enable application codec/rate adaptation;

-	How to expose 5GS information for application codec/rate adaptation.

NOTE 1:	Parameters for exposure may be coordinated with RAN and SA WG4.

NOTE 2:	Potential overlap with enhancements done in FS_EDGE_Ph2/FS_UPEAS should be considered.

”

# 2. Proposal

This paper proposes the following updates to TR 23.700-60 clause 6.

* Start of change *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[6]	3GPP TR 22.847: "Study on supporting tactile and multi-modality communication services; Stage 1".

[x]	RFC 3168: The Addition of Explicit Congestion Notification (ECN) to IP

[y]	RFC 8311: Relaxing Restrictions on Explicit Congestion Notification (ECN) Experimentation

* Next change *

## 6.X	Solution #X: Use of Relaxed ECN to enable codec/rate adaptation to meet requirements for services

### 6.X.1	Key Issue mapping

Editor's Note:	This clause lists the key issue(s) addressed by this solution.

This solution addresses KI#3.

### 6.X.2	Description

Editor's Note: This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

ECN as defined in RFC 3168[x], instead of dropping packets, uses link congestion indications and rate adjustments proportional to the congestion indications.

To further address service requirements, using of ECN marking of payload packets is further relaxed as specified in RFC 8311[7], the NG-RAN exposes current load level. Relaxed ECN marking interacts with the application layer, wherein the application layer triggers rate adaptation based on Relaxed ECN feedback. In this solution NG-RAN makes use of Relaxed ECN marking for both, DL and UL direction. . Enablement of L4S in a packet flow is based on e2e support for L4S.

The solution is based on following two components:

- - Use of Relaxed ECN marking in NG-RAN

- - Enablement of using Relaxed ECN

NOTE: This solution is applicable only in case UE and Application Server support Relaxed ECN defined in RFC 8311[x] and use protocols that support relaxed ECN feedback between UE and application server.

### 6.X.2.1 Use of Relaxed ECN marking in NG-RAN

It is assumed that the varying radio conditions and resource availability in NG-RAN are the main contributors to the need for the application to adapt its rate accordingly.

Given that it is NG-RAN that has the visibility of the resource availability and sudden changes on the radio interface that impact the performance in terms of latency, any fast reaction to trigger rate adaptation, that is required for services with tight latency requirements and benefit from bounded latency, must be triggered by NG-RAN.

NG-RAN makes use of Relaxed ECN marking of payload packets as specified in RFC 8311[7]. Such marking interacts with the application layer, wherein the application layer triggers rate adaptation based on feedback. In this solution NG-RAN makes use relaxed ECN marking for both, DL and UL direction.

NOTE: The criteria that RAN perform the marking is up to RAN implementation.

Editor’s Note: Whether RAN can support such a marking need to be coordinated with RAN WG.

### 6.X.2.2 Enablement of using Relaxed ECN

To enable Relaxed ECN, an existing or a separate QoS flow can be used for traffic supporting RFC 8311[x]. In context of this descriptive text for readability reasons, we refer to it as an‘Relaxed ECN QoS Flow’. It shall be noted that this solution does not propose to introduce a new concept and instead it reuses the 5QI framework. Namely, use of a certain QoS flow for Relaxed ECN traffic, enabling Relaxed ECN treatment in NG-RAN, is achieved via a preconfigured 5QI value. One or several 5QI(s) may be defined for this purpose by the operator in a deployment.

There are 3 main principles to establish a Relaxed ECN QoS flow within a PDU session:

-	Statically: At PDU session establishment the Relaxed ECN QoS flow is always established. This by either a pre-configured PCC rule if PCF is used, or by configuration in SMF.

NOTE: In this case, the configuration should ensure the application using the QoS Flow supports Relax ECN and uses one of the protocols that support Relaxed ECN feedback.

-	Dynamically based on 5GC configurations: SMF installs a Detection filter in UPF. This is either based on a request from PCF or from local configuration. The Detection filter can either use the ECN bits in the IP header or the XR server IP address(es). When Relaxed ECN traffic is detected, the UPF notifies SMF. If PCF is used, SMF notifies PCF which triggers a Relaxed ECN QoS flow establishment, or if PCF is not used, SMF is configured to establish a Relaxed ECN QoS flow.

Editor’s Note: How to differ the marking of ECN and Relaxed ECN bits is FFS.

-	Dynamically based on AF request: The AF makes an authorization request to activate the Relax ECN for a specific traffic flow either via NEF (untrusted) or directly to PCF, which triggers a Session modification procedure as per clause 4.3.3 in TS 23.502 [3].

The filters provided by SMF in the QoS rule to the UE and PDR to the UPF to identify traffic to be routed onto the Relaxed ECN QoS flows can be a combination of existing filters and/or the ECN bits in the IP packet header of the XR service.

Editor's Note: Other means to enable of Relaxed ECN use are FFS.

### 6.X.3	Procedures

Editor's Note: This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts on services, entities and interfaces

Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

NG-RAN: support for Relaxed ECN marking of payload packets as specified in RFC 8311[7].

Editor's Note: Impacts to other NFs, including e.g. SMF, UPF, AF, NEF, PCF, are FFS.

* End of changes *