3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2205717r03

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluation and conclusion for KI#2

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract: It is proposed to have evaluation and conclusion for KI#2.

# 1. Introduction/Discussion

There are 13 solutions for KI#2. This KI relates to:

- Whether and how the 5GC can be made aware whether or when the UE enforces a URSP rule to route an application traffic to a PDU Session based on the URSP rule provisioned by 5GC.

- Whether there are any actions the 5GS can take after 5GC is aware whether the UE enforces a URSP rule for specific application traffic or not. If any, what action 5GC should take?

- User privacy needs to be considered when defining solution (e.g. 5GC will know if a user started/stopped "any" application).

The details are shown in Table 1-1.

| Solutions | Title | Packet detection | Authorization by CP | URSP enforcement feedback |
| --- | --- | --- | --- | --- |
| 7 | URSP Compliance Verification |  | X |  |
| 8 | URSP Rule Precedence reporting for awareness of URSP enforcement | X |  |  |
| 9 | Per-PDU session awareness of URSP enforcement | X |  |  |
| 10 | Network based URSP rules enforcement feedback | X |  |  |
| 11 | 5GC awareness of URSP recognition by UE |  |  | X |
| 12 | URSP rule enforcement validation via gating control | X |  |  |
| 13 | URSP enforcement via PDU Session authorization/authentication when UE associating with application |  | X |  |
| 14 | 5GC awareness of URSP rule evaluation and verification of network slice usage |  | X |  |
| 15 | UE URSP enforcement validation by the network |  | X |  |
| 30 | Leveraging NWDAF to determine UEs that enforce URSP rules incorrectly | X |  |  |
| 31 | New URSP Notification Component |  |  | X |
| 32 | Application Detection using Domain Descriptor | X |  |  |

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-85.

# * * * * First change * * * *

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.x	Evaluation on Key Issue #2: 5GC awareness of URSP enforcement

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.x	Interim Conclusion on Key Issue #2: 5GC awareness of URSP enforcement

It is agreed to adopt the following interim conclusions for each of the KI aspects:

### 8.X.1 How the 5GC can be made aware whether or when the UE enforces a URSP rule to route an application traffic to a PDU Session based on the URSP rule provisioned by 5GC

-  When a UE receives the UE Policy container from the PCF, the UE may evaluate each URSP rule and include additional information about the URSP rules not supported by the UE in the result in step 4 of clause 4.2.4.3 of TS 23.502 [3].

-

The PCF can update the URSP rules based on the UE’s feedback.

Editor’s note: Whether “PCF” refers to SM-PCF, UE-PCF, or both is FFS

Editor’s note: Further conclusions are FFS

### 8.X.2 Whether there are any actions the 5GS can take after 5GC is aware whether the UE enforces a URSP rule for specific application traffic or not. If any, what action 5GC should take

### Editor’s note: Actions the 5GS can take are FFS

# * * * * End of changes * * * *