SA WG2 Meeting #152e	S2-2205919r08

# August 17th – August 26th ; Elbonia               			(revision of S2-220)

Source: 	Intel, AT&T, Deutsche Telekom

Title: 	KI#1: Conclusion

Document for: 	Approval

Agenda Item: 	9.1

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution:

# 1 Discussion

# 2 Proposal

It is proposed to update TR 23.700-18 as follows.

Start of change

# 8	Conclusions

## 8.1   Key Issue #1: Traffic Steering Policy and SFC Enhancements

It is recommended to use the following principles and procedures as the basis for the normative work.

-

-

-	Support the N6-LAN traffic steering control and AF-influenced traffic steering control to be applicable to the same traffic simultaneously.

-	5GC may receive from the AF policies associating for one, a group or all UE(s) some traffic (filter) with a pair of SFC IDs (one per direction) together with optional metadata to send to the SFC once the corresponding traffic has been identified.

Editor’s Notes: it is still FFS whether metadata are supported.

-	The PCF determines a policy per SDF/application for the purpose of steering the subscriber's traffic to appropriated N6 service functions deployed by the operator or a 3rd party service provider. The policy is expressed either

-  in a pair of Traffic Steering Policy (TSP) IDs that may be different in UL and DL directions, or

-  in a pair of Service Function Path (SFP) IDs that may be different in UL and DL directions.

-	NEF checks whether the indicated SFC IDs and Metadata correspond to an authorized SFC policy for the AF. The NEF may need to parse or interpret the Metadata e.g. to check whether the requested Metadata is on the agreed SLA list, but 5GC CP does not make any other use of the Metadata apart from forwarding them via the PCF/SMF to the UPF implementing the SFC functionality.

Editor’s Note: It is FFS whether NEF or PCF performs the check whether the indicated SFC IDs and Metadata correspond to the agreed SLA list.

-

-

-	The PCF maps the SFC IDs into either:

-  a pair of TSP IDs (one per direction) that refers to a traffic steering behaviour that is configured in the SMF/UPF. The SMF/UPF don’t need to be aware of SFC IDs, or

-  a pair of SFP IDs (one per direction) that refers to a specific Service Function Path in the UPF implementing the SFC functionality. The SMF/UPF don’t need to be aware of SFC IDs.

-	The PCF provides the TSP IDs or SFP IDs in the PCC rules to SMF.

-	The SMF provisions corresponding PDRs, FARs, QERs etc to support SFC by either:

-  creating a FAR with the Forwarding Policy parameters set to the TSP ID. The UPF serving as PSA uses the TSP ID to steer traffic over N6 towards the node implementing SFC functionality. In this case, the TSP ID identifies a specific Service Function Path in the SFC. It is assumed that all UPFs in the operator network serving as PSA for the DNN/S-NSSAI/DNAI subject to SFC control need to be configured with the same traffic steering information for SFC processing, or

-  creating a FAR with the Forwarding Policy parameters set to the SFP ID. In this case, the SFP ID identifies a specific Service Function Path in the UPF implementing the SFC functionality. In this case, the UPF PSA is not impacted.

Editor’s Notes: if metadata are supported then PCC rules and FAR are modified to support the (transparent) transfer of metadata; In the deployment option relying on TSP IDs where SFC functionality is implemented in a node residing on N6, it is FFS how the metadata is conveyed between the PSA and the node implementing SFC functionality.

-

-

-

-

-