SA WG2 Meeting #152e	S2-2206603r04

# August 17th – 26th, 2022; Elbonia                  	       (revision of S2-22xxxxx)

Source: 	vivo

Title: 	FS_eNPN_Ph2 KI#4 New Sol: Transferring hosting network selection information from hosting network to UE’s home network

Document for: 	Approval

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This contribution proposes a new Solution to KI#4.

# 1	Discussion

As a possible deployment scenario, the UE’s home network has business relationship with hosting network, but not with the Localized service provider. The hosting network and the Localized service provider are two parties. Then when the UE establishes business relationship with the Localized service provider, e.g. buying tickets, then the Localized service provider could notify the Hosting network and then trigger the Hosting network to send hosting network selection and access information to the UE.

This solution is applicable to the scenario where the hosting network is an SNPN or a PNI-NPN.

# 2 Proposal

It is proposed to update TR 23.700-08 as follows.

# 6.X	Solution #X: Solution for Transferring hosting network selection information from hosting network to UE’s home network

### 6.X.1	Introduction

This solution mainly addresses Key Issue #4 and focuses on this scenario: the UE’s home network has business relationship with hosting network, but not with the Localized service provider.

This solution is applicable to the scenario where the hosting network is an SNPN or a PNI-NPN.

### 6.X.2	Functional Description

In this solution, it is proposed that when the UE establishes business relationship with the Localized service provider, e.g. buying tickets, then the Localized service provider could notify the Hosting network and then trigger the Hosting network to send hosting network selection and access information to the UE via the UE’s home network.

The Hosting Network Selection and Access Information consists the following information:

-	Hosting Network Identifier, e.g. SNPN ID in case of the hosting network is an SNPN, CAG ID in case of the hosting network is a PNI-NPN.

-	The time condition information when the hosting network provides access service.

-	The location condition information where the hosting network provides access service.

The UE receives the Hosting Network Selection and Access Information from the localized service provider. The UE selects a hosting network when the time and location condition information is satisfied.

### 6.X.3	Procedures

Figure 6.X.3-1: Hosting network selection and access information transfer from hosting network to UE’s home network

0.	The UE establishes business relationship with the Localized service provider.

NOTE 1:	This step may be out of 3GPP scope, e.g. the user buys a ticket for a football match.

1.	The Hosting network AF may subscribe Localized service registration notification towards the UDR in the Hosting network.

NOTE 2:	This step is optionally performed. Otherwise, the UDR can be configured to report of localized service registration to the AF.

2.	The AF of Localized service provider sends Notification of Localized service registration to the UDR of Hosting network via NEF of Hosting network. Notification of Localized service registration describes the UE is successfully perform registration to the Localized service. Localized service ID and UE ID (e.g. UE’s phone number) are included in this notification message.

3.	The UDR in the Hosting network sends Report of Localized service registration to the AF of the Hosting network.

The UDR may also include DNN and S-NSSAI in Hosting network corresponding to the Localized service.

4. The AF of the Hosting network sends Hosting network selection and access information to the UDM of the UE’s home network via NEF of the home network. UE ID is also included in the message.

DNN and S-NSSAI in Hosting network corresponding to the Localized service is also included in the message if received in step3.

NOTE 3:	The AF of the Hosting network can use the UE’s phone number to determine UE’s home network.

Editor’s note: whether message 4 is a message via NEF or BSS communication is FFS as intention is to make an update of the subscription.

5. The UDM of the UE’s home network sends the hosting network selection and access information to the UE using SoR or UPU procedure.

The UDM may update the UE’s subscription to allow the UE to use DNN and S-NSSAI in Hosting network when the UE wants to use the Localized service.

### 6.X.4	Impacts on services, entities, and interfaces

UE impact:

-	Ability to receive Hosting Network Selection and Access Information via SoR or user parameter update procedure.

Home Network impact:

-	UDM: Ability to receive Hosting Network Selection and Access Information from the AF of the Hosting network and sends Hosting Network Selection and Access Information to the UE using SoR procedure or user parameter update procedure.

-	NEF: Ability to transfer Hosting Network Selection and Access Information from the AF in the Hosting network to the UDM.

Hosting Network impact:

-	AF: Ability to receive Hosting Network Selection and Access Information from the UDM of the hosting network and send the information to the UDM in Home network. Optionally, ability to subscribe report of Localized service registration for UEs. Hosting Network Selection and Access Information

-	UDR: Ability to send report of Localized service registration to AF of Hosting network and send Hosting Network Selection and Access Information to the AF of the Hosting network.

Localized service provider Network impact:

-	AF: Ability to send notification of localized service registration to Hosting Network.

************************** END OF CHANGE ***********************