SA WG2 Meeting #152e	S2-2206829r05

# August 17th – August 26th ; Elbonia               			(revision of S2-220)

Source: 	Intel, vivo

Title: 	KI#5 Interim Conclusion

Document for: 	Approval

Agenda Item: 	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution:

1 Discussion

This pCR provides the interim conclusion proposal for key issue #5. Based on the evaluation of solutions provided in S2-2206828, it is proposed to agree the interim conclusion for KI#5 as provided below.

2 Proposal

It is proposed to update TR 23.700-81 as follows.

Start of change

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

8.X   KI#5: Enhance trained ML model sharing

Interim Conclusion:

It is recommended to use the following principles and procedures as the basis for the normative work.

-	The following are the trained ML model sharing parameters to enable ML model sharing between different vendors - Interoperability indicator as specified in Sol#15 and optional Interoperability token as specified in Sol#47.

-  The Interoperability indicator indicates a list of AnLF providers (vendors) that are allowed to retrieve ML models from the MTLF;

-  The Interoperability token maps to an interoperable model information, e.g.  file format, platform, whose encoding or format is up to vendors’ implementation.

-	During registration of NWDAF containing MTLF with NRF, Interoperability indicator and optional Interoperability token to trained ML model sharing are provided as part of NF profile to the NRF.

-	During NWDAF containing MTLF discovery procedure, the consumer (NWDAF containing AnLF) may include Interoperability indicator and optional Interoperability token per Analytics ID(s) in the discovery request.

NOTE: When ML model containing MTLF and NWDAF containing AnLF belong to different vendors, how the ML model privacy is supported is in scope of SA3.

Editor's note:	Whether others parameters (e.g. ML model file format and/or AI Framework information) to enable ML model sharing between different vendors is FFS.

End change