3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208615

Elbonia, Oct 10th – 17th, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	Conclusion for FS_GMEC KI#5

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing Conclusion for KI#5.

1. Introduction/Discussion

Based on offline email discussions at SA2#152E, NOTEs should be updated in the conclusion for KI#5.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-74.

# * * * * First change * * * *

8.5	Key Issue #5: Allowing UE to simultaneously send data to different groups with different QoS policy

In the case when Application is capable to replicate multiple copies of the data, the following are ways forward:

-	If different groups (IP/Ethernet multicast groups) are associated to the same DNN and S-NSSAI combination used for 5G VN group, then different QoS Flows of single PDU Session may be used to transfer the data copy sent to different groups

-	If different groups (IP/Ethernet multicast groups) are associated to the different DNN and S-NSSAI combinations used for 5G VN group, then different PDU Sessions are used to transfer the data copy sent to different groups

In the case when Application is not capable to replicate multiple copies of the data, the following are ways forward.

-	UE establishes a PDU Session to a DNN/S-NSSAI, as per R17 specifications. This can be a special DNN/S-NSSAI configured by the operator for e.g. an electrical system.

-	Each group and group combination is associated with a separate multicast address.

-	Application sends traffic to a multicast address depending on what group(s) it wants to target. This allows an application to send a single packet reaching multiple destinations and also multiple groups.

-	The network configures a QoS policy for each UE where a multicast address is associated with a QoS level. The QoS level is set according to the QoS requirements for the group(s) the multicast address represents. Corresponding QoS Flow(s) is activated on each UE's PDU Session, as needed.

-	Application sends one UL copy.

NOTE 1:	No normative changes will result from this conclusion.

NOTE 2: The outcome of KI#1, KI#2, KI#3 are assumed to apply also if a UE is a member of multiple different groups.

# * * * * End of changes * * * *