SA WG2 Meeting #153E	S2-2208460r01

# 10 - 17 October, 2022, Electronic meeting		       (revision of S2-220xxxx)

Source: 	Charter Communications, Nokia, Nokia Shanghai Bell, Lenovo, Motorola Mobility, Broadcom, Apple, CableLabs

Title:	KI#5 Conclusion (clause 8.3), addressing the 1st EN in clause 8.3

Document for: 	Approval

Agenda Item: 	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes a resolution to the first EN in clause 8.3 Conclusions for KI#5, resulting in removal of the EN.

- Introduction/Discussion

This document addresses the 1st editor's note related to whether a new registration type or a new indication for non-3GPP path switching is used during the Registration procedure:

1)	The path switching between non-3GPP accesses is performed during the Registration procedure. The UE sends a Registration Request via the target non-3GPP access network and requests user-plane establishment using the "PDU Sessions to be activated" parameter.

Editor’s note:	It is FFS whether a new registration type (e.g., Registration type = non-3GPP path switching) or a new indication shall be used.

Discussion:

The following options for the Registration Type were discussed during the pre-SA2#153e conference call held on September 23, 2022:

- Option #1:  introduce new Registration type value of "Non-3GPP path switching"

- Option #2:  use existing Registration type value of "Mobility Registration Update" and introduce a new parameter indicating "Non-3GPP path switching"

Either option seemed acceptable to the participants of the conference call, but there was a slight preference for Option #2. Therefore this paper recommends deleting the EN and recommending Option #2.

Proposal:

It is proposed to remove the EN and update the conclusion text as per Option #2.

2. Text Proposal

It is proposed to capture the following changes into the TR 23.700-53 v1.0.0.

# * * * * First change * * * *

8.3	Conclusions for KI #5: Switching traffic of an MA PDU Session between two non-3GPP access paths

The solution to be specified in the normative phase shall support the following principles:

1)	The path switching between non-3GPP accesses is performed during the Registration procedure. The UE sends a Registration Request message with the Registration type value set to “Mobility Registration Update” and optionally a new parameter indicating “Non-3GPP path switching” via the target non-3GPP access network and also requests user-plane establishment using the "PDU Sessions to be activated" parameter.

2)	The AMF updates the registration for non-3GPP access in UDM after the data traffic is switched to the new access.

3) 	Path switching between any two non-3GPP accesses shall be supported, (e.g., trusted to trusted, trusted to non-trusted, trusted to wireline, or any permutation)

4) 	Capability exchange between UE and network shall be performed.

Editor’s note:  It is FFS whether this capability exchange is performed at 5G Mobility Management (5GMM) signaling (Registration procedure) or 5G Session Management (5GSM) signaling (i.e., PDU Session establishment procedure) or both.

# * * * * End of changes * * * *