3GPP SA WG2 Meeting #153E	S2-2209061r03

e-meeting, October 10 – 17, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#4: Evaluation and Conclusion for DCCF / MFAF relocation

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_PH3/Rel-18

Abstract of the contribution: This paper proposes update to the evaluation and conclusion for KI#4, Category-2 in TR 23.700-81.

Discussion

23.700-81 clause 7.4 evaluates in “category-2”, methods for DCCF and MFAF relocation. This contribution corrects assertion in the analysis, adds additional insights to the evaluation and proposes a conclusion aligned with the evaluation. The additional insights include:

- Analysis for solution #44 asserts: “Unlike NWDAF relocation, where the target NWDAF may need the same data and ML model from the source NWDAF in order to have consistent analytics performance, nothing is really needed by the target DCCF from the source DCCF”. However, when a DCCF processes notifications as defined in TS23.288 clause 5A.4, it maintains state information while notifications are processed to determine requested metrics. This is similar to the NWDAF maintaining state information while determining analytics, That information needs to be transferred to a target DCCF to prevent loss of data. For example, if processing entails determining the average and variance of a parameter over a period of interest (e.g. for Location Reports, the average and variance of the time interval between TA boundary crossings for the past 4 hours), the partially computed metric in a source DCCF needs to be sent to the target DCCF. Without this, information is lost and the consumer receives incorrect results.

- When an MFAF is deployed, the issue described above (in “1.”) also applies to MFAF relocation. Solution #44 does not support MFAF relocation, however the MFAF performs formatting and processing, and state information associated with partially computed metrics needs to be transferred from the source MFAF to a target MFAF or information will be lost and the consumer will receive incorrect results.

- While the Solution #44 procedure has fewer steps compared to Solution #12, it is not necessarily more message efficient. In Solution #44 ,when a UE moves to an area served by another DCCF, the entire procedure needs to be repeated for each consumer, including possible interactions with an NRF to discover a DCCF. In Solution #12 the procedure is executed once, there is a single interaction with the NRF and consumers are updated with a new Subscription Correlation ID in the last step.

- Solution #44 requires all DCCF Service Consumers (NWDAF, AMF, SMF, NEF, AF, etc.) to be upgraded to support reception of a notification from the DCCF that the UE can no longer be served, which subsequently triggers the Service Consumer to select a new DCCF, whereas Solution 12 impacts the DCCF and MFAF.

Proposal

It is proposed to add the following to TR 23.700-81.

*** Start of changes ***

8.4	Key Issue #4: How to Enhance Data collection and Storage

1.	For Managing Impact of storing data in NFp during muting, Solutions #41 and #45 are adopted for normative work.

2.	For ADRF / NWDAF Data Storage Management, ADRF is configured with operator policies for data storage as defined in Solution#46.

3.	For storing ML models in ADRF, MTLF can store ML model in ADRF based on MTLF policy. ADRF shall not duplicate the functionality provided by MLModelProvision Service.

Editor's note:	Detailed procedures, service operations and corresponding parameters on how MTLF stores the ML model in ADRF are FFS.

For DCCF relocation:

-	For data consumer or central DCCF reselecting target DCCF , Solution 44 and 66 is the baseline for normative work.

-	For source DCCF reselecting target DCCF, Solution 12 and 66 is the baseline for normative work.

NOTE 1:	Whether data consumer, source DCCF or central DCCF reselecting target DCCF, depends on deployments.

*** End of changes ***