SA WG2 Meeting #156E (e-meeting)	S2-230xxxx

April 17 – 21, 2023	revision of S2-230xxxx

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | x- | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unicast tunnel establishment and release for resource sharing across multiple broadcast MBS Sessions during network sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | April 7th, 2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Editor´s note mark additions for MBS tunnel management during MBS session establishment and release for resource sharing across multiple broadcast MBS Sessions during network sharing as FFS. It was agreed at SA2#155 to wait for a correction of the baseline procedures, and such a correction is now proposed in CR#xxx. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Procedures for unicast tunnel establishment and release for resource sharing across multiple broadcast MBS Sessions during network sharing are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures for unicast tunnel establishment and release for resource sharing across multiple broadcast MBS Sessions during network sharing are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1a, 7.3.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.3.1a	MBS Session Start for resource sharing across multiple broadcast MBS Sessions during network sharing

To support resource sharing across multiple broadcast MBS Sessions during network sharing (see clause 6.17), in addition to clause 7.3.1, the following enhancement applies:

-	Step 2: The MB-SMF includes Associated Session ID in the N2 SM information in Namf_MBSBroadcast_ContextCreate.

-	Step 4: A shared NG RAN node identifies associated MBS sessions via a configured association of TMGIs or via the same Associated Session Identifier being received for the associated sessions. The associated MBS sessions shares radio resource configuration based on the existing Broadcast MBS Session context.

-	Step 5: If the NG-RAN determines there is already an existing broadcast MBS session delivering the same content, the NG-RAN may skip joining the multicast group based on operator policy, i.e., step 5 can be skipped.

-	Step 6: If the NG-RAN determines there is already an existing broadcast MBS session delivering the same content, a shared NG-RAN node may not establish N3mb DL tunnel by not providing N3mb DL Tunnel Info based on operator policy.

-	Step 9: If the NG-RAN determines that radio resource has been allocated for another broadcast MBS Session delivering the same content based on the Associated Session Identifier or the configured association of TMGIs, the NG-RAN advertises the TMGI of the broadcast MBS session, and reuses such already allocated radio resource for the MBS data transmission for the broadcast MBS session.

NOTE:	The 5G QoS Profile values for the broadcast MBS sessions are assumed to be pre-agreed amongst the multiple operators participating in the network sharing, and are the same irrespective of the PLMN from which the MBS Session Start is sent.

-	Step 10 ~ step 12:

The handling is the same as steps 5, 6, 7 and 8.

-	Step 14: If the user plane of a broadcast MBS session from its MB-UPF to the shared NG-RAN is not established, the NG-RAN will not receive the MBS packets from the MB-UPF.

-	Step 15: If the NG-RAN receives multiple DL media streams for the same content represented by the same Associated Session ID via multiple CNs, the NG-RAN should deliver only one DL media stream.

### 7.3.2a	MBS Session Release for resource sharing across multiple broadcast MBS Sessions during network sharing

To support resource sharing across multiple broadcast MBS Sessions, in addition to clause 7.3.2, the following enhancement applies:

-	Step 6:

-	If the NG-RAN did not joining the multicast group (LL SSM) in clause 7.3.1a, the NG-RAN does not send a Leave message (LL SSM)..

-	If the NG RAN node did not establish an N3mb DL tunnel in clause 7.3.1a, the NG-RAN does not release the related resources.

-	If the NG-RAN joined the multicast group (LL SSM) or established an N3mb DL tunnel in clause 7.3.1a, and if the NG-RAN determines there are other associated broadcast MBS Sessions sharing the same resources, the NG-RAN may trigger procedure of Transport change for resource efficiency across broadcast MBS Sessions in network sharing as specified in clause 7.3.7.