3GPP TSG-WG SA2 Meeting #156-e	SP-230xxx

# April 17 - 21, 2023, Electronic

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | XXXX | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBSR Authorization Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  |  |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As detailed in the discussion paper, this CR proposes to use the UE Configuration Update procedure to inform the MBSR IAB-UE of changes in the MBSR authorization, and specifies the procedure for handing over UEs connected to an MBSR to other cells when said MBSR is no longer allowed to operate as such. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the EN related to handling of changes in MBSR authorization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR XXXX |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.35A.4	MBSR authorization

For a MBSR, the subscription information stored in the HPLMN indicates whether it is authorized to operate as MBSR, and the corresponding location and time periods.

Editor's note:	The subscription information for the MBSR will be further specified.

When MBSR roaming is supported, a roaming agreement between VPLMN and HPLMN regarding MBSR operation is in place, and the 5GC can make use of it for authorization of MBSR in VPLMN. MBSR (IAB-DU) can use IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to integrate into VPLMN to provide service.

The MBSR(IAB-UE) is assumed to be configured with preferred PLMN lists and forbidden PLMNs by the HPLMN for the MBSR operation.

Editor's note:	Configuration mechanism will be further described based on development of clause 5.35A.2.1.

When the MBSR (IAB-UE) performs initial registration with the serving PLMN, it indicates the request to operate as a MBSR as described in clause 5.35A.1. The AMF authorizes the MBSR based on the subscription information, and provides MBSR authorized indication to NG-RAN. The MBSR establishes the connection to OAM system using the configuration information for MBSR operation.

Editor's note:	It is FFS whether the MBSR indication is a new IE or part of UE 5G MM capabilities.

Editor's note:	Whether existing IAB-Operation allowed indication and IAB authorized indication can be reused for MBSR will be determined.

NOTE 1:	How the MBSR obtains the configuration information for MBSR operation is described in clause 5.35A.2.1.

The AMF of the MBSR can indicate to the MBSR IAB-UE that it is not allowed to act as an MBSR IAB node as part of registration procedure, and in this case the AMF does not include MBSR authorization indication to donor-gNB. The AMF may provide the indication either in a Registration Accept (if the PLMN allows the MBSR IAB-UE to be registered in the PLMM) or in a Registration Reject (if the PLMN does not allow the MBSR IAB-UE to be registered in the PLMN).

If the MBSR authorization changes due to updates in the subscription information, or for radio network optimization (e.g. temporarily disable MBSRs to avoid interference if a high number of MBSRs is currently operating in the area, or to optimize energy consumption depending on the number and distribution of MBSRs and UEs in the area), the AMF informs the NG-RAN of the authorization change and sends a UE Configuration Update to the MBSR IAB-UE. If the MBSR authorization is changing from authorized to un-authorized, the UEs currently connected to the MBSR are handed over to other cells as described in clause 5.35A.3.

NOTE 2:	The mechanism applies to both roaming and non-roaming MBSR operations.

# * * * * End of changes * * * *