3GPP TSG-WG SA2 Meeting #156E 	S2-2304013r01

# April 17-24, 2023	(revision of S2-230XXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1283 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting Network Centric AR Telephony Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-03-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | WID NG_RTC has objective to support AR telephony communication in IMS network, which has been concluded in TR 23.700-87. This CR reflects the TR conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause on network centric AR Architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause XX.2.2 (new), Clause XX.3.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.228 CR#1284 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Annex XX and its subclause structure is introduced in CR#1284 for this spec. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start  of changes * * * *

## AC.2.4	 AR Communication Architecture

This clause defines how IMS can be enhanced to provide AR communication service.

NOTE X: 	The term ‘AR communication’ includes many different combinations of use cases, device capabilities and AR media content for which network assisted functions can be provided.

Multiple realization alternatives for AR communication are available depending on device capability and what requirements the AR application has on the media transport protocol used between the UE and the network assisted AR function. AR services can be delivered via the following mechanisms:

- The UE-A and UE-B use an established application data channel to transport AR metadata and related media. This scenario represents a standard P2P data channel application as per clause AC 7.2 and consequently not requiring any further description.

- The AR application server provides network assisted rendering by having all AR media being transported using an established application data channel between the UE and the AR application server. This scenario represents a standard P2A/A2P data channel application as per clause AC.7.2 and consequently not requiring any further description.

- An UnifiedMedia Function (UMF)/Media Resource Function (MRF) having the Augmented Reality Media Function (ARMF)  capability, provides required network assisted AR media processing.

Figure XX.2.2-1 shows the architecture to support Network Assisted AR communication.

Figure XX.2.2-1: Architecture to support AR communication

NOTE X:	The ARMF capability can be provided by either a Unified Media Function or enhanced MRF.

The AR communication architecture is based on the data channel architecture in Annex AC.2.1, with the following additional network functions:

1.	The AR Application Server is responsible for AR service control related to AR communication, including AR session media control, AR media capability negotiation with the UE.

NOTE Y:	The AR Application Server is a specific DC Application Server and is out of scope of 3GPP.

2.	The Unified Media Function (UMF) or Media Resource Function (MRF) providing ARMF capability, is responsible for AR media processing.

The following reference points are used for IMS AR related signalling.

-	DC4: Reference point between the AR Application Server and the DCSF for AR service handling and AR session media control. This is out of scope of this Release.

-	DC2: Service based reference point between the IMS AS and the UMF for AR media resource management.

-	Mr/Cr: Reference point between the IMS AS and the MRF for AR media resource management.

The following reference points are used for IMS AR related media.

-	MDC2: Reference point between the AR Application Server and the UMF for transmission of application data channel traffic.

NOTE Z:	The MDC2 reference point is not specified in 3GPP.

-	Mb: Reference point used for IMS media transport and AR media transmission.

*****************Next Change************

## XX.3.2 Network Centric Procedure

Figure XX.3.2-1 depicts a typical call flow procedure to establish a network assisted AR IMS session from a UE-A perspective, being a device with limited AR capabilities. The main steps in the call flow are as follows:

1.	UE-A initiates an IMS communication with the UE-B, including establishment of bootstrap data channel connections.

2.	The user of UE-A upgrades the IMS session with AR experience. The DCSF/AR Application Server concludes that the UE has limited AR capabilities requiring network assisted media processing.
The downloaded AR application in UE-A, initiates a re-INVITE adding media descriptors required by the network assisted AR application. The DCSF requests IMS AS to have the added media descriptors anchored in the ARMF.

3.	AR data is in this scenario captured by both UE-A and UE-B.

4.	UE-A and UE-B send the captured AR media to the ARMF of the originating network.

5	The ARMF processes, renders and encodes received AR media on behalf of UE-A as per AR application instructions.

6	The rendered AR media is sent to UE-A.

7	UE-A displays the AR media to the user.

Figure XX.3.2.1: Establishment of NW Assisted AR communication session

# * * * * End of changes * * * *