# 3GPP TSG-SA WG2 Meeting #156E	S2-2304193r03

Electronic Meeting, 17 - 21 April, 2023	(Revision of S2-230xxxx)

Title:	[Draft]Reply LS on Network Triggered Service Request for a UE in Suspend State

Response to:	LS on Network Triggered Service Request for a UE in Suspend State (S2-2303941/C4-230664)

Release:	Rel-17

# Work Item:	     5G_CIoT, TEI17

Source:	SA2

To:	CT4

cc:	RAN3

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks CT4 for questions related to the handling of network triggered service request for UE in CM-IDLE with suspend. SA2 would like to provide following answers.

Question1: What information should the SMF include in the N2 Information for the N1N2MessageTransfer Request message?

SA2 answer: Two possible solutions have been identified by SA2. It would be possible to introduce a dummy N2 information if there is no N1 message to deliver, with CT4 to specify the dummy N2 message details.. The other alternative is to use Namf_MT_EnableUEReachability which would trigger the desired AMF behaviour.

Question2: Upon subsequent Connection Resume in CM-IDLE with Suspend procedure as response to the paging, how should the AMF deal with the N2 Information received in the N1N2MessageTransfer Request triggering the paging (the AMF is expected to send an N2 Connection Resume Response or N2 Path Switch Ask to NG-RAN, see clause 4.8.2.3 of 23.502)?

SA2 answer: If N1N2MessageTransfer with dummy N2 data is used, when the UE resumes (either via NGAP Connection Resume Request or Path Switch Request), the AMF follows the flow in clause 4.8.2.3 of 23.502. For the dummy N2 message alternative above, the dummy N2 message can be discarded by AMF. The dummy N2 message encoding is not relevant with Namf_MT_EnableUEReachability.

SA2 has not decided between these two alternatives and would like to ask for CT4 to specify the PDU and IE encoding as appropriate, and to inform SA2 of the outcome.

2. Actions:

To CT4

ACTION: 	SA2 kindly asks CT4 to choose between the above alternatives and to inform SA2 for specification alignment..

3. Date of Next SA2 Meeting:

SA2#157                   	May 22 - 26, 2023                                 	Berlin, DE

SA2#158   		August 21 - 25, 2023 				Goteborg, SE