3GPP TSG-WG SA2 Meeting #156-e	S2-2304887r03

# Elbonia, April 17 – 21, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4084 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the procedures for PDTQ policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The descriptions of PCF subscribes to NWDAF analytics should be updated based on the NOTE 3 in clause 6.1.2.7 of TS 23.503, i.e., whether the PCF subscribes to "Network Performance" analytics or "DN Performance" analytics is based on PCF configuration. The PDTQ data should be added to Policy Data as defined in Table 5.2.12.2.1-1, and the PDTQ policy can be used to the procedure of PCF initiated SM Policy Association Modification as described in clause 4.16.5.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the descriptions of PCF subscribes to NWDAF analytics. Add PDTQ data to Policy Data. Add the description that PCF makes a policy decision based on the PDTQ policy. Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to provide correct 5GC mechanism to support the negotiation for PDTQ policy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.5.2, 4.16.15.2.1, 4.16.15.2.2, 5.2.12.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.16.5.2	PCF initiated SM Policy Association Modification

The PCF may initiate SM Policy Association Modification procedure based on internal PCF event or triggered by other peers of the PCF (AF, NWDAF, CHF, UDR and TSCTSF).

Figure 4.16.5.2-1: PCF initiated SM Policy Association Modification

This procedure may be triggered by a local decision of the PCF or based on triggers from other peers of the PCF (AF, NWDAF, CHF, UDR and TSCTSF):

An SM Policy Association is established, with the PCF as described in clause 4.16.4 before this procedure is triggered.

For local breakout roaming, the interaction with HPLMN (e.g. step 1b and step 2) is not used. In local breakout roaming, the V-PCF interacts with the UDR of the VPLMN.

1a.	Alternatively, optionally, the AF, NEF or TSCTSF provides/revokes service information to the PCF e.g. due to AF session signalling, by invoking Npcf_PolicyAuthorization_Create Request or Npcf_PolicyAuthorization_Update Request or Npcf_PolicyAuthorization_Subscribe Request service operation. The PCF responds to the AF, NEF or TSCTSF.

1b.	Alternatively, optionally, the CHF provides a Spending Limit Report to the PCF as described in clause 4.16.8. and responds to the CHF.

1c.	Alternatively, optionally, the UDR notifies the PCF about a policy subscription change by invoking Nudr_DM_Notify (Notification correlation Id, Policy Data, SUPI, updated data, "PDU Session Policy Control Data" | "Remaining allowed Usage data"); The PCF responds to the UDR.

1d.	Alternatively, optionally, some internal event (e.g. timer, or local decision based on analytics information requested and received from NWDAF) occurs at the PCF. The analytics (i.e. Analytics ID) which can be requested from NWDAF are described in clause 6.1.1.3 of TS 23.503 [20].

2.	If the PCF determines a change to policy counter status reporting is required, it may alter the subscribed list of policy counters using the Initial, Intermediate or Final Spending Limit Report Retrieval procedures as defined in clause 4.16.8.

NOTE 1:	The PCF ensures that information received in step 1 and 2 can be used by later policy decisions.

NOTE 2:	For local breakout roaming, PDU Session policy control subscription information and Remaining allowed usage subscription information for monitoring control as defined in clause 6.2.1.3 of TS 23.503 [20] are not available in V-UDR and V-PCF uses locally configured information according to the roaming agreement with the HPLMN operator.

3.	The PCF makes a policy decision. The PCF may determine that updated or new policy information need to be sent to the SMF. In the non-roaming case, the PCF may also decide to subscribe to a new Analytics ID from NWDAF as described in clause 6.1.1.3 of TS 23.503 [20].

If the AF provided a Background Data Transfer Reference ID in step 1a, the PCF may retrieve it from the UDR by invoking the Nudr_DM_Query (BDT Reference ID, Policy Data, Background Data Transfer) service.

If the AF provided a PDTQ Reference ID in step 1a, the PCF may retrieve it from the UDR by invoking the Nudr_DM_Query (PDTQ Reference ID, Policy Data, Planned Data Transfer with QoS requirements) service.

4.	If the PCF has determined that SMF needs updated policy information in step 3 or if the PCF has received a Port Management Information Container for the PDU Session and related port number from the AF or TSCTSF in step 1a, the PCF issues a Npcf_SMPolicyControl_UpdateNotify request with possibly updated policy information about the PDU Session.

If the PCF has received a subscription for 5GS Bridge/Router information Notification in Step 1a, the PCF can include a subscription for SMF event for "5GS Bridge/Router information" associated with the PDU Session into the Npcf_SMPolicyControl_UpdateNotify request. In this case, if the SMF has stored the 5GS Bridge/Router information and has not reported the event to the PCF, the SMF notifies the PCF for the event of "5GS Bridge/Router Information ".

If the PCF has received a Npcf_PolicyAuthorization_Unsubscribe request to unsubscribe for 5GS Bridge/Router information Notification, the PCF can remove the subscription for SMF event for "5GS Bridge/Router information" associated with the PDU Session and issue a Npcf_SMPolicyControl_UpdateNotify request with the updated policy information about the PDU Session.

NOTE 3:	If the TSCTSF receives a Requested 5GS delay and the TSCTSF does not have the 5GS Bridge/Router information for the AF-session, the TSCTSF can subscribe for the 5GS Bridge/Router information from the PCF by triggering a Npcf_PolicyAuthorization_Subscribe request.

If the PCF has received a subscription to notification on BAT offset along with the TSC Assistance Container from TSCTSF in step 1a, the PCF can include a subscription to notification on BAT offset associated with the PDU Session into the Npcf_SMPolicyControl_UpdateNotify request.

5.	The SMF acknowledges the PCF request with a Npcf_SMPolicyControl_UpdateNotify response.

If the Npcf_SMPolicyControl_UpdateNotify request is received from new PCF instance in the PCF Set, the SMF store the SM policy association towards the new PCF instance.

# * * * * Next change * * * *

4.16.15.2.1	Procedures for negotiation of planned data transfer with QoS requirements

This clause describes the PDTQ procedures to negotiate viable time window for the planned application data transfer via the support of the NEF.

UDR

NWDAF

H-PCF

NEF

AF

1a. Nnef_PDTQPolicyNegotiation_Create Request

1b. Authorized AF’s PDTQ Negotiation Request

1c. Nnef_PDTQPolicyNegotiation_Create Response

2. Npcf_PDTQPolicyControl_Create Request

3. Nudr_DM_Query Request

4. Nudr_DM_Query Response

7. Npcf_PDTQPolicyControl_Create Response

8. Nnef_PDTQPolicyNegotiation_Create Response

9. Nnef_PDTQPolicyNegotiation_Update Request

10. Npcf_PDTQPolicyControl_Update Request

11. Npcf_PDTQPolicyControl_Update Response

12. Nnef_PDTQPolicyNegotiation_Update Response

13. Nudr_DM_Update Request

14. Nudr_DM_Update Response

AF discovers its serving NEF (TS 23.501, clause 6.3.14)

5. Network or DN Performance Analytics as described in TS 23.288 to determine the proper QoS policies and transmission time window(s) based on specific info provided by the AF.

6.  PDTQ policy determination

Figure 4.16.15.2.1-1: Negotiation for planned data transfer with QoS requirements

Prior to the transport of the Application AI/ML data, the AF negotiates with the 5G Core for the PDTQ policies to provide assistance for its application data transfer. The AF discovers its serving NEF, if it has not done so, using the mechanism as described in clause 6.3.14 of TS 23.501 [2].

1a.	The AF invokes the Nnef_PDTQPolicyNegotiation_Create Request (ASP Identifier, Number of UEs, list of Desired time windows, QoS Reference or individual QoS parameters, Alternative Service Requirements (optional), Network Area Information, Request for notification). The Request for notification is an indication that PDTQ warning notification can be sent to the AF.

NOTE:	Based on AF's internal logic (policy), the AF may determine the minimum QoS requirements based on the UEs who are expected to participate in the PDTQ transfer window and the network input data and the group application data transfer trigger conditions.

1b-1c.	The NEF may authenticate the AF and authorize the PDTQ request from the AF. If the authentication/authorization of the AF's request has failed, the NEF will respond to the AF's request through the Nnef_PDTQPolicyNegotiation_Create Response with a failure result, and the following steps are skipped.

2.	Based on an AF request, the NEF may translate the information provided by the AF (e.g. Network Area Information etc.) based on the local policy and invokes the Npcf_PDTQPolicyControl_Create (ASP Identifier, Number of UEs, list of Desired time windows, QoS Reference or individual QoS parameters, Alternative Service Requirements (optional), Network Area Information, Request for notification) with the H-PCF to authorize the creation of the policy regarding the PDTQ. If the PCF was provided with Request for notification, then PCF will send PDTQ warning notification to the AF as specified in clause 4.16.15.2.2 to notify the AF when the network performance or DN Performance in the area of interest goes below the Reporting Threshold set by the PCF based on operator configuration as described in clause 6.1.2.7 of TS 23.503 [20].

3.	H-PCF queries the UDR to retrieve all existing PDTQ polices for all the ASPs using Nudr_DM_Query (Policy Data, Planned Data Transfer with QoS requirements) service operation.

4.	The UDR provides all the stored PDTQ policies and corresponding related information (e.g. the Number of UEs, the list of Desired time windows) to the H-PCF.

5.	Based on information provided by the AF and other available information, the H-PCF queries or/and subscribes to the NWDAF as defined in clause 6.6.4 or clause 6.14.4 of TS 23.288 [50] to request the Network Performance analytics or the DN Performance analytics.

When requesting the Network Performance analytics or the DN performance analytics, if "any UE" is used, then the AoI information is used to identify the target gNB(s) for the prediction of the availability of the network resources.

6.	By referring to the outcome of the analytics report as described in clause 6.1.2.7 of TS 23.503 [20], H-PCF determines one or more PDTQ policies. Each PDTQ policy includes a recommended time window for the traffic transfer for each of the AF session for each of the UEs involved.

7.	The PCF sends one or more PDTQ policies to NEF in Npcf_PDTQPolicyControl_Create Response including the PDTQ Reference ID.

8.	The NEF sends a Nnef_PDTQPolicyNegotiation_Create response to the AF to provide one or more PDTQ policies together with the PDTQ Reference ID. If the NEF received only one PDTQ policy from the PCF, steps 9-12 are not executed and the flow proceeds to step 13. Otherwise, the flow proceeds to step 9.

9.	If more than one PDTQ policies were provided to the AF, the AF selects one of the PDTQ policies and notifies NEF for the selected PDTQ policy via Nnef_PDTQPolicyNegotiation_Update together with the PDTQ Reference ID. The AF stores the PDTQ Reference ID for the future interaction with the PCF.

10-12.	The NEF notifies H-PCF about the selected PDTQ policy by the AF. The H-PCF acknowledges NEF. The NEF responds to the AF request with a Nnef_PDTQPolicyNegotiation_Update Response.

13-14.	The H-PCF stores the PDTQ Reference ID together with the new PDTQ policy in the UDR by invoking Nudr_DM_Update (PDTQ Reference ID, Policy Data, Planned Data Transfer with QoS requirements). The UDR sends a response to the H-PCF as acknowledgement.

# * * * * Next change * * * *

4.16.15.2.2	Procedure for PDTQ warning notification

UDR

H-PCF

NEF

AF

3. Nudr_DM_Query Request

4. Nudr_DM_Query Response

9. Npcf_PDTQPolicyControl_Notify

10. Nnef_PDTQPolicyNegotiation_Notify

14. Nudr_DM_Delete Request

15. Nudr_DM_Delete Response

6. Identifying new candidate PDTQ policies

11. AF checks PDTQ policies in the candidate list

1. Negotiation for PDTQ policies is complete

2. Network performance has degraded

5. Identifying affected PDTQ policies

7. Nudr_DM_Update Request

8. Nudr_DM_Update Response

12. Step 9-14 from section 4.16.15.2.1 are executed

13. Step 9-12 from section 4.16.15.2.1 are executed

Alternative A: AF selects a PDTQ policy from the candidate list

Alternative B: AF does not select any PDTQ policy from the candidate list

Figure 4.16.15.2.2-1: The procedure for PDTQ warning notification

1.	The negotiation for PDTQ policy as described in clause 4.16.15.2.1 is completed. In addition, the PCF has subscribed to analytics on "Network Performance" or "DN Performance" from NWDAF for the area of interest and time window of a PDTQ policy following the procedure and services described in TS 23.288 [50], including a Reporting Threshold in the Analytics Reporting information. The value for Reporting Threshold is set by the PCF based on operator configuration.

2.	The PCF is notified with the Network Performance analytics or DN Performance analytics in the area of interest from the NWDAF when the NWDAF determines that the Network Performance or DN Performance goes below the Reporting Threshold as described for the Network Performance analytics or DN Performance analytics in TS 23.288 [50].

3.	The H-PCF requests from the UDR the stored PDTQ policies using Nudr_DM_Query (Policy Data, Planned Data Transfer with QoS requirements) service operation.

4.	The UDR provides all the PDTQ policies together with the relevant information received from the AFs (as defined in clause 6.1.2.7 of TS 23.503 [20]) to the H-PCF.

5.	The H-PCF identifies the PDTQ policies affected by the notification received from NWDAF. For each of them, the H-PCF determines the ASP of which the PDTQ traffic will be influenced by the degradation of network Performance or DN Performance and which requested the H-PCF to send the notification. The PCF then performs the following steps for each of the determined ASPs, i.e. Steps 6 - 16 can occur multiple times (i.e. once per ASP).

6.	The PCF decides based on operator policies, whether a new list of candidate PDTQ policies can be calculated for the ASP. If the PCF does not find any new candidate PDTQ policy, the previously negotiated PDTQ policy shall be kept, no interaction with that ASP shall occur and the procedure stops for that PDTQ policy.

NOTE 1:	The PDTQ policies of an ASP which did not request to be notified are kept and no interaction with this ASP occurs.

7.	The PCF sets the no longer valid PDTQ policy in the UDR as invalidated by invoking Nudr_DM_Update (PDTQ Reference ID, invalidation flag) service.

8.	The UDR sends a response to the H-PCF as acknowledgement.

9.	The PCF sends the notification to the NEF by invoking Npcf_PolicyControl_Notify (PDTQ Reference ID, list of candidate PDTQ policies) service operation.

10.	The NEF sends the PDTQ warning notification to the AF by invoking Nnef_PDTQPolicyNegotiation_Notify (PDTQ Reference ID, list of candidate PDTQ policies) service operation.

11.	The AF checks the new PDTQ policies included in the candidate list in the PDTQ warning notification.

12.	If the AF selects any of the new PDTQ policies, the steps 9-14 from clause 4.16.15.2.1 are executed.

13.	If the AF doesn't select any of the new PDTQ policies, the steps 9-12 from clause 4.16.15.2.1 are executed, with the AF indicating that none of the candidate PDTQ policies is acceptable.

14-15.	If the step 13 is executed, the PCF removes the no longer valid PDTQ policy from UDR for the corresponding PDTQ Reference ID.

NOTE 2:	The PCF can also remove the no longer valid PDTQ policy after an operator configurable time for the case that the AF does not respond.

The AF can send a Stop notification by invoking Nnef_PDTQPolicyNegotiation_Update service, when the AF requests not to receive the PDTQ warning notification anymore. Then, the NEF invokes Npcf_PDTQPolicyControl_Update service in order to provide this information for the H-PCF.

# * * * * Next change * * * *

5.2.12.2.1	General

The operations defined for Nudr_DM service use following set of parameters defined in this clause:

-	Data Set Identifier: uniquely identifies the requested set of data within the UDR (see clause 4.2.5).

-	Data Subset Identifier: it uniquely identifies the data subset within each Data Set Identifier. As specified in the procedures in clause 4, e.g. subscription data can consist of subsets particularised for specific procedures like mobility, session, etc.

-	Data Keys defined in Table 5.2.12.2.1-1

For Nudr_DM_Subscribe and Nudr_DM_Notify operations:

-	The Target of Event Reporting is made up of a Data Key and possibly a Data Sub Key both defined in Table 5.2.12.2.1-1. When a Data Sub Key is defined in the table but not present in the Nudr_DM_Subscribe this means that all values of the Data Sub Key are targeted.

-	The Data Set Identifier plus (if present) the (set of) Data Subset Identifier(s) corresponds to a (set of) Event ID(s) as defined in clause 4.15.1

An NF Service Consumer may include an indicator when it invokes Nudr_DM Query/Create/Update service operation to subscribe the changes of the data, to avoid a separate Nudr_DM_Subscribe service operation.

Depending on the use case, it is possible to use a Data Key and/or one or multiple Data sub keys to further identify the corresponding data, as defined in Table 5.2.12.2.1-1 below.

Table 5.2.12.2.1-1: Data keys

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
|  | Access and Mobility Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMF Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | UE context in SMF data | SUPI | PDU Session ID or DNN |
| Subscription Data (see clause 5.2.3.3.1) | SMS Management Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMS Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Session Management Subscription data | SUPI | S-NSSAI |
|  |  |  | DNN |
|  |  |  | Serving PLMN ID and optionally NID |
|  | Slice Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Group Data (NOTE 5) | Internal Group Identifier or External Group Identifier | - |
|  | Identifier translation | GPSI |  |
|  |  | SUPI | Application Port ID, MTC Provider Information, AF Identifier |
|  | Intersystem continuity Context | SUPI | DNN |
|  | LCS privacy | SUPI | - |
|  | LCS mobile origination | SUPI | - |
|  | UE reachability | SUPI | - |
|  | Group Identifier Translation | Internal Group Identifier or External Group Identifier | - |
|  | UE context in SMSF data | SUPI | - |
|  | V2X Subscription data | SUPI | - |
|  | A2X Subscription data | SUPI | - |
|  | ProSe Subscription data | SUPI | - |
|  | User consent | SUPI | Purpose |
|  | ECS Address Configuration Information (See Table 4.15.6.3d-1) | SUPI, Internal group identifier or external group identifier or any UE | DNN, S-NSSAI, (Serving) PLMN ID (NOTE 7) |
|  | MBS Subscription data | SUPI | - |
|  | Shared data | Shared Data ID | - |
| Application data | Packet Flow Descriptions (PFDs) | Application Identifier | - |
|  | AF traffic influence request information for traffic routing | AF transaction internal ID |  |
|  | (See clause 5.6.7 and clause 6.3.7.2 of TS 23.501 [2]) | S-NSSAI and DNN and/or Internal Group Identifier(s) and/or Subscriber Category(s) or SUPI or "any UE" indication (NOTE 4) (NOTE 6) |  |
|  | AF traffic influence request information for service function chaining | AF transaction internal ID |  |
|  | (See clause 5.6.16 and clause 6.3.7.2 of TS 23.501 [2]) | S-NSSAI and DNN and/or Internal Group Identifier or SUPI or "any UE" indication (NOTE 4) |  |
|  | Background Data Transfer (NOTE 3) | Internal Group Identifier or SUPI |  |
|  | Service specific information (See clause 4.15.6.7) | S-NSSAI and DNN or Internal Group Identifier or SUPI or "any UE" indication (NOTE 4) or "PLMN ID(s) of inbound roamer" |  |
|  | EAS Deployment Information (See clause 7.1 of TS 23.548 [74]) | DNN and/or S-NSSAI | Application Identifier and/or Internal Group Identifier |
|  | AM influence information (See clause 4.15.6.9.3) | AF transaction internal ID |  |
|  |  | S-NSSAI and DNN and/or Internal Group Identifier or SUPI or "any UE" indication (NOTE 4) |  |
|  | AF request for QoS information (See clause 4.15.6.14) | AF transaction internal ID |  |
|  |  | S-NSSAI and DNN and/or Internal Group Identifier or SUPI or "any UE" indication (NOTE 4) |  |
| Policy Data | UE context policy control data (See clause 6.2.1.3 of TS 23.503 [20]) | SUPI |  |
|  | PDU Session policy control data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 of TS 23.503 [20]) |  | DNN |
|  | Policy Set Entry data (See clause 6.2.1.3 of TS 23.503 [20]) | SUPI (for the UDR in HPLMN) |  |
|  |  | PLMN ID (for the UDR in VPLMN) |  |
|  | Remaining allowed Usage data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 of TS 23.503 [20]) |  | DNN |
|  | Sponsored data connectivity profiles (See clause 6.2.1.6 of TS 23.503 [20]) | Sponsor Identity |  |
|  | Background Data Transfer data (See clause 6.2.1.6 of TS 23.503 [20]) | Background Data Transfer Reference ID. (NOTE 2) |  |
|  |  | None. (NOTE 1) |  |
|  | Network Slice Specific Control Data (See clause 6.2.1.3 of TS 23.503 [20]) | S-NSSAI |  |
|  | Operator Specific Data | SUPI or GPSI |  |
|  | Planned Data Transfer with QoS requirements data (See clause 6.2.1.6 of TS 23.503 [20]) | PDTQ Reference ID. (NOTE X2) |  |
|  |  | None. (NOTE X1) |  |
| Exposure Data | Access and Mobility Information | SUPI or GPSI | PDU Session ID or |
| (see clause 5.2.12.1) | Session Management information | SUPI or GPSI | UE IP address or DNN |
|  | DNAI mapping information | DNN and/or S-NSSAI |  |
| NOTE 1: Retrieval of the stored Background Data Transfer data for all ASP identifiers in the UDR requires Data Subset but no Data Key or Data Subkey(s). NOTE 2: Update of a Background Data Transfer data in the UDR requires a Data key to refer to a Background Data Transfer data as input data. NOTE 3: The Background Data Transfer includes the Background Data Reference ID and the ASP Identifier that requests to apply the Background Data Reference ID to the UE(s). Furthermore, the Background Data Transfer includes the relevant information received from the AF as defined in clause 6.1.2.4 of TS 23.503 [20]. NOTE 4: When the Data Key targets "any UE", then the request to UDR applies on Application data that applies on all subscribers of the PLMN. For encoding, see TS 29.519 [82]. NOTE 5: Group Data includes 5G VN group configuration and any other data related to a group stored in the UDR. NOTE 6: If a list of Internal Group IDs is used, the AF traffic influence request information request applies to the UEs that belong to every one of these groups, i.e. a single UE needs to be a member of every group in the list of Internal Group IDs. NOTE 7: When the Data Key targets "PLMN ID", then the request to UDR applies on subscription data about subscribers roaming in this PLMN. NOTE X1: Retrieval of the stored Planned Data Transfer with QoS requirements data for all ASP identifiers in the UDR requires Data Subset but no Data Key or Data Subkey(s). NOTE X2: Update of a Planned Data Transfer with QoS requirements data in the UDR requires a Data key to refer to a Planned Data Transfer with QoS requirements data as input data. |  |  |  |

The content of the UDR storage for (Data Set Id= Application Data, Data Subset Id = AF TrafficInfluence request information) is specified in clause 5.6.7, Table 5.6.7-1 of TS 23.501 [2]. This information is written by the NEF and read by the PCF(s). PCF(s) may also subscribe to changes onto this information.

Editor's note:	Related to the DNAI mapping information, it is FFS whether and how UDR is used (e.g. How UDR data is to be structured).

# * * * * End of changes * * * *