3GPP TSG-SA2 Meeting #156E	S2-2305116

# E-meeting, April 17 – 21, 2023						                                                  (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0142 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on handling the AF traffic influencing request for HR-SBO PDU Sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the HR-SBO session, the AF can send AF request to the V-NEF to influence the traffic routing. In the current specification, for HR-SBO session, the AF request should always be stored in the V-UDR and notified to the V-SMF via V-NEF in later procedures no matter whether the target UE is identified by SUPI or UE IP address. But for LBO and non-roaming sessions, the AF request targeting a UE by the UE IP address should be sent to the PCF as defined in 4.3.6.4. So the V-NEF should differentiate the scenarios and invoke different services to handle the AF requests. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications and corrections are added about how V-NEF differentiate the scenarios of HR-SBO session and LBO session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to differentiate the AF requests for HR-SBO and LBO sessions and how to hanlde the AF requests (storing in the UDR or sending to the PCF) are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.502 CR 4110 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** 1st Change **********

6.7.4	AF request on PDU Sessions supporting HR-SBO

For HR-SBO PDU Sessions, the AF in VPLMN may send to V-NEF an AF request to influence traffic routing (e.g. for the purpose of subscription to UP path management events on HR-SBO Sessions in VPLMN). The AF request for the HR-SBO PDU Session (which can be differentiated from the non-roaming and LBO PDU Session by the V-NEF according to the UE IP address in the AF request and the PLMN’s UE IP address ranges) from the AF is stored as Application Data (Data Subset = AF traffic influence request information) in the UDR of VPLMN as described in clause 4.3.6 of TS 23.502 [3]. To obtain the AF traffic influence request information, the V-SMF managing the PDU Session supporting HR-SBO subscribes to the NEF in VPLMN for notification of Application Data modification as specified in clause 4.3.6 of TS 23.502 [3].

********** End of Change ************