3GPP TSG-SA2 Meeting #157	S2-2307029

# 22 - 26 May, 2023, Berlin, Germany

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4212 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to filtering criteria for UE member selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To assist of UE member selection, some filtering criteria have been specified. To select the UEs that can fulfil the requirements of various AIML operations and maintain the service quality, the AF may indicate the NEF more filtering criteria based on the characteristics of the corresponding AIML services. To participate in the AIML services, the UEs should be in RM registered and CM and RRC connected states. To bring the UEs to the expected states before the AIML starts, the CN or RAN may need to page the UEs, which will involve considerable signalling to the area and may result in network congestion. Furthermore, the UE registration and connection establishment take considerable time, which may delay the AIML service. Therefore, the UE’s RM, CM and RRC connected state could be considered as filtering criteria for the AIML member selection. During the AIML operation, the AF may train the AIML model only if all members have uploaded models. If the variations of some UEs' end-to-end data volume transfer time are relatively large, the AF has to wait unexpected extra time for the model uploading from members. Therefore, the AF may indicate the required variation of transfer time that the selected UEs should meet or not exceed. If the UEs are misbehaved, e.g. unexpected radio link failures and Ping-ponging across neighbouring cells, the transmission of the AIML traffic will be interrupted. As a result, the AIML service quality will degrade. Therefore, the UEs with unacceptable levels of abnormal behaviours should not be recommended by the NEF to participate in the AIML operation as required by the AF. The NEF can collect the UE abnormal behaviour related data by subscribing to the abnormal behaviour analytics from NWDAF. |  |  |  |  |  |  |  |  |
| t |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new filtering criteria to assist the NEF in UE member selection: - UE registration state, connectivity state and reachability filtering criteria; - Variation of End-to-end data volume transfer time filtering criteria,and - UE Abnormal behaviour |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The member selection functionality cannot recommend the UEs that can fulfil the requirements of various AIML services and avoid network congestion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.2, 4.15.13.x (new), 4.15.13.x.1 (new), 4.15.13.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 4.15.13.2	UE Member Filtering Criteria for 5GS assistance to UE member selection

Table 4.15.13.2-1 provides a summary of the UE member filtering criteria that the AF may request.

Table 4.15.13.2-1: Description of UE Member filtering criteria

| UE Member filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the selected UEs match or exceed the QoS of the filtering criteria |  | 4.15.13.3 |
| Access Type and/or RAT Type of the PDU Session | Indicate the Access Type and/or RAT Type of the selected UE for the PDU Session used by the application (e.g. 3GPP/NR, Non-3GPP/WLAN, additional Access Type and RAT Type for MA PDU session) | Service operation: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI Event ID: Change of Access Type and/or Change of RAT Type | This can be provided by existing service operations, events, and procedures. |
| End-to-end data volume transfer time | Indicate the end-to-end data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE to AF | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = End-to-end data volume transfer time, target = GPSI(s) or SUPI(s) | 4.15.13.x |
| UE current location | Indicate the certain area that the selected UE currently located in. | Service operation: Namf_EventExposure Filter: a list of GPSI(s) or SUPI(s) | 4.15.13.3 |
| UE historical location | Indicate the certain area that the selected UE appeared in a historical period of time | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period | 4.15.13.3 |
| UE direction | Indicate the selected UEs should include different moving direction | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID= UE Mobility, Direction | 4.15.13.3 |
| UE registration state, connectivity state and reachability | Indicate the registration state (Registered or Deregistered), connectivity state (CM IDLE or CONNECTED), and/or UE reachability (RRC state) of the selected UEs. | Service operation: Namf_EventExposure service Filter: target = GPSI(s) or SUPI(s), Event ID = Reachability Filter | This can be provided by existing service operations, events, and procedures. |
| Variation of End-to-end data volume transfer time | Indicate the variation of end-to-end data volume transfer time filtering criteria the selected UEs should not exceed. | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = End-to-end data volume transfer time, target = GPSI(s) or SUPI(s), DNN/S-NSSAI, Application ID | 4.15.13.x |
| UE Abnormal behaviour | Indicate the Exceptions of UE Abnormal behaviour, e.g. if a UE abnormal behaviour is out of AF requirements, this UE will not be recommended by the NEF. | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = Abnormal behaviour, target = GPSI(s) or SUPI(s), Exceptions of different Exception IDs | Clause 6.7.5 of TS 23.288 [50]. |

Editor's note:	The list of UE Member filtering criteria and their corresponding actions are FFS.

Editor's note:	Each Filter Criteria may add a new clause where the specific procedures for such filter criteria are described. For example:"4.15.13.3	Specific procedure for criterial foo".

* * * Next Change * * *

#### 4.15.13.x	Specific procedure for end-to-end data volume transfer time related UE member filtering criteria

##### 4.15.13.x.1	General

An AF may invoke Nnef_UEMemberSelectionAssistance_Subscribe service operation with end-to-end data volume transfer time related filtering criteria for receiving a list of UEs that do not exceed the value of such filtering criteria.

In addition to the mandatory parameters, the AF also includes in the request:

-	End-to-end data volume transfer time filtering criteria: this may include the end-to-end data volume transfer time for a specific data volume between UE and AF and/or the variation of the end-to-end data volume transfer time.

-	Optionally, an Area of Interest: location area of the candidate UEs.

-	Optional time windows for selecting the candidate UEs: start time and stop time.

##### 4.15.13.x.2	UE member selection with end-to-end data volume transfer time related filtering criteria

Figure 4.15.13.x.3-1: 5GC assistance to UE member selection for end-to-end data volume transfer time related filtering criteria

- AF subscribes to the member selection assistance functionality by invoking Nnef_ UEMemberSelectionAssistance_subscribe request including the Application ID, DNN/S NSSAI, AoI, and the end-to-end data volume transfer time related filtering criteria including the end-to-end data volume transfer time and/or the variation of the transfer time.

- NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executed based on the end-to-end data volume transfer time related filtering criteria provided by the AF.

- If an Application ID is present in the request from the AF, the NEF derives the S-NSSAI, DNN and DNAI(s) to which this Application ID have access. NEF discovers querying UDM and NRF the SMFs that are deployed in the Area of Interest. The NEF may also restrict the discovery to those SMFs that serve some S-NSSAI and DNN combination (this S-NSSAI and DNN combination has been derived from the Application ID).

- NEF sends an Analytics request/subscribe to NWDAF by invoking Nnwdaf_AnalyticsSubscription_Subscribe / Nnwdaf_AnalyticsInfo_Request including Analytics ID = end-to-end data volume transfer time, Application ID, S-NSSAI, AoI, and target UEs based on the initial UE list obtained from the AF.

- The NWDAF collects data from multiple sources for end-to-end data volume transfer time analytics as specified in clause 6.18.2 of TS 23.288 [50].

- The NWDAF provides the required output analytics to the consumer NF as specified in clause 6.18.4 of TS 23.288 [50] by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 4.

- Based on the analytics received from the NWDAF, the NEF consolidates results and derives the list(s) of candidate UE(s) that fulfil the filtering criteria requested by the AF. The NEF may use the end-to-end data volume transfer time of specific volumes of data and/or the variations to derive the list(s) of candidate UEs that meet the requirements from the AF.

- NEF sends a Nnef_ UEMemberSelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and additional information.

* * * End of Changes * * *