SA WG2 Meeting #S2-157	S2-2307215

22 - 26 May, 2023, Berlin, Germany

Title:	[DRAFT]LS reply on Design of RTP Header Extension for PDU Set handling

Response to: 	LS on the Design of RTP Header Extension for PDU set handling(S4-230419/S2-2306280)

Release:	Rel-18

Work Item:	5G_RTP, XRM, NR_XR_enh

Source:	3GPP SA2

To:	3GPP SA4

Cc:	RAN2

Contact Person:

Name:	Xiaowan Ke

Tel. Number:

E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall description

SA2 thanks SA4 for their LS (S4-230419/ S2-2306280) on the design of RTP Header Extension for and would like to provide the following feedback to SA4’s questions:

Question#1: SA4 would like to ask if it is possible for a single QoS flow to include both PDUs marked with PDU set header extension and unmarked PDUs?

SA2 Answer:

SA2 agreed that a single QoS Flow can include either PDUs with GTP-U header extensions for PDU Sets or PDUs without such extension, but doesn’t to support to include both.

Furthermore, SA2 kindly seeks SA4’s clarification on the following:

- Q1: if it is possible for a single RTP stream to include both PDUs marked with PDU set header extension and unmarked PDUs?

- Q2: if the answer to Q1 is yes, please further clarify the motivation that marking packets with PDU Set Header but leaving some other packets unmarked in an RTP stream.

Question#2: Does SA2 expect the application server to decide which PDUs belong to a data burst and signal related information in the RTP header extension?

SA2 Answer:

As stated in the previous LS to SA4 (S2-2303849), in Rel-18 SA2 kindly asked SA4 to provide guidelines on End of Data Burst Indication marked by the application server via RTP header information (e.g., the timestamp, sequence number and "M" bit) or define the 3GPP specific RTP header extension if the current RTP header cannot provide such information to determine End of Data Burst Indication.

2. Actions:

To SA WG4

ACTION: 	SA2 kindly ask SA4 to take the above reply into account and SA2 kindly seeks SA4’s clarification on the following:

-  Q1: if it is possible for a single RTP stream to include both PDUs marked with PDU set header extension and unmarked PDUs

- Q2: if the answer to Q1 is yes, please further clarify the motivation that marking packets with PDU Set Header but leaving some other packets unmarked in an RTP stream.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #157	from 2023-05-22	to 2023-05-26		Berlin, Germany

TSG-SA2 Meeting #158	from 2023-08-21	to 2023-08-25		Goteborg, Sweden