3GPP TSG-WG SA2 Meeting #158	S2-2308900

# Goteborg, Sweden, Aug 21 – 25, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0891 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on registration and discovery procedure for FL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | No need to restrict the selected FL server NWDAF is currently executing an Federated Learning procedure, it’s sufficient that the selected FL server NWDAF has the capabality to support the Federated Learning procedure. Step 7-9 are intended to check if the FL Client NWDAF(s) received in step 6 can meet the ML model training requirement. However, the FL Client NWDAF(s) received in step 6 may already meet some criteria (e.g., Analytic ID of the ML model required, FL capability Type (i.e. FL client), NF type(s) of data sources, Time Period of Interest and Interoperability Indicator, etc.), so step 7-9 are optional, and the FL Server NWDAF can select FL Client NWDAF(s) based on the information received in step 6 and other information received in step 9 (if available). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change “check if the selected FL Server NWDAF is currently executing an Federated Learning procedure” to “check if the selected FL Server NWDAF is currently supporting the Federated Learning procedure”. Clarify that the FL Server NWDAF can select FL Client NWDAF(s) based on the information received in step 6 and other information received in step 9 (if available), and Figure 6.2C.2.1-1 is updated. Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of registration and discovery procedure for FL is not very appropriate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2C.2.1	Registration and Discovery procedure for Federated Learning

FL Server NWDAF

FL Client NWDAF 1

NRF

...

FL Client NWDAF N

1a. Nnrf_NFManagement_NFRegister Request (FL Server NWDAF profile)

1b. Nnrf_NFManagement_NFRegister Request (FL Client NWDAF profile)

2. Store FL Server NWDAF and FL Client NWDAF profile

3a. Nnrf_NFManagement_NFRegister Response

3b. Nnrf_NFManagement_NFRegister Response

4. Nnrf_NFDiscovery_Request Request

6. Nnrf_NFDiscovery_Request Response

(one or multiple FL Client NWDAF instances, i.e., FL Client NWDAF 1, ..., N)

5. Authorize NF Service Discovery

NWDAF Registration

7. FL preparation request

8. Decide to join or not

9. Response to FL Server NWDAF

10. Select FL Client NWDAF(s)

NWDAF Discovery

Figure 6.2C.2.1-1: Registration and Discovery procedure for Federated Learning

Steps 1 to 3 are the NWDAF registration procedure.

1-3.	NWDAF containing MTLF as FL Server NWDAF or FL Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability type information (i.e. FL server or FL client) and Time interval supporting FL as described in clause 5.2.

Steps 4 to 6 are the NWDAF Discovery procedure.

4-6.	NWDAF containing MTLF determines ML model requires FL based on Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as FL Server NWDAF, the MTLF first discovers and selects FL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, Model filter information as defined in TS 23.288 [5], FL capability Type (i.e. FL server), check if the selected FL Server NWDAF is currently supporting the Federated Learning procedure for the Analytics ID, Time Period of Interest, Service Area.

Once the FL Server NWDAF (the requested or the selected one) is determined, it discovers and selects other NWDAF(s) containing MTLF as FL Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, FL capability Type (i.e. FL client), Service Area, NF type(s) of data sources from which the FL Client NWDAF is able to collect data for local model training, Time Period of Interest, Interoperability Indicator.

7.	[Optional] FL Server NWDAF sends Federated Learning preparation request to the FL Client NWDAF(s), using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag, to check if the FL Client NWDAF(s) can meet the ML model training requirement (e.g. Analytics ID, ML Model Interoperability information, Available data requirement (a list of Event IDs of the local data for training, Available data requirement may also include the dataset statistical properties, the time window of the data samples and the minimum number of data samples), Availability time requirement (time span needed for the FL process), etc.).

8.	[Optional] FL Client NWDAF(s) checks if it can meet the ML model training requirement and/or can successfully download the model if the model information is provided in the request and decides whether to join the Federated Learning process based on implementation. Example criteria used by FL Client NWDAF(s) may be based on its data availability and time availability, computation and communication capability, and ML Model Interoperability information.

9.	[Optional] FL Client NWDAF(s) invokes Nnwdaf_MLModelTraining_Notify service operation or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate FL Server NWDAF if it will join the FL procedure, may contain the reason in the response message if it cannot join the FL process.

10.	FL Server NWDAF selects  FL Client NWDAF(s) based on the information received in step 6 and other information received in step 9 (if available).

# * * * * End of changes * * * *