3GPP TSG-WG SA2 Meeting #158	S2-2309040

# Goteborg, Sweden, Aug 21 – 25, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0027 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Procedures for Network based SL positioning for UE without NAS connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some descriptions in Clause 5.5.3 is duplicated. It was agreed that each Located UE that can establish the NAS connection may report the Ranging/SL positioning measurement data or result and the routing identifier from Target UE to the LMF. But the corresponding part is missing from the procedure. It proposes that how each Located UE reports the Ranging/SL positioning measurement and the deferred routing ID to the LMF in the procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To delete the duplicated part in the clause 5.5.3. - Each Located UE reports the Ranging/SL positioning measurement and the deferred routing ID to the LMF in the procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure and the functionality parts are not algined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3, 6.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.5.3	Network based SL positioning for UE without NAS connection

When Target UE cannot establish the NAS connection with AMF due to the Target UE being out of coverage and at least one of the discovered Located UEs can establish NAS connection for 5GC-MO-LR, the following principles are applied:

NOTE 1:	5GC-MT-LR is not applicable in this release of the specification.

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE and Located UE(s) perform Ranging/SL positioning.

-	The Target UE selects at least one of the Located UEs that can use SL-MO-LR service for SL Positioning.

-	The Target UE may transmit its ranging measurement data/results to the Located UE(s).

-	Each Located UE that can establish the NAS connection may report the Ranging/SL positioning measurement data or result and the routing identifier from Target UE to the LMF via the serving AMF of the Located UE. This may include ranging measurement data/results received from the Target UE. The endpoints for LPP messages are the LMF and the Located UE(s).

NOTE 2:	The routing identifier indicates an LMF that manages the Target UE positioning, and also used to assist the LMF in identifying the Target UE positioning, as defined in clause 6.3.1 of TS 23.273 [8].

-	The LMF may use the received information (i.e. Ranging/SL positioning measurement or estimation result with the same routing identifier) and the location of Located UE(s) to calculate the location of the Target UE, and provide the resulting location via the Located UE to the Target UE.

-	If a LMF is aware of multiple Located UEs associated with the same Target UE based on the same routing identifier, it may perform the calculation using all information from these Located UEs.

- In case the LMF has the Target UE context, the LMF may provide the resulting location via the GMLC to the LCS Client or AF as specified in TS 23.273 [8] clause 6.3. In case the UE context was deleted, then this functionality cannot be performed.

Editor's note:	Security aspects require confirmation from SA WG3.

# * * * * Second change * * * *

### 6.5.2	Procedures for Network based SL positioning for UE without NAS connection

The procedures for Network based SL positioning for UE without NAS connection apply to the following cases:

-	when the Target UE initiates 5GC-MO-LR request to obtain its own absolute location and cannot establish a NAS connection or cannot be served by a Ranging/SL positioning capable LMF

-	when the target UE detects an event configured by the network during an initial 5GC-MT-LR procedure when the Target UE has the NAS connection

The procedures for Network based SL positioning for UE without NAS connection takes the SL-MO-LR procedure in TS 23.273 [8] as baseline with following adaptations based on the functionalities described in clause 5.5.3:

-	UEx (one of the UE2/.../UEn) is the Target UE that cannot set up a NAS connection or that cannot be served by a Ranging/SL positioning capable LMF, and UE1 is Located UE that is in coverage and can be served by a Ranging/SL positioning capable LMF.

NOTE:	UE1 as the Located UE can be multiple.

-	In step2, it is the UEx (Target UE) that performs UE1 (Located UE) discovery.

-	In step 6, when the UEx(Target UE) initiates SL-MO-LR service request to UE1(Located UE), the UEx(Target UE) requests UE1(Located UE) to return its absolute location and may indicate whether the absolute location should be transferred to an LCS client or AF. For transfer to an LCS Client or AF, Target UE ID and the Deferred routing ID may be included in the SL-MO-LR service request.

-	It is the UE 1 as the Located UE that performs the SL-MO-LR with the step 8~23 with following adaptions:

-	In Step 8: UE1(Located UE) sends SL-MO-LR request which indicates that the Target UE’s absolute location is required, may indicate that the location calculation assistance by the network is needed, and may indicate that location results need to be transferred to the LCS client or AF for UEx (Target UE). For transfer to the LCS Client or AF, the Target UE ID and the Deferred routing ID may be included along with the SL-MO-LR request. The SL positioning measurement data may be included in step 8.

-	In Step 9, the AMF forwards SL positioning measurement data to the serving LMF based on the deferred routing identifier if it is included in the SL-MO-LR service request.

-	The LMF determines the network based calculation taking the obtained capability of UE in step 10 and step 11 into account.

-	The LMF requests for the location information from the UE1(Located UE) in step14.

-	The step17 for UE based calculate is not needed.

-	The UE1(Located UE) returns the SL location measurement data to the LMF in step18.

-	When SL positioning measurement data was provided in step 8, step 12~17 may be omitted.

-	The LMF calculates the location result (i.e. the absolute Location of Target UE) based on the SL location measurement data and the location of the UE1(Located UE) in step19, and provides the location of UEx(Target UE) to the UE1 (Located UE) in step 22.

-	In case the LMF has the valuable UEx(Target UE) context based on the Deferred routing ID, then the LMF provides the resulting location of the UEx(Target UE) via the GMLC to the LCS Client or AF in step 22. In case the UEx(Target UE) context was deleted, transferring location of the UEx(Target UE) to the LCS Client or AF is not proceeded.

-	For multiple Located UEs involement, UEx (Target UE) performs step 6 for each Located UE. Other Located UEs (some of the UE2/.../UEn) may also performs step 8~9 of the SL-MO-LR procedure to report the Ranging/SL positioning measurement and the deferred routing ID obtained from Target UE to the LMF. The LMF is aware of multiple Located UEs associated with the same Target UE based on the same routing ID, it performs the calculation using all information from these Located UEs.

-	The UE1 (Located UE) returns the absolute location result (i.e. the absolute Location of Target UE) to the UEx(Target UE) in step23 over PC5.

# * * * * End of changes * * * *