SA WG2 Meeting #158	S2-2309293

# 21-25 Aug 2023, Goteborg, Sweden	(was S2-23xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4429 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NSAC for maximum number of UE with at least one PDU session/PDN connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNSAC |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NSAC for maximum number of UE with at least one PDU session and one PDN connection was discussed and a new clause 4.11.5.9a was introduced in 23.502. This new clause describes the additions comparing with the clause 4.2.11.2 and clause clause 4.2.11.4. One addition is that a new update type value “update” is introduced for option 1 to cover the case when the UE moves across 3GPP access and non 3GPP access the SMF+PGW-C updates the NSACF with new Access type. For option 1 there are issues on the following new sentence: ----------------------------------- If there was one UE entry in the new Access Type with same NF ID then NSACF will not increase the count again. If there was one UE entry in the new Access Type with different NF ID then NSACF will add this new NF ID but does not increase the count. If there was one entry with same NF ID in the old Access Type then NSACF will remove the UE entry and decrease the count. ------------------------------------ The sentence above implies that the NSACF create the UE entry per Access Type, and the NSACF will always do UE counting per Access Type. However there is no requirement to do counting per Access Type in the TS 23.501 and TS 23.502. The NSACF maintains the UE entry per UE basis and may take the Access Type into account when do UE counting. Since this sentence is about one of NSACF internal implementation, it is proposed to remove it. For option 2, since the new “update” value has been supported in clause 4.2.11.4 and how the NSACF handle the update type is described in clause 5.15.11.5a, there is no need to describe here because this new clause only describe additions. It is proposed to remove the description of update type. Description on hierarchical NSAC architecture is missing. |  |  |  |  |  |  |  |  |
| t |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the sentence above Add the missing ID after AMF Add description on hierarchical NSAC architecture |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between different clauses and duplicated description in specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.5.9a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

4.11.5.9a	Network Slice Admission Control in 5GS for maximum number of UE with at least one PDU session and one PDN connection

Support of NSAC in conjunction with interworking with EPC for maximum number of UE with at least one PDU session/PDN connection is described in clause 5.15.11.5a of TS 23.501 [2].

For deployment option 1, the same mechanisms in clause 4.2.11.2 (Number of UEs per network slice availability check and update procedure) is used with the following additions:

-	The SMF+PGW-C invokes this procedure during PDU session establishment and PDU connection release. In this case, the AMF in figure 4.2.11.2-1 is replaced with SMF+PGW-C.

-	Step 2: The SMF+PGW-C includes in the message the S-NSSAI, identity of SMF+PGW-C, UE ID and update flag. The update flag may include either 'increase', 'decrease' or 'update' values.

-	Step 2: The AMF ID is replaced by the SMF+PGW-C ID.

-	Step 3: If the update flag parameter from the SMF+PGW-C indicates 'update', the NSACF replace the Access Type of existing UE registration with new Access Type during inter access mobility.

-	The NSACF determines whether or not to accept the request as described in clause 5.15.11.5a of TS 23.501 [2]

NOTE 1:	EAC mode is not applicable here.

For deployment option 2, same mechanisms in clause 4.2.11.4 (Number of PDU Sessions per network slice availability check and update procedure) are used with the following additions:

-	The NSACF also counts the maximum number UE with at least one PDU session/PDN connection based on the update value received from SMF+PGW-C.

-	The NSACF determines whether to accept the request as described in clause 5.15.11.5a of TS 23.501 [2]

If hierarchical NSAC architecture is deployed, when the local maximum number or local threshold is reached the NSAC may interact with the Primary NSACF before it returns the response back to the SMF+PGW-C. For more details on handling at the NSACF and Primary NSACF see clause 5.15.11.1.2 of TS 23.501 [2].

# >>>>END OF CHANGES<<<<