3GPP TSG-WG SA2 Meeting #S2-159	S2-2310565

# 09 - 13 October, 2023, Xiamen, China		(revision of xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0048 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The provision of Application layer ID information for a UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Inspur |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-09-18 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Application Layer ID is the third party information, which is out of the control of 3GPP system. It is concluded in TS 23.586 that AF can provide the mapping information between Application Layer ID and GPSI in UDR. And the relative positiong request targeting with Application Layer ID can be translated into SUPI of target UE in GMLC if needed. But it is unreasonable that it is assumed that the authorized AF always exists and stores the mapping information for the UE’s Appplication Layer ID. On the other hand, UE installing the V2X and/or ProSe application also can get the Application Layer ID(s) information from the upper layer. So the UE has the complete information of Application Layer ID(s) itself. So the UE can report or update the Application Layer ID information to network as well via N1 NAS message. AMF then transfers the information to UDM via Nudm_ParameterProvision_Update service operation. UDM can store the Application Layer ID(s) information for the UE to UDR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the new function that the UE can provide the mapping between UE and Application Layer ID to UDM/UDR. - A high level feature is added to specify the provision of Application Layer ID for a UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Miss the another approach to get the mapping information between UE and it’s Application Layer ID if the method of provisioning Application Layer ID(s) information by AF is not available. - The description of high level feature to caputre Applicaion Layer ID information is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1, 4.3.3, 4.3.4, 4.3.5, 5.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.3.1	UE

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the UE may support the following functions:

-	Reporting the following Ranging/SL Positioning Capabilities to 5GC over the N1 reference point:

-	Capability of supporting Ranging/SL Positioning over PC5;

NOTE:	Based on Ranging/SL Positioning control, a UE capable of Ranging/SL Positioning may take different roles in the operation, e.g. Target UE, SL Reference UE, Located UE.

-	Capability of supporting SL Positioning Server UE over PC5.

-	Procedures for Ranging/SL Positioning over PC5.

-	Procedures to Network based SL Positioning, and Network assisted SL Positioning.

-	Procedures to UE-only SL Positioning.

-	Procedures to Ranging/SL Positioning service exposure.

-	Indicating UE Policy Provisioning Request in UE Policy Container for UE triggered Ranging/SL Positioning Policy provisioning, which requests one or multiple types of policies/parameters as listed below:

-	Policy/parameters for Ranging/SL Positioning over PC5;

-	Policy/parameters for Located UE;

-	Policy/parameters for Target UE in addition to the functions defined in TS 23.273 [8] clause 4.3.5;

-	Policy/parameters for SL Positioning Client UE;

-	Policy/parameters for SL Positioning Server UE.

-	Receiving the Ranging/SL Positioning Policy from 5GC over N1 reference point.

-	Configuration of parameters for Ranging/SL Positioning over PC5. These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over SR1 reference point from the Ranging/SL Positioning Application Server.

-	Providing Application Layer ID information for V2X application(s), ProSe application(s) and both in the UE to UDR via UDM.

# * * * * Second change * * * *

4.3.3	AMF

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the AMF performs the following functions:

-	Select a PCF supporting Ranging/SL Positioning Policy/Parameter provisioning based on indication of Ranging/SL Positioning Capability as part of the "5GMM capability" in the Registration Request.

-	Store the Ranging/SL Positioning Capability.

-	Select the LMF supporting Ranging/SL positioning and forward the Ranging/SL Positioning Capability of Target UE to LMF.

-	Select a GMLC supporting Ranging/SL positioning and request the GMLC for Ranging/SL Positioning result.

-	Forward the Ranging/SL Positioning Capability to PCF.

-	Obtain from UDM the subscription information related to Ranging/SL positioning and store them as part of the UE context data.

-	Provision the NG-RAN with indication about the UE authorization status about Ranging/SL Positioning over PC5.

-	Provision the NG-RAN with PC5 QoS parameters related to RSPP transport over PC5.

-	Receive the latest Application layer ID information from a UE and transfer to a UDR via UDM.

# * * * * Third change * * * *

4.3.4	UDM

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the UDM performs the following functions:

-	Subscription management for Ranging/SL Positioning over PC5.

-	Subscription management for Network based SL Positioning, and Network assisted SL Positioning.

-	Subscription management for Ranging/SL Positioning service exposure.

-	Receive the latest Application Layer ID information for a UE from the UE or AF and store the UE’s Application Layer ID information to UDR.

# * * * * Fourth change * * * *

4.3.5	UDR

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the UDR performs the following functions:

-	Stores Ranging/SL Positioning service parameters.

-	Store the mapping between Application Layer ID, SUPI and GPSI for a UE provisioned by AF or the UE.

# * * * * Fifth change * * * *

5.4.x	Provision of Application Layer ID information for a UE

Application Layer ID in a UE is defined in TS 23.287 [6] and TS 23.304 [7]. So the Application Layer ID information for a UE includes the Appplication Layer ID(s) for all V2X and ProSe applications in the UE. A valid time period may be included in the Application Layer ID Information for a UE.

A generation or change to the UE Application Layer ID information is determined by the UE and provided to the network using N1 NAS message. It may be updated by UE any time. An authorized AF is allowed to provision the Application Layer ID information for specific UE(s) via NEF.

The updated Application Layer ID information is stored into the UDR by the UDM with SUPI and GPSI after the interaction with the AMF or NEF.

In addition, a notification is sent by the UDM in order to notify the subscribed consumer (i.e. GMLC and NEF) about the change of Application Layer ID information:

-	Target UE identity (one or both of GPSI and SUPI);

-	Updated UE Application Layer ID information.

# * * * * End of changes * * * *