SA WG2 Meeting #SA2-159	S2-2311106

Oct 09-13 2023, Xiamen, China

# Title:	Reply LS on 5G ProSe UE-to-UE relay discovery

# Response to:	LS to SA2 on 5G ProSe UE-to-UE relay discovery (S2-2310091/ C1-236448)

# Release:	Rel-18

# Work Item:	5G_ProSe_Ph2

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Jianning LIU

E-mail Address:	liujianning@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-231xxxxx

# Overall description

SA2 thanks CT1 for their LS on 5G ProSe UE-to-UE relay discovery (S2-2310091/ C1-236448).

SA2 would like to provide following clarification for each questions as:

Question 1: How can this issue be resolved?

For Question1, SA2 has discussed and agree to attached CRs to add the required information (i.e., User Info ID and RSC(s)) during 5G ProSe Direct Discovery with Model A and Model B procedures (see the attached CRs)

Question 2: How can the 5G ProSe UE-to-UE relay UE tell that the 5G ProSe UE-to-UE Relay Discovery Solicitation message is for the relay reselection so that it can skip finding the target end UE and respond to the source end UE directly?

For Question 2, as described in step 2 and 3 in clause 6.3.2.4.4, due to the absence of the RSC in the received 5G ProSe UE-to-UE Relay Discovery Solicitation message.

Question 3: How can the 5G ProSe UE-to-UE relay UE know which service the discoverer end UE wants to use before accepting the 5G ProSe UE-to-UE Relay Discovery Solicitation message?

For Question3, in step4 in clause 6.3.2.4.4, the 5G ProSe UE-to-UE relay will include the RSC information in the 5G ProSe UE-to-UE relay Discovery response message.

Question 4: Why are different approaches applied to the user info of the target 5G ProSe end UE in the above procedures?

For Question 4.

2. Actions:

To CT1:

ACTION: 	SA2 kindly asks CT1 to take the above information into account.

3. Date of Next SA2 Meetings:

3GPP TSG-SA2 Meeting #160	13-17 November 2023	CHICAGO, USA

3GPP TSG-SA2 Meeting Ad Hoc	22-26 January 2023 	online