3GPP SA WG2 Meeting #159	S2-2311204

# 9-13 October 2023, Xiamen, China	(was S2-230xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5060 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MBSR authorization handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In CT1 LS on MBSR authorization state (S2-2310089/C1-236002), it was clarified that: For Case-1 and Case-3, CT1 agreed that generic UE configuration update procedure is used to inform the UE about the MBSR authorization state change. Deregistration procedure was not extended. Deregistration procedure with "re-registration required" indication without any extensions can be used. For Case-2, CT1 agreed that deregistration procedure with existing cause can be used so there is no need of extensions. For Case-4 and Case-5, CT1 agreed that registration accept is used to inform the UE about the MBSR authorization state. For Case-6, CT1 agreed that registration reject with existing cause can be used so there is no need of extensions. Therefore, no new cause code for Deregistration would be defined by stage 3. The existing descriptions in clasue5.35A.4 needs to be updated accordingly. Additionally, the authorization status naming also need to be aligned with the CT1 description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the MBSR authorization description based on CT1 reply, and remove the EN on additional information form AMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent stage 2 and stage 3 specifications on MBSR authorization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 5.35A.4	MBSR authorization

For a MBSR, the subscription information stored in the HPLMN indicates whether it is authorized to operate as MBSR, and the corresponding location and time periods.

NOTE 1:	For non-roaming MBSRs, the operator local policy can be taken into consideration for MBSR authorization, e.g. based on the network status, or a limit on the number of MBSRs operating in a certain area.

When the MBSR (IAB-UE) performs initial registration with the serving PLMN, it indicates the request to operate as a MBSR as described in clause 5.35A.1. The AMF authorizes the MBSR based on the subscription information, and provides MBSR authorized indication to the MBSR node over NAS and NG-RAN over NGAP as described in the registration procedure in TS 23.502 [3]. The MBSR establishes the connection to OAM system using the configuration information for MBSR operation upon the reception of MBSR authorization indication ("authorized").

When MBSR roaming is supported, a roaming agreement between VPLMN and HPLMN regarding MBSR operation is in place. The AMF can make use of the subscription data for authorization of the MBSR in the V-PLMN.

MBSR (IAB-DU) can use IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to integrate into the serving PLMN to provide service.

NOTE 2:	How the MBSR obtains the configuration information for MBSR operation is described in clause 5.35A.2.

If the MBSR operation is not authorized (e.g. due to location or time limitation), the AMF of the MBSR can indicate to the MBSR IAB-UE that it is not allowed to act as an MBSR, i.e. the MBSR authorization indication ("not-authorized"), as part of registration procedure. In this case, the AMF includes the MBSR authorization indication ("not-authorized") to donor-gNB. The AMF may provide the indication either in a Registration Accept (if the PLMN allows the MBSR IAB-UE to be registered in the PLMM) or in a Registration Reject (if the PLMN does not allow the MBSR IAB-UE to be registered in the PLMN) message. The MBSR provides the authorization indication ("not-authorized") to its AS layer. The AMF may provide additional information to the MBSR regarding the authorization indication ("not-authorized") to assist the MBSR behaviour control.

Editor's note:	Whether the MBSR needs to provide the authorization indication (not authorized) to its AS layer will be further synch with RAN WGs.

When the MBSR authorization state changes for a registered MBSR node (either "authorized", or "not-authorized"), the AMF updates the MBSR and the NG-RAN accordingly. Based on the operator configuration, the AMF may use either Deregistration (including the option with re-registration required indication) or the UE Configuration Update procedure to inform the MBSR regarding the updated authorization status:

-	When Deregistration procedure is used, AMF includes the "re-registration required" indication if the MBSR registration status is changed from "not-authorized" to "authorized" or the MBSR IAB-UE is allowed to register in the PLMN. The AMF provides the new authorization indication when the MBSR registers again. Otherwise, i.e. when the registration status is changed from "authorized" to "not-authorized" and the MBSR IAB-UE is not allowed to operate as a UE, the AMF deregisters the MBSR.

-	When UE Configuration Update procedure is used, the AMF provides the new authorization indication and additional information to the MBSR in the UE Configuration Update Command. The MBSR provides the new authorization indication to its AS layer.

The AMF informs the NG-RAN of the new authorization status using UE Context Modification, Initial Context Setup procedure or the DOWNLINK NAS TRANSPORT message, with the following principles:

-	If the authorization state changes from "authorized" to "not-authorized" and AMF uses the UE Configuration Updated procedure to update the MBSR, the AMF updates the NG-RAN with the new authorization indication ("not authorized") by including this information in the DOWNLINK NAS TRANSPORT message. The NG-RAN completes handover of the UEs served by the MBSR before releasing the F1 connection to the MBSR IAB-DU.

-	If the authorization state changes from "authorized" to "not-authorized" and the AMF uses the Deregistration procedure to update the MBSR, the AMF sends the UE Context Modification message to NG-RAN before triggering the deregistration towards MBSR after a certain period (e.g. based on the expiration of a timer configured on the AMF).

If a PDU session is used to provide OAM access for MBSR when it is not authorized but remains registered, the AMF may notify the SMF serving the PDU session for O&M access to trigger the release of the PDU Session.

NOTE 3:	The AMF delays the MBSR de-registration to allow the IAB-donor gNB to move all connected UEs via MBSR to other cells as specified in clause 8.9.10 of TS 38.401 [42].

NOTE 4:	The mechanism applies to both roaming and non-roaming MBSR operations.

# >>>>END OF CHANGES<<<<