# SA WG2 Meeting SA2#160-Ad Hoc-e	S2-2400122r03

# 22 - 29 January, 2024, Elbonia

Title:	Draft Reply LS on security aspects for Ranging/Sidelink Positioning

Response to:	Reply LS on security aspects for Ranging/Sidelink Positioning (S2-2313236 / S3-235078)

Release:	Release 18

Work Item:	Ranging_SL

Source:	SA2

To:	SA3

Cc:	CT1, RAN2

Contact Person:	Lars Nord

Lars dot Nord at sony dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks SA3 on the response to the SA2 LS on security aspects for Ranging/Sidelink Positioning in S3-235078/S2-2313236. SA2 have discussed the questions related to exposure through PC5 link in the response and provides the following answers.

On the Network based operation proposal for exposure through PC5 link:

Question1 to SA2: SA3 would like to request confirmation from SA2 whether the UE already supports adding user info of the Client UE within SL-MO-LR and the GMLC already supports to provide the required service to the LMF triggering privacy check or can be supported in Rel-18 timeframe.

Answer 1: It is currently not supported that UE1 includes user info of the Client UE within the SL-MO-LR. A separate GMLC service to perform privacy check of a specific Client UE is not specified. The GMLC performs the Privacy check of a Client UE as part of the Ngmlc_Location_ProvideRanging_Request (see TS 23.586, clause 6.7.1.1, step5) triggered by the AMF. If possible SA2 would like to avoid specifying the additional functionality.

On the UE only based operation proposal for exposure through PC5 link:

Question2 to SA2/RAN2: SA3 would like to request confirmation from SA2/RAN2 whether PC5 message or SLPP message already supports to carry the required information between the UEs or can be supported in Rel-18 timeframe.

Answer 2: To support the Privacy check in UE-only operation, it is assumed that supplementary RSPP message(s) sent over PC5 would include the required user info of the Client UE, as agreed in the LS (S2-2313889) sent to CT1, CT4 and RAN2. Supplementary RSPP signalling message is to be developed by CT WGs.

2. Actions:

To SA3

ACTION:

Kindly ask SA3 to take the above answers into account.

3. Date of Next SA2 Meetings:

SA2#161	26 February -1 March 2024	Athens, GR

SA2#162	15 - 19 April 2024	China (TBC), CN