# SA WG2 Meeting SA2#160-Ad Hoc-e	S2-2400122r09

# 22 - 29 January, 2024, Elbonia

Title:	Draft Reply LS on security aspects for Ranging/Sidelink Positioning

Response to:	Reply LS on security aspects for Ranging/Sidelink Positioning (S2-2313236 / S3-235078)

Release:	Release 18

Work Item:	Ranging_SL

Source:	SA2

To:	SA3

Cc:	CT1, RAN2

Contact Person:	Lars Nord

Lars dot Nord at sony dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks SA3 on the response to the SA2 LS on security aspects for Ranging/Sidelink Positioning in S3-235078/S2-2313236. SA2 have discussed the questions related to exposure through PC5 link in the response and provides the following answers.

On the Network based operation proposal for exposure through PC5 link:

Question1 to SA2: SA3 would like to request confirmation from SA2 whether the UE already supports adding user info of the Client UE within SL-MO-LR and the GMLC already supports to provide the required service to the LMF triggering privacy check or can be supported in Rel-18 timeframe.

Answer 1: It is currently not supported that UE1 includes user info of the Client UE within the SL-MO-LR. If possible SA2 would like to avoid specifying additional functionality.

On the UE only based operation proposal for exposure through PC5 link:

Question2 to SA2/RAN2: SA3 would like to request confirmation from SA2/RAN2 whether PC5 message or SLPP message already supports to carry the required information between the UEs or can be supported in Rel-18 timeframe.

Answer 2: To support the Privacy check in UE-only operation, it is assumed that supplementary RSPP message(s) can be sent over PC5 and it would include the required user info of the Client UE. LS (S2-2313889) which sent to CT1, CT4 and RAN2 can be referred.

2. Actions:

To SA3

ACTION:

Kindly ask SA3 to take the above answers into account and provides feedback and update, if any..

3. Date of Next SA2 Meetings:

SA2#161	26 February -1 March 2024	Athens, GR

SA2#162	15 - 19 April 2024	China (TBC), CN