3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401373r03

# E-meeting, January 22 – 29, 2024		(revision of S2-24xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 00119 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update service exposure to SL Positioning Client UE through PC5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony, Ericsson, Qualcomm Incorporated?, Vivo?, Interdigital, Philips, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA3 sent a LS in S2-24000027 on Privacy checking of SL Positioning Client UE when using sevice exposure through PC5. This CR clarifies that the Privacy check is done locally in each UE that is participating in the Ranging/SL Positioning procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the authorization check options more clearly. Clarify that the privacy check is performed locally in each UE, triggered by a supplementary RSPP messages transmitted over SR5 between participating UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification on the authorization of a SL Positioning Client UE and the privacy check. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.2, 6.7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This’CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.6.2.2	Ranging/SL Positioning service exposure through PC5

For Ranging/SL Positioning service exposure to a UE through PC5 in the following way:

-	SL Positioning Client UE discovers one of the two or more UEs described in clause 5.6.1, e.g. the SL Reference UE(s) or Target UE(s), using the Ranging/SL Positioning UE discovery procedure (refer to clause 6.4);

-	SL Positioning Client UE invokes the Ranging/SL Positioning service request to the discovered UE, e.g. SL Reference UE/Target UE, with the attributes described in clause 5.6.1. For absolute location, this service request includes the SL Positioning Client’UE's user info and Target’UE's user info, and may also include the user info for a list of candidate Located UE(s). For relative location or ranging information, the service request includes the SL Positioning Client’UE's user info, Target’UE's user info, and SL Reference’UE's user info. The Ranging/SL Positioning service request/response is transmitted in the Supplementary RSPP signalling message over SR5 and handled in the Ranging/SL Positioning layer.

-	UE privacy is based on the local configured privacy verification information to determine whether its location related information can be exposed to the SL Positioning Client UE or not. The UE that received the service request transmits a Supplementary RSPP signalling message over SR5 to the other UE(s) to trigger the local privacy verification. The other UE(s) respond(s) back to the originating UE with the privacy result.

-	If the SL Positioning Client UE is authorized to invoke Ranging/SL Positioning service exposure through PC5 and that participating UE(s) have no privacy concern to expose location related information to the SL Positioning Client UE, then the UEs perform the Ranging/SL positioning control between SL Reference UE and Target UE (as defined in clause 5.3), and return the Ranging/SL positioning result to SL Positioning Client UE.

# * * * * Next change * * * *

#### 6.7.1.1	Procedures for Ranging/SL Positioning service exposure through PC5

Figure 6.7.1-1: Service exposure to SL Positioning Client UE via PC5

1.	When a Ranging/SL Positioning service is triggered, SL Positioning Client UE may perform discovery procedure for the Ranging/SL Positioning service defined in clause 6.4 with following updates:

-	When SL positioning service is triggered, the client UE tries to discover the target UE and/or one or more SL Reference UE(s)/Located UE(s). When Model B discovery used, in the Solicitation message, target’UE's user info is included and role of target UE and role of SL Reference UE/Located UE are indicated in the RSPP metadata.

-	When Ranging service is triggered, the client UE tries to discover UE1 and/or UE2. When Model B discovery used, in the Solicitation message, ’E1's user info and ’E2's user info are included and role of target UE is indicated in the RSPP metadata.

During discovery for SL positioning service, the information of SL reference UE(s)/Located UE(s) is acquired.

2.	If discovery procedure succeeds, the SL Positioning Client UE establishes a PC5 connection with the discovered UE. The SL Positioning Client UE authorization check is performed during PC5 connection establishment as specified in TS 33.533 [13].

3.	The SL Positioning Client UE is informed of’UE's status information indicating that UE has NAS connection or no NAS connection from the discovered UE(s).

Edi’or's note:	Need to determine if such indication is provided during discovery or step 3, and whether or not it requires RAN WG2 involvement.

The SL Positioning Client UE may receive list of SL Reference UE(s)/Located UE(s) from target UE over the PC5 connection.

4.	Based on’UE's status information, the SL Positioning Client UE may downselect UE(s) to perform SL Positioning operation and select a UE (here UE1) to receive SL Positioning Service Request. The SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of Target UE and info of SL Reference UE(s)/Located UE(s).

For absolute location, the SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of Target UE, and required positioning QoS. The Service Request may include info of list of candidate Located UE(s).

For relative location, the SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of Target UE, info of SL Reference UE(s), and required positioning QoS.

For ranging, the SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of UE1, info of UE2, and required positioning QoS.

For ranging, if UE1 and UE2 are not in proximity, the SL Positioning Service Request is responded with reject by the receiving UE (either UE1 or UE2) and remaining steps are not performed.

5. 	The UE1 that received the service request transmits a Supplementary RSPP signalling message over SR5 to UE2 to trigger the local privacy verification, the message includes the SL Positioning Client UE ID. If the privacy result was negative in any of UE1 and/or UE2, UE1 will return a reject to the SL Positioning Client UE and remaining steps are not performed.

6a.	When UE1 or UE2 do not have NAS connection,  UE1 selects a SL Positioning Server UE. Ranging/SL Positioning operation is performed as described in clause 6.8 among target UE, SL Reference UE(s)/Located UE(s) and SL Positioning Server UE or LMF.

NOTE: The SL Positioning Server functionality may reside in either UE1, UE2 or other UE.

6b.	When UE1 have NAS connection, UE1 sends SL-MO-LR  as described in clause 6.20.1 of TS23.273[8].

7.	The Ranging/SL Positioning result is provided to the SL Positioning client UE by UE1.

# * * * * End of changes * * * *