3GPP SA WG2 Meeting #160-Ad Hoc-e	S2-2400592

# E-meeting, January 22 – 29, 2024

Source: 	China Mobile, Xiaomi, Samsung, Tencent, Nokia, Nokia Shanghai Bell

Title: 	Key issue for WT4: Enhancement for XR related network information exposure

Document for: 	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM / Rel-19

Abstract of the contribution: Propose key issue for WT4.

# 1 Discussion

In XRM Ph2 SID, the WT#4 is stated as below:

WT#4 Network exposure: Study whether and how XR related network capability/information (e.g. if the QoS profile requested by AF cannot be met, network can indicate the alternative QoS profile) can be exposed towards the application layer.

This Key Issue will reflect the WT#4’s requirement.

# 2 Proposal

It is proposed to update TR 23.700-70 as follows.

# * * * * First change * * * *(all new texts)

## 5.X	Key Issue #X: Enhancement for XR related network information exposure

### 5.X.1	Description

In Rel-18, the 5GS can expose the following information based on the QoS Monitoring to the AF: congestion information, data rate information and round trip delay, as described in clause 5.37.4 of TS 23.501[x]. For congestion information, this parameter can be exposed via Nupf_EventExposure service from UPF to AF, or via user plane based L4S method. For data rate information, this can be measured and reported by PSA UPF via Nupf_EventExposure service or via SMF/PCF/NEF to AF. For round trip delay, this parameter is exposed from PCF to NEF/AF.

The objective of this Key Issue is to study how to enhance network exposure mechanism to better support the network information/capability exposed to the application layer. In particular, the key issue includes the following aspects:

-	Whether and how XR related network capability/information (e.g. if the QoS profile requested by AF cannot be met, network can indicate the alternative QoS profile) can be exposed towards the application layer.

# * * * * End of change * * * *