3GPP TSG-WG SA2 Meeting #160	S2-231

# Chicago, US, Nov 13 – 17, 2023		(revision of S2-231xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR |  | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of UE IP address to Nnef_MemberUESelectionAssistance service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | While GPSI information can be available to an AF, the IP address information of the UE participating in a service is always available (as the AF is sending the user plane packets to this IP address). Hence, it is necessary to add the UE IP address as possible input parameter to the Nnef_MemberUESelectionAssistance service in order to enable the usage of this service for all AFs. Furthermore, there are also a couple of small errors to be corrected: - In the message flow conditional steps are shown as optional - The input of Nnef_MemberUESelectionAssistance_Subscribe service should also include the AF Identifier, which is needed by the NEF to verify the authorization of the AF request. - The description of a list of target UE(s) should be aligned between the procedures and the service operation. - Member UE filtering criteria should be the required input of Nnef_MemberUESelectionAssistance_Subscribe service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE IP address is added as possible input parameter to the Nnef_MemberUESelectionAssistance service. Change of step description in message flow from optional to conditional. Terminology correction from “list of UE(s)” to “list of UEs”. Add the AF Identifier to the input of Nnef_MemberUESelectionAssistance_Subscribe service. Change the “Target of Member UE Selection Assistance Reporting” to “a list of target UEs”. Remove the Member UE filtering criteria from the optional input of Nnef_MemberUESelectionAssistance_Subscribe service. Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality not supported for AFs that only know the UE IP addresses. Errors and inconsistencies in the description of the procedure and the service operations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.0, 4.15.13.1, 5.2.6.32.2, 5.2.6.32.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.15.13.0	General

AF provides a list of target UEs in the form of a list of GPSIs or a list of UE IP addresses and at least one Member UE filtering criterion as part of the service operation parameters to assist the candidate UEs selection. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve the information for each UE in the list of target UEs from 5GC NFs. Before sending the list(s) of candidate UEs to the AF, NEF consolidates all the information collected from other 5GC NFs and derives one or more list(s) of candidate UEs and possibly additional information according to the Member UE filtering criteria requested by the AF.

The Member UE selection assistance capability can be used to assist the AF to select the group of UEs to support application service (e.g. FL operation) and it is further defined in clause 5.46.2 of TS 23.501 [2].

Additionally, AF may leverage the 5GC network exposure for Member UE selection without the NEF assistance as described in clause 5.46.2 of TS 23.501 [2]. An example of how AF leverages the 5GC network exposure for Member UE selection is described in (informative) Annex I.

# * * * * Next change * * * *

#### 4.15.13.1	Member UE selection general information flow

This clause describes the procedures that are generally applicable independently of the Member UE filtering criteria sent by the AF.

Figure 4.15.13.1-1: 5GC assistance to Member UE selection and update

1.	AF subscribes the Member UE selection assistance functionality by sending Nnef_MemberUESelectionAssistance_subscribe request including a list of target UEs, one or more member UE filtering criteria listed in the Table 4.15.13.2-1 and optionally, time window(s). Subsequently, the AF may update the filtering criteria of the subscription by invoking Nnef_MemberUESelectionAssistance_subscribe and providing a Subscription Correlation ID.

2a.	[CONDITIONAL] If the AF request does not contain a Subscription Correlation ID, the NEF verifies the authorization of the AF Request and identifies which information needs to be collected for each UE in the list of target UEs and executes the corresponding service operation based on the Member UE filtering criteria provided by the AF, e.g. events, analytics and/or notifications.

2b.	[CONDITIONAL] If the AF request contains a Subscription Correlation ID, the NEF correlates the Nnef_MemberUESelectionAssistance_Subscribe request to an existing subscription according to the Subscription Correlation ID. The NEF uses the target UEs received in step 1 for the Member UE update using the updated filtering criteria.

3.	NEF interacts with different 5GC network functions to collect the required information for each UE in the list of target UEs. The set of interactions between NEF and among 5GC NFs are dependent on the Member UE filtering criteria provided by the AF. See Table 4.15.13.2-1 for details.

4.	Based on the collected information from other 5GC NFs, NEF consolidates all the information collected from other 5GC NFs to derive the list(s) of candidate UEs which fulfil the Member UE filtering criteria in the AF request. The NEF may derive recommended time window(s) considering the validity period(s) of the analytics used for Member UE selection criteria, which are a subset of the time window(s) received from the AF.

5.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UEs and possibly additional information. See clause 5.2.6.32.4 for details.

# * * * * Next change * * * *

##### 5.2.6.32.2	Nnef_MemberUESelectionAssistance_Subscribe service operation

Service operation name: Nnef_MemberUESelectionAssistance_Subscribe

Description: The NF consumer subscribes to receive the Member UE selection assistance information, or the subscription is updated if the same subscription is already defined in NEF.

Inputs, Required: AF Identifier, Notification Target Address (+ Notification Correlation ID), one or multiple Member UE filtering criteria shown in Table 4.15.13.2-1.

Inputs, Conditional Required:

If no Subscription Correlation ID is provided in the subscription, a list of target UEsin the form of a list of GPSIs or a list of UE IP addresses is required.

Inputs, Optional: Application ID, Subscription Correlation ID (in the case of modification of the existing subscription), Expiry time, time window(s) for selecting the candidate UEs, specific parameters depending on the Member UE filtering criteria, Periodicity (the periodicity of member update), maximum number of UEs (indicates the maximum number of candidate UEs that need to be fed back).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

# * * * * Next change * * * *

##### 5.2.6.32.4	Nnef_MemberUESelectionAssistance_Notify service operation

Service operation name: Nnef_MemberUESelectionAssistance_Notify

Description: NEF reports the Member UE selection assistance information to the consumer that has previously subscribed.

Inputs, Required: Notification Correlation Information.

Inputs, Conditional Required:

At least one of the following inputs is required:

-	One or more list(s) of candidate UEs in the form of a list of GPSIs or a list of UE IP addresses.

Inputs, Optional: Recommended time window for performing the application operation per list of candidate UEs as described in clause 4.15.13.1, specific value of the parameters that NEF gathered for the Member UE filtering criteria per candidate UE, a number for each filtering criterion that indicates the UEs in the initial list which do not meet the criterion (provided if there are multiple filtering criteria in the subscribe request).

NOTE:	This number can be an indication for AF to revise the corresponding filtering criterion.

Outputs, Required: Operation execution result indication.

# * * * * End of changes * * * *