3GPP WG SA2 Meeting #161	S2-2402331

# 26 February – 1 March, 2024, Athens, Greece	        (was S2-2400137_S2-2312432)

Source: 	NEC

Title: 	New KI: AF assistance for energy efficiency control in the 3GPP system

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution proposes a new key issue based on WT#3 and TR22.882.

# 1	Discussion

This contribution proposes a new Key Issue based on WT#3 of FS_ EnergySys and the TR22.882

The FS_ EnergySys objectives for WT#3 is:

- WT #3: 	Study 5GS enhancements (e.g., energy usage adjustment for NF from CN aspect, energy saving related decision making, NF selection leveraging NF energy states) for network energy saving including 5GC(NFs) and NG-RAN interactions, analytics, etc. Impacts on the UE are not ruled out e. g., for scenarios specified in TR 22.882 by SA1 EnergyServ.

- TR22.882, CPR 6.4-5: Subject to operator’s policy and agreement with 3rd party, the 5G system shall support a mechanism for the 3rd party to provide current or predicted energy consumption information over a specific period of time

# 2 Proposal

It is proposed to include the below new Key Issue ‘AF assistance for energy efficiency control in the 3GPP system’ into TR 23.700-66 v0.3.0

* * * Start of change * * * *

## 5.X	Key Issue #X: AF assistance for energy efficiency control in the 3GPP system

### 5.X.1	Description

A trusted 3rd party AF may assist the energy efficiency control for its users within the 3GPP system by providing parameters to configure and control the energy efficiency. For example, an AF may provide energy credit limits per UE, or per network slice and update them when needed based on the SLA agreement with the network operator or the AF may assist the energy states control within the 3GPP system.

This Key Issue is to study:

- Whether and how a trusted 3rd party AF may assist the energy efficiency control in the 3GPP system via energy efficiency related parameters provision and update.

- Whether and what energy efficiency related parameters can be provided by a trusted 3rd party AF.

* * * End of change * * * *

`