SA WG2 Meeting #161	S2-2402373

Athens, GREECE, February 26 – March 01, 2024	(revision of S2-240xxxx)

Source:	LG Electronics

Title:	KI#2&3, Sol#15: Solution updates

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This paper proposes to update Solution#15.

Discussion

Solution #15: Enhancements to existing 5GS procedures for energy saving was captured in TR 23.700-66v0.3.0 for KI#3 "5GS enhancements for network energy saving and efficiency" and KI#2 "Subscription and policy control to support energy efficiency and energy saving as service criteria".

This paper proposes to update Solution#15 related to KI#2 (per UE aspect) and KI#3 (per slice aspect).

Proposal

It is proposed to agree the following changes into TR 23.700-66.

* * * * Start of 1st Change * * * *

6.15	Solution #15: Enhancements to existing 5GS procedures for energy saving

6.15.1	Key Issue mapping

Editor's note:	This clause lists the key issue(s) addressed by this solution.

This solution addresses Key Issue #3 "5GS enhancements for network energy saving and efficiency" and Key Issue #2 "Subscription and policy control to support energy efficiency and energy saving as service criteria".

6.15.2	Functional Description

Editor's note:	This clause will describe the solution principles and assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

Existing 5G MM and SM procedures can be enhanced based on the requirements of network energy saving and energy efficiency.

The AMF can decide Access and Mobility parameters related to network energy saving based on AM subscription data, AM policy, UE related analytics (e.g. UE mobility and abnormal behaviour analytics), network energy control information, etc. and provide the parameters to the UE during 5G MM procedures (e.g. Registration and UE Configuration Update procedures).

The SMF can decide Session Management parameters related to network energy saving based on SM subscription data, SM policy, UE related analytics (e.g. UE communication and abnormal behaviour analytics), network energy control information, etc. and provide the parameters for the PDU Session during 5G SM procedures (e.g. PDU Session establishment/modification procedures).

The AM policy and SM policy are provisioned by the PCF taking into account the requirements of network energy saving. The AM and SM subscription data can be extended to contain validity conditions (e.g. time/area) for more features/capabilities (e.g. Subscribed UE-AMBR, 5GS Subscribed QoS profile, Subscribed-Session-AMBR, ATSSS information) to facilitate network energy saving (i.e. limiting network energy consumption per UE).

NOTE:	The AM and SM subscription data (see clause 5.2.3.3.1 of TS 23.502 [3]) contains already some features/capabilities with validity condition, e.g. CAG and Network Slice with validity time/location information.

The AMF and the SMF may requests network energy control information from the NEMF (Network Energy Management Function), a new 5GC NF introduced for network energy management. The network energy control information contains NF IDs of the 5GC NFs which are put in energy saving states with validity time and area information.

Editor's note:	The role of the NEMF and need of NEMF needs to be clarified.

Editor's note:	The analytics requires input information that has not been identified.

Editor's note:	The PCF seems to get energy related info from AMF, SMF but it is not described. If it is for the UE how is it possible as it is at initial registration time.

6.15.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

6.15.3.1	Mobility Management procedures enhanced for energy saving

Figure 6.15.3.1-1: Enhanced MM/Registration Procedures for energy saving

1.	The UE initiates Registration procedure towards the network as described in step 1, clause 4.2.2.2.2 of TS 23.502 [3].

The Registration procedure continues as specified in clause 4.2.2.2.2 of TS 23.502 [3], with the following enhancements:

2.	In step 14b of Figure 4.2.2.2.2-1 in TS 23.502 [3], the AM subscription data retrieved from the UDM contains validity conditions for one or more subscription parameters (e.g. Subscribed UE-AMBR per time period and/or area) to facilitate network energy saving (i.e. limiting network energy consumption per UE).

3.	In step 16 of Figure 4.2.2.2.2-1 in TS 23.502 [3], during AM Policy Association Establishment/Modification procedure, the PCF decides AM policy, e.g. UE-AMBR, List of UE-Slice-MBR, List of allowed TAIs, taking into account network energy related information, and provides the AM policy to the AMF. The PCF may collect the network energy related information by subscribing to energy related notifications from the AMF, SMF and/or UPF, or requesting/subscribing to energy related analytics from the NWDAF. The PCF may decide to reduce the UE-AMBR or UE-Slice-MBR or to expand a list of allowed TAIs, if the network energy related information indicates that the UE is associated with high network energy consumption in an area or for a time period.

4.	In step 21 of Figure 4.2.2.2.2-1 in TS 23.502 [3], the AMF provides the parameters e.g. Mobility restrictions, Allowed NSSAI, in Registration Accept message taking into account network energy related information. Also, in the Registration Accept message, one or more network supported features are provided with validity conditions (e.g. support of Control plane CIoT 5GS optimization with validity time and/or validity area). Based on this, the UE decides only to apply functions corresponding to the network supported features under the validity condition, thus saving network energy by avoiding unnecessary information processing (i.e. receiving and rejecting UE requests).

Regarding subscription and policy control to support energy efficiency and energy saving for a UE, the AMF sends energy efficiency related information in N2 message to the NG-RAN. The energy efficiency related information includes authorization information for applying energy efficiency policy and maximum energy consumption rate based on the subscription information. Later, based on this, the NG-RAN may perform energy efficiency operations, e.g. handover or redirect the UE to other NG-RAN with higher energy efficiency.

Regarding 5GS enhancements for network energy saving and efficiency for slice(s), the AMF sends authorization information for applying slice level energy efficiency policy based on the subscription information in N2 message to the NG-RAN. Later, based on this, the NG-RAN may perform energy efficiency operations, e.g. handover or redirect the UE to other NG-RAN with higher energy efficiency to reduce the energy consumption for specific slice(s) used by the UE.

After the completion of Registration procedure, the AMF may perform the following steps for energy saving:

5.	The AMF requests "UE Mobility" analytics and "Abnormal behaviour" analytics from the NWDAF as specified in clauses 6.7.2 and 6.7.5 of TS 23.288 [14].

6.	Optionally, the AMF requests network energy control information from the NEMF. The network energy control information contains NF IDs of the 5GC NFs which are put in energy saving states with validity time and area information.

Editor's note:	Whether and how the AMF gets network energy control information from the NEMF is FFS.

7.	Based on UE subscription data, AM policy, "UE Mobility" analytics and "Abnormal behaviour" analytics, network energy control information (if available), etc, the AMF performs access and mobility management procedures for energy saving, e.g. deregistering the UE with unexpected long-live/large rate flows based on "Abnormal behaviour" analytics if the registration procedure is for mobility registration update or periodic registration update, or forwarding the Registration Request to a target AMF based on the network energy control information (e.g. if the AMF itself is put in Energy Saving state and then a target AMF is selected for the UE based on the network energy control information).

8.	The AMF sends energy saving assistance information in N2 message to the NG-RAN. The energy saving assistance information may be part of CN assisted RAN parameters tuning based on UE mobility analytics, e.g. statistics/prediction of UE location and UEs' geographical distribution. Based on this, the NG-RAN performs energy saving operations, e.g. redirecting the UEs to cell(s) with higher energy efficiency.

NOTE 1:	Regarding providing the energy saving assistance information to NG-RAN, N2 signalling load should be considered. Non-UE associated N2 signalling can be considered for transmitting the energy saving assistance information regarding UEs' geographical distribution, which is up to RAN WG3 to decide.

NOTE 2:	How the NG-RAN performs energy saving operations with both OAM configuration and energy saving assistance information from the 5GC is up to implementation.

Editor's note:	It is FFS whether the NG-RAN supports performing energy saving operation based on assistance information from the 5GC.

6.15.3.2	Session Management procedures enhanced for energy saving

Figure 6.15.3.2-1: Enhanced SM/PDU Session Establishment Procedures for energy saving

1.	The UE initiates PDU Session Establishment procedure towards the network as described in step 1, clause 4.3.2.2.1 of TS 23.502 [3].

The PDU Session Establishment procedure continues as specified in clause 4.3.2.2.1 of TS 23.502 [3], with the following enhancements:

2.	In step 4 of Figure 4.3.2.2.1-1 in TS 23.502 [3], the SM subscription data retrieved from the UDM contains validity conditions for one or more subscription parameters (e.g. 5GS Subscribed QoS profile or Subscribed-Session-AMBR per time period and/or area, ATSSS information with validity time and/or area) to facilitate network energy saving (i.e. limiting network energy consumption per UE).

3.	In step 7b and 9 of Figure 4.3.2.2-1 in TS 23.502 [3], during SM Policy Association Establishment/Modification procedures, the PCF decides SM policy, e.g. Authorized Session-AMBR, Authorized default 5QI/ARP, taking into account network energy related information (e.g. energy consumption of the network slice for the PDU Session) and energy related application specific parameters (e.g. authorized energy consumption credit per network slice and/or application) provisioned by the AF, and provides the SM policy to the SMF. The PCF may collect the network energy related information by subscribing to energy related notifications from the AMF, SMF and/or UPF, or requesting/subscribing to energy related analytics from the NWDAF. The PCF may decide to reduce the Authorized Session-AMBR or change the Authorized default 5QI/ARP for the PDU Session, if the network energy related information indicates that the UE is associated with high network energy consumption in an area or for a time period.

4.	In step 11 of Figure 4.3.2.2-1 in TS 23.502 [3], the SMF provides the parameters in N2 SM information (e.g. QoS Profile(s), Session-AMBR, Maximum Data Rate) and parameters in PDU Session Establishment Accept message (e.g. QoS Flow level QoS parameters, Session-AMBR, Always-on PDU Session Granted, Small Data Rate Control parameters), taking into account network energy related information.

After the completion of PDU Session establishment procedure, the SMF may perform the following steps for energy saving:

5.	The SMF requests "UE Communication" analytics and "Abnormal behaviour" analytics from the NWDAF as specified in clause 6.7.3 and 6.7.5 of TS 23.288 [14].

6.	Optionally, the SMF requests network energy control information from the NEMF.

Editor's note:	Whether and how the SMF gets network energy control information from the NEMF is FFS.

7.	SMF may perform an SMF initiated SM Policy Association Modification procedure if the Policy Control Request Trigger condition(s) is met as specified in TS 23.502 [3] and TS 23.503 [4]. The PCF may update the SM policy for the PDU Session based on network energy related information, e.g. energy consumption of the PDU Session reaches or goes beyond a predefined threshold.

8.	Based on UE subscription data, SM policy, "UE Communication" analytics and "Abnormal behaviour" analytics, network energy control information (if available), etc, the SMF performs Session Management procedures for energy saving, e.g. modifying the QoS parameters for specific applications based on "UE Communication" analytics.

9.	The SMF sends energy saving assistance information in N2 message to the NG-RAN. The energy saving assistance information may be part of CN assisted RAN parameters tuning based on UE communication analytics, e.g. statistics/prediction of traffic volume, periodic communication indicator and periodic time for the UE and/or the PDU Session. Based on this, the NG-RAN performs energy saving operations, e.g. redirecting the UEs to cell(s) with higher energy efficiency.

NOTE:	How the NG-RAN performs energy saving operations with both OAM configuration and energy saving assistance information from the 5GC is up to implementation.

Editor's note:	It is FFS whether the NG-RAN supports performing energy saving operation based on assistance information from the 5GC.

6.15.4	Impacts on existing services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

AMF:

-	Performs Access and Mobility Management taking into account network energy saving requirements, based on AM subscription data, AM policy, UE related analytics (e.g. UE Mobility analytics and Abnormal behaviour analytics), and/or network energy control information, etc.

-	Provides energy efficiency related information based on the subscription information to the NG-RAN.

-	Provides authorization information for applying slice level energy efficiency policy based on the subscription information to the NG-RAN.

-	Optionally, obtaining network energy control information from the NEMF.

SMF:

-	Performs Session Management taking into account network energy saving requirements, based on SM subscription data, SM policy, UE related analytics (UE Communication analytics and Abnormal behaviour analytics), network energy control information, etc.

-	Optionally, obtaining network energy control information from the NEMF.

PCF:

-	Collecting network energy related information by subscribing to network energy related notifications from the AMF, SMF and/or UPF, or requesting/subscribing to network energy related analytics from the NWDAF.

-	Obtains energy related application specific parameters from the AF (via the NEF/UDM/UDR).

-	Provisions AM policy and SM policy based on network energy related information and optionally energy related application specific parameters from the AF.

AF:

-	Provisions energy related application specific parameters to the PCF (via the NEF/UDM/UDR).

NEMF (new, optional):

-	Perform network energy management, e.g. providing to other 5GC NFs the network energy control information which contains information of the 5GC NFs in energy saving states with validity time and area information.

RAN:

-	Performs energy saving/efficiency operations base on assistance information provided by the 5GC.

* * * * End of Changes * * * *